3GPP TSG-RAN WG3#129bis 	R3-256537

Prague, Czechia, 13-17 October 2025

Agenda item:	12.2.2 Intra-CU LTM

Source:	Nokia

Title:	Discussion on AI/ML for Intra-CU LTM

Document for:	Discussion and Decision

# 1	Introduction

Rel-20 Study Item in RP-251868 introduces AI/ML with respect to L1/L2 Triggered Mobility (LTM) for both Intra-gNB and Inter-gNB scenarios. The objectives are as follows.

• The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This paper discusses principles relevant for AI/ML applied to Intra-gNB LTM use cases with split gNB architecture.

# 2	Intra-CU LTM

L1/2 Triggered Mobility introduced in Rel-18 and Rel-19 allows continuous mobility across a number of predetermined cells without need for additional RRC Reconfiguration procedures issued toward the UE. In principle, the base LTM procedure assumes that decision for selecting the cells to be prepared for LTM is based on L3 measurements received at the gNB-CU. However, the decision for early synchronization as well as issuance of cell switch can be based either on L1 measurements or L3 measurements respectively and executed at the lower layers (i.e. gNB-DU). Likewise, LTM provides the capability to perform early DL and UL synchronization, which can fasten access to the target cell. The main benefit of the LTM approach over legacy mobility methods is to achieve faster mobility and lower interruption time aiming at seamless handover.

A consequence of the LTM framework is that from the initial stage of the LTM procedure, admission control and resource reservation for each of the cells is carried out a priori. This also applies to further resources reserved for purpose of RACH-less mobility and early synchronization procedures. Furthermore, after LTM cell switch, resource reservation is kept, allowing for subsequent mobility events without RRC Reconfiguration toward the UE. However, compared to legacy mobility, the LTM framework has an increased signaling, especially for the preparation phase.

To this end, AI/ML may support decision making for LTM, e.g., at preparation phase to determine the appropriate candidate cells to become candidates, at decision making for early synchronization given radio conditions, as well as during the execution phase to select the most suitable target out of the available candidates.

## 2.1 LTM Candidate Cell Selection

The LTM framework relies on L3 measurements to initiate the LTM candidate cell selection. Based on received L3 measurements, the gNB-CU determines a candidate cell set to which LTM configuration will be requested.

The maximum number of LTM candidates to be configured for a UE is limited to 8. Thus, there is a trade-off between configuring likely targets early, and being late to do so. On the first case, many LTM cells could be included, on the expectation to avoid RRC Reconfiguration to the UE and fast mobility. However, this comes at the cost of over reserving resources at all candidates. One could say that a simpler approach is to add only extremely likely targets. However, that may also result in single target and no possibility for subsequent seamless mobility. Hence, would end up requiring RRC Reconfiguration for every additional candidate added. In that sense, there is a benefit in optimizing the number of candidates and maximize probability of subsequent LTM usage.

Proposal 1: RAN3 to investigate AI/ML mechanisms to aid and support the LTM candidate selection based on L3 measurements. Likewise, as required, introduce additional information based on existing AI/ML mechanisms to support the LTM candidate cell selection.

Further, given that resource reservation for early synchronization happens prior to early synchronization triggering, it is worth also investigating what additional information via AI/ML can help the gNB-CU avoid over reserving early synchronization resources based on past history. That is, optimize and determine which cells should be LTM cells with early synch and which should not, to reduce signaling and avoid admission control failures.

Proposal 2: RAN3 to investigate what information can aid to avoid unnecessary early synchronization configuration at LTM preparation stage.

## 2.2 Early Synchronization

The LTM framework, after preparation allows two possibilities, L1 measurement and L3 measurement based early synchronization. Each early synchronization event incurs additional signaling both at the network between nodes and toward the UE. Further, certain gNB-DU implementations may also incur interruption at the source gNB-DU to take an according action if it is pending receiving a TA value that was requested from target cell. Hence, for both cases, the optimization target is the same, that is, to trigger early synch only to the most likely candidates and minimize impact from early synchronization.

Additionally, given LTM allows for variants with respect to the measurements to be utilized to trigger early synchronization, the trigger point will differ for both cases. For L1 measurement-based LTM the gNB-DU will decide and trigger early synch, while on L3 measurement based LTM the gNB-CU is usually expected to take the decision and indicate the gNB-DU the action to take. Therefore, we see relevant that RAN3 investigates what information and mechanisms can aid optimize that the target cells that are queried for early synch are best suited, and what information will be relevant to provide to the source gNB-DU.

Proposal 3: RAN3 to investigate AI/ML mechanisms to aid triggering of early synchronization (DL and UL) for both L3 based LTM and L1 based LTM.

## 2.3 LTM Cell Switch

The LTM cell switch, similar to early synchronization, is possible to base on L1 or L3 measurements. However, a given candidate LTM cell cannot be simultaneously configured for both. On the other hand, it is possible that out of the pool of candidate LTM cells configured at a UE, some may be target for L1 measurement-based LTM and others for L3 measurement-based LTM. Likewise, it is role of the gNB-CU during LTM preparation to decide which LTM approach to configure.

At LTM section phase, the node that takes the decision to trigger the LTM cell switch to the UE depends on which variant was configured for the candidate cell. For L1 measurement-based LTM the source gNB-DU decides, while for L3 measurement-based LTM the gNB-CU is expected to take the decision to issue an LTM cell change toward the gNB-DU.

AI/ML may be used to aid the target cell selection beyond currently received measurements alone, such as beam based trajectory prediction and past UE history and trajectory. Likewise, in addition to the target cell, the beam selection can be further improved based on cell beam relations learnt via prior mobility events.

Proposal 4: RAN3 to investigate AI/ML mechanisms (e.g., beam-based trajectory prediction, cell-beam relations and UE trajectory) to aid target cell selection for both L3 based LTM and L1 based LTM.

Proposal 5: Introduce a finer granularity based on beam-level for UE trajectory prediction.

# 3	Conclusions

Proposal 1: RAN3 to investigate AI/ML mechanisms to aid and support the LTM candidate selection based on L3 measurements. Likewise, as required, introduce additional information based on existing AI/ML mechanisms to support the LTM candidate cell selection.

Proposal 2: RAN3 to investigate what information can aid to avoid unnecessary early synchronization configuration at LTM preparation stage.

Proposal 3: RAN3 to investigate AI/ML mechanisms to aid triggering of early synchronization (DL and UL) for both L3 based LTM and L1 based LTM.

Proposal 4: RAN3 to investigate AI/ML mechanisms (e.g., beam-based trajectory prediction, cell-beam relations and UE trajectory) to aid target cell selection for both L3 based LTM and L1 based LTM.

Proposal 5: Introduce a finer granularity based on beam-level for UE trajectory prediction.

# Annex	TP for TR 38.745

<< First Change >>

## 4.2	AI/ML assisted Intra-CU LTM

L1/2 Triggered Mobility allows continuous mobility across a number of predetermined cells without need for additional RRC Reconfiguration procedures issued toward the UE. Likewise, LTM provides the capability to perform early DL and UL synchronization, which can fasten access to the target cell. The main benefit of the LTM approach over legacy mobility methods is to achieve faster mobility and lower interruption time aiming at seamless handover.

The candidate cell selection is based on L3 measurements, while the early synchronization and cell switch execution can be based on either L1 or L3 measurements, depending on which variant was configured for the candidate cell. Likewise, in the LTM framework, candidate cell selection, admission control and resource reservation for each cell is carried out in the initial stage of the LTM procedure, i.e. at the preparation phase.

However, compared to legacy mobility, the LTM framework has increased signaling. To this end, AI/ML may support decision making for LTM, e.g., at preparation phase to determine the appropriate candidate cells to become candidates, at decision making for early synchronization given radio conditions, as well as during the execution phase to select the most suitable target out of the available candidates.

### 4.2.1	Use case description

Editor’s Note: Capture the description of use case

Use Case 1: AI/ML assisted intra-gNB LTM candidate cell selection

Description:

The LTM framework relies on L3 measurements to initiate the LTM candidate cell selection. Based on received L3 measurements, the gNB-CU determines a candidate cell set to which LTM configuration will be requested. With assistance from AI/ML, the gNB-CU can optimize the candidate cell selection, to limit the candidate cells to those that are most likely to become target for mobility events (including subsequent LTM) and thus avoiding unnecessary resource reservation.

Expected benefits:

Optimized number of LTM candidate cells and maximize probability of subsequent LTM usage, reducing unnecessary resource reservation, lowering access control rejection and reducing network signaling.

Early determination of LTM candidate cells subject to early synch configuration, reducing network signaling.

Use case 2: AI/ML assisted LTM early synchronization triggering

Description:

The LTM framework allows both L1 measurement and L3 measurement based early synchronization. Each early synchronization event incurs additional signaling both at the network between nodes and toward the UE. Further, certain implementations may also incur interruption at the source cell to take an according action until a requested TA value has been received. Hence, with assistance from AI/ML, the gNB can optimize and trigger early synchronization only to the most likely candidate cells, reducing network signaling and speeding mobility events.

Expected benefits:

Optimized selection of candidate cells subject to early synchronization, reducing network signaling and speeding mobility events.

Use case 3: AI/ML assisted intra-gNB LTM cell switch execution

Description:

The LTM framework allows both L1 measurement and L3 measurement based LTM cell switch, which is decided by the gNB-CU at the LTM preparation phase. AI/ML can assist the target cell selection beyond currently received measurements alone, such as via use of beam based trajectory prediction and past UE history. Likewise, in addition to the target cell, the beam selection can be further improved based on cell beam relations learnt via prior mobility events.

Expected benefits:

Optimized selection of target cells, improving handover success rate and reducing risk of radio link failures.

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

<< End of Changes