3GPP TSG-RAN WG3 Meeting #129-bis	R3-256540

Prague, Czech, Oct 13 - 17, 2025

Agenda Item:	10.3.1

Source:	ZTE Corporation

Title:		Discussion on Principles of RAN-CN Interface

Document for:	Discussions & Approval

1	Introduction

During the RAN #109 meeting, the requirements for 6G architecture and migration were captured in TR 38.914 as outlined below.

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

In this paper, we provide discussion and our perspectives on the principles of 6G RAN-CN interface.

2	Discussion

In 4G and 5G, the functional split between RAN and Core has been clearly defined. For 6G, the design principles of the RAN–CN interface should also take into account potential changes in its functional splitting. To initiate the discussion, it is beneficial to first review the functional split adopted in 4G and 5G, and then identify the new functionalities that may need to be introduced in 6G.

## 2.1 Interface for Retained 5G Functionalities

According to TS 36.300, the figure below illustrates the functional split between E-UTRAN and EPC. The eNB manages radio resources, connectivity, and connected state mobility. The MME handles NAS signaling, idle state mobility, and EPS session control.The SGW and PGW primarily process user plane data, while also supporting some control plane functions such as IP address allocation . The S1 interface enables the separation of user plane and control plane between the RAN and EPC via S1-U (E-UTRAN/SGW) and S1-C (E-UTRAN/MME).

Figure 1: Functional Split between E-UTRAN and EPC (Captured in TS36.300 Figure 4.1-1)

According to TS 38.300, the figure below illustrates the functional split between NG-RAN and 5GC. The NG-RAN hosting functionality similar to eNB in 4G, including to manage radio resources, connectivity, and connected state mobility. In 5GC, the architecture introduces a full CP/UP separation compared to EPC. Session management and UE IP allocation, previously handled by MME and PGW, are now centralized in SMF. Mobility management, access control, and NAS security are assigned to AMF. On the user plane, UPF handles packet processing and transmission. The NG-RAN connects to 5GC via the NG interface, with NG-C (AMF) and NG-U (UPF), enabling CP/UP separation.

Figure 2: Functional Split between NG-RAN and 5GC (Captured in TS38.300 Figure 4.2-1)

From our perspective, most functionalities in 5G will still be kept or evolved in 6G, and we consider it’s beneficial to use the functional split between 5G RAN and 5G Core as a baseline for most of the basic communication functionalities in 6G. This approach leverages the fact that the 5G functional split has already been extensively studied, specified, and deployed. Maintaining this baseline facilitates faster and more cost efficient 6G deployment by reusing proven design principles and operational experience. For example, the 6GR would handle RRM, RB control, connection mobility, radio admission control, measurement configuration and provision, and resource allocation (scheduling), while the 6GC would discuss NAS security, idle mobility, PDU session control, UE IP address allocation, Slicing and PDU handling. The detailed analysis and discussion on 6G RAN functions, can be found in our contribution in [1].

Observation 1: Most basic communication functionalities in 5G will still be kept or evolved in 6G, and it is beneficial to adopt the functional split between 5G RAN and 5G Core as the baseline for these retained functionalities. This leverages the fact that the 5G functional split has already been extensively studied, specified, and deployed. Maintaining this baseline enables faster and more cost efficient 6G deployment by reusing proven design principles and operational experience.

Based on the above discussion, NG+ interface could be adopted for the 6G RAN–CN interface with respect to these retained 5G functionalities. Therefore, we propose the following principles for the NG+ interface design.

Proposal 1: Taking the functional split between 5G RAN and 5G Core as the baseline for the retained 5G functionalities in 6G, and approve the following principles for the 6G RAN–CN interface design to enable faster and cost efficient 6G deployment:

- NG+ interface similar to the legacy NG interface, could be adopted for the 6G RAN–CN interface with respect to retained 5G functionalities;

- Keep NG+-C for the control plane and NG+-U for the user plane, thereby enabling CP/UP separation over RAN–CN interface;

- Keep the NG+ interface as a point-to-point interface, e.g., based on SCTP (for NG+-C) and GTP-U (for NG+-U) which has been fully validated in 5G system.

## 2.2 Interface for New 6G Services

2.2.1 New 6G Services

Potential new 6G services (see in RP-252960) may include, but are not limited to: AI, sensing, Immersive Communication which require information exchange with RAN. These services are expected to require communication not only within 6G RAN, but also with Core Network functions, e.g, NWDAF, SF, NEF, as well as with trusted third parties.

AI Services:

RAN3 will further discuss how to address the functional framework for AI/ML in other agenda item during 6G SI phase, there are currently no restrictions on which network entities used for model training, inference, data collection, or actor at this stage. From 6G architecture view, a dedicated 6G core NF may be responsible for training and transmitting inference related information to entity like other NB+,  6G Core NF may also need to collect data from RAN for training, vise versa, if the training and inference is located in NB+. A dedicated 6G core NF (similar to 5GC’s NWDAF) may also needed in 6GC which is pending to SA2 discussion. 6G RAN can handle AI functions on the RAN side, while UE may deploy AI functions locally as well. AI service needs to be supported based on the coordination cross domains in 6G.

Figure 4: Data transmission between 6G network entities

The exchange of AI/ML related data/signaling, may occur between the 6G RAN and a dedicated 6G core NF.The 6G network needs to be more practical than the 5G network, with more flexible and efficient information exchange design. To support this, RAN3 should consider the interface allowing direct AI signalling and related data exchange between the 6G RAN and the dedicated 6G core NF, rather than routing these data through intermediate entity in CN (e.g., AMF in 5GC).

Observation 2: It is necessary for RAN3 to study on the interface allowing direct AI signalling and related data exchange between Core NF of AI and 6G RAN during 6G SI phase.

Sensing services:

It is obvious that multiple sensing use cases have been included in the SA1 5G-A and 6G requirements. These use cases are promising, and useful especially for daily life, including UAV detection, pedestrian detection and tracking, vehicle detection and tracking, infrastructure collapse monitoring, etc. It is benefit for RAN3 to further discuss how to support the sensing function at the 6G network architecture.

Basically, the sensing schemes can be grouped into the following 6 sensing modes. In the mode A, i.e. BS mono-static mode, it is very like radar, the single 6G RAN node transmits sensing signal and also receives the reflected signal through the sensing target. In the mode B, i.e. BS bi-static mode where 6G RAN node A transmits sensing signal and 6G RAN node B receives the reflected signal. In the mode C, it is the same as the current 5G DL measurement where 6G RAN node transmits sensing signal and UE receives the reflected signal. In the mode D, it is the same as the current 5G UL measurement where UE transmits sensing signal and 6G RAN node receives the reflected signal.

Figure 5: Six sensing modes

In the mode E and F, they are similar as mode A and mode B respectively where BS is replaced by UE. For sensing mode F, i.e. bi-static sensing modes, the discussion related to sidelink may be needed. From our perspective, RAN3 should study the RAN impact for all 6 sensing modes at the 6G SI phase.

Moreover, it is observed that the sensing related information including the configuration, assistance information, measurement data, and sensing result may be transmitted between each sensing related entities. The following figure shows the general network structure and data transmission relationship between each of them.

Figure 6: 6G Sensing network structure and interface

RAN3 is responsible to discuss how to transmit the sensing data between network related entities,e.g. 6G RAN node, SF. From our perspective, to reduce network complexity and enhance usability, it is beneficial for RAN3 to discuss whether and how to introduce a direct interface between 6G RAN node and SF. This would allow sensing data to be exchanged between these two entities without being relayed through another core NF.

Observation 3: It is necessary for RAN3 to study on the interface between Core NF of sensing and 6G RAN in order to support sensing services during 6G SI phase.

In 5G, there is no standardized and unified mechanism for many-to-one access to obtain RAN-related information. This makes it difficult for multiple core NFs or applications to efficiently retrieve data from RAN nodes. Furthermore, While 5G Core introduced NEF for capability exposure, RAN exposure remained limited and inconsistent, constraining cross-domain optimization. Considering new 6G services such as AI, Sensing, and also there are other use cases need RAN to report some information to core NFs or to third party platform/applications via core, e.g., energy saving, network performance monitoring, a unified framework for more flexible RAN information and service exposure is required between RAN and 6GC.

A unified framework to achieve information exchange for new service between RAN and CN, as illustrated in the figure below:

Figure 7: RAN exposure function and interface for new 6G services

It is worth considering a new direct interface between RAN node and Core NFs (such as NEF, NWDAF, LMF, SMF, etc.) to achieve more flexible interaction between RAN and Core NFs, facilitating the exchange of RAN related information, e.g., AI related data, Sensing related data, capability, data collected from UE, XR/Immersive Communication related data, etc.

The new interface shall fulfill the following requirements:

- Enable many-to-one and one-to-many access for RAN information retrieval.

- Support real-time exchange of RAN related data (e.g., AI/ML training/inference data, sensing measurements, measurements collected from UE).

- Facilitate cross-domain optimization and network visual management by retrieving RAN statistical data (e.g., network KPIs, mobility, network/cell load, energy cost, UE performance) to Core NFs, or to the third party applications via Core.

- Friendly to flexible new services deployment and on-demand infrastructure, e.g., SBI.

- Both signalling and data transmission, should be supported over this new interface.

- The security of signalling and data transmission needs to be guaranteed.

Observation 4: The new interface between 6G RAN and CN shall fulfill the following requirements:

- Enable many-to-one and one-to-many access for RAN information retrieval.

- Support real-time exchange of RAN related data (e.g., AI/ML training/inference data, sensing measurements, measurements collected from UE).

- Facilitate cross-domain optimization and network visual management by retrieving RAN statistical data (e.g., network KPIs, mobility, network/cell load, energy cost, UE performance) to Core NFs, or to the third party applications via Core.

- Friendly to flexible new services deployment and on-demand infrastructure, e.g., SBI.

- Both signalling and data transmission, should be supported over this new interface.

- The security of signalling and data transmission needs to be guaranteed.

2.2.2 New Interface Design

The functional split between the RAN and Core for this new interface needs further discussion not only in RAN3 but also incolves SA2. RAN3 needs to collaborate with SA2 to define the functional split for new 6G services, ensuring the requirements as analyzed in 2.2.1.

Proposal 2: RAN3 needs to collaborate with SA2 to define the functional split for new 6G services, ensuring the requirements as analyzed in 2.2.1. The following principles are proposed for the new 6G RAN–CN interface design:

- A new interface between 6G RAN and 6G CN is considered to support new 6G services and functionalities;

- For these new services, which are not directly associated with UE dedicated signalling and traffic, both signalling and the corresponding service related data transmission are carried over this new interface;

- Evaluation is needed to determine whether an SBI-based or a point-to-point (P2P) interface can meet above requirements.

## 2.3 General Principles of Interface

Based on the above discussion, it is proposed to capture the following figure to illustrate the 6G RAN–CN functional split and interfaces.

internet

NB+

Inter Cell RRM

RB Control

Connection Mobility Cont.

Radio Admission Control

Measuremen Configuration & Provision

Dynamic Resource Allocation(Scheduler)

AI

Sensing

6G AMF(FFS)

NAS Security

Idle Mobility Handling

6G UPF(FFS)

Mobility Anchoring

PDU Handling

6G SMF(FFS)

UE IP address

Allocation

PDU Session Control

6GC AI NF

Intelligence Function

6GC Sensing NF

Sensing Function

6GC Exposure NF

Exposure Function

NG+

New Interface

6GR

6GC

Trusted Third Parties

Figure 8: Functional Split and interfaces between 6GR and 6GC

Proposal 3: Discuss and capture the above figure for the 6G RAN–CN functional split in TS 38.760-3.

2.3.1 NG+interface

Based on the discussion in 2.1 for interface for the retained 5G communication functionalities, the general principles for the specification of the NG+ interface in 6G, which can inherit most of the design principles of the NG interface, are as follows:

-	the NG+ interface shall be open;

-	the NG+ interface shall support the exchange of signalling information between the 6GR and 6GC;

-	from a logical standpoint, the NG+ is a point-to-point interface between a 6G RAN node and an 6GC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the 6G RAN and 6GC;

-	the NG+ interface shall support control plane and user plane separation;

-	the NG+ interface shall separate Radio Network Layer and Transport Network Layer;

-	the NG+ interface shall be decoupled with the possible 6G RAN deployment variants.

-	the NG+ Application Protocol shall support modular procedures design and use a syntax allowing optimized encoding /decoding efficiency.

From our perspective, the functions of the NG+ interface, similar to those of the legacy NG interface described in TS 38.300, are listed below:

NG+-C provides the following functions:

-	NG+ interface management;

-	UE context management;

-	UE mobility management;

-	Transport of NAS messages;

-	Paging;

-	PDU Session Management;

-	Configuration Transfer;

-	Warning Message Transmission.

NG+-U provides the following functions:

-	Non-guaranteed delivery of user plane PDUs

Proposal 4: Discuss and capture the above principles and functions for NG+ in TR38.760-3 (The corresponding TP is provided in Annex).

2.3.2 6G New Interface

The new 6G interface is intended to support the potential new 6G services and functionalities which may include, but are not limited to: AI, sensing, Immersive Communication which require information exchange between RAN and Core NFs. From our perspective, the most important principles of the new interface, are as follows:

-	the new interface shall be open;

-	the new interface shall support RAN information report to Core NFs, or to the third party applications via Core;

-	the new interface shall be friendly to flexible new services deployment and on-demand infrastructure, e.g., API.

Proposal 5: Discuss and capture the above principles for the new interface in TR38.760-3 (The corresponding TP is provided in Annex).

- Conclusion

Based on above discussion, we provide the following observation and proposal for the 6G RAN-CN interface for retained 5G functionalities:

Observation 1: Most basic communication functionalities in 5G will still be kept or evolved in 6G, and it is beneficial to adopt the functional split between 5G RAN and 5G Core as the baseline for these retained functionalities. This leverages the fact that the 5G functional split has already been extensively studied, specified, and deployed. Maintaining this baseline enables faster and more cost efficient 6G deployment by reusing proven design principles and operational experience.

Proposal 1: Taking the functional split between 5G RAN and 5G Core as the baseline for the retained 5G functionalities in 6G, and approve the following principles for the 6G RAN–CN interface design to enable faster and cost efficient 6G deployment:

- NG+ interface similar to the legacy NG interface, could be adopted for the 6G RAN–CN interface with respect to retained 5G functionalities;

- Keep NG+-C for the control plane and NG+-U for the user plane, thereby enabling CP/UP separation over RAN–CN interface;

- Keep the NG+ interface as a point-to-point interface, e.g., based on SCTP (for NG+-C) and GTP-U (for NG+-U) which has been fully validated in 5G system.

Based on above discussion, we provide the following observations and proposal for the 6G RAN-CN interface for new 6G services:

Observation 2: It is necessary for RAN3 to study on the interface allowing direct AI signalling and related data exchange between Core NF of AI and 6G RAN during 6G SI phase.

Observation 3: It is necessary for RAN3 to study on the interface between Core NF of sensing and 6G RAN in order to support sensing services during 6G SI phase.

Observation 4: The new interface between 6G RAN and CN shall fulfill the following requirements:

- Enable many-to-one and one-to-many access for RAN information retrieval.

- Support real-time exchange of RAN related data (e.g., AI/ML training/inference data, sensing measurements, measurements collected from UE).

- Facilitate cross-domain optimization and network visual management by retrieving RAN statistical data (e.g., network KPIs, mobility, network/cell load, energy cost, UE performance) to Core NFs, or to the third party applications via Core.

- Friendly to flexible new services deployment and on-demand infrastructure, e.g., SBI.

- Both signalling and data transmission, should be supported over this new interface.

- The security of signalling and data transmission needs to be guaranteed.

Proposal 2: RAN3 needs to collaborate with SA2 to define the functional split for new 6G services, ensuring the requirements as analyzed in 2.2.1. The following principles are proposed for the new 6G RAN–CN interface design:

-   A new interface between 6G RAN and 6G CN is considered to support new 6G services and functionalities;

-  For these new services, which are not directly associated with UE dedicated signalling and traffic, both signalling and the corresponding service related data transmission are carried over this new interface;

-  Evaluation is needed to determine whether an SBI-based or a point-to-point (P2P) interface can meet above requirements.

We provide the corresponding TP for TR38.760-3 for RAN-CN interface in Annex based on the following proposals:

Proposal 3: Discuss and capture the figure(discussed in 2.3) for the 6G RAN–CN functional split in TS 38.760-3.

Proposal 4: Discuss and capture the principles and functions (discussed in 2.3.1) for NG+ in TR38.760-3.

Proposal 5: Discuss and capture the principles for the new interface(discussed in 2.3.2) in TR38.760-3.

4	Reference

[1] R3-256543, Discussion on RAN logical architecture and functions, ZTE Corporation

- 5 Annex: TP for TR 38.760-3

5.3.1 	RAN-CN functional split

The following figure illustrates the 6G RAN–CN functional split and interfaces between 6G RAN and CN.

internet

NB+

Inter Cell RRM

RB Control

Connection Mobility Cont.

Radio Admission Control

Measurement Configuration & Provision

Dynamic Resource Allocation(Scheduler)

AI

Sensing

6G AMF(FFS)

NAS Security

Idle Mobility Handling

6G UPF(FFS)

Mobility Anchoring

PDU Handling

6G SMF(FFS)

UE IP address Allocation

PDU Session Control

6GC AI NF

Intelligence Function

6GC Sensing NF

Sensing Function

6GC Exposure NF

Exposure Function

NG+

New Interface

6GR

6GC

Trusted Third Parties

Figure 5.3.1.x: Functional Split and interfaces between 6GR and 6GC

6.1.1	RAN-CN interface general principles and functions

6.1.1.1 NG+ interface

The NG+ interface is intended to support the fundamental communication functionalities of 5G that are retained or evolved in 6G.

The general principles for the specification of the NG+ interface are as follows:

-	the NG+ interface shall be open;

-	the NG+ interface shall support the exchange of signalling information between the 6G RAN and 6GC;

-	from a logical standpoint, the NG+ is a point-to-point interface between a 6G RAN node and an 6GC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the 6G RAN and 6GC;

-	the NG+ interface shall support control plane and user plane separation;

-	the NG+ interface shall separate Radio Network Layer and Transport Network Layer;

-	the NG+ interface shall be decoupled with the possible 6G RAN deployment variants.

-	the NG+ Application Protocol shall support modular procedures design and use a syntax allowing optimized encoding /decoding efficiency.

The functions of the NG+ interface are as follows:

NG+-C provides the following functions:

-	NG+ interface management;

-	UE context management;

-	UE mobility management;

-	Transport of NAS messages;

-	Paging;

-	PDU Session Management;

-	Configuration Transfer;

-	Warning Message Transmission.

NG+-U provides the following functions:

-	Non-guaranteed delivery of user plane PDUs;

6.1.1.2 New 6G Interface

The new 6G interface is intended to support the potential new 6G services which may include, but are not limited to: AI, sensing, Immersive Communication, and also there are other use cases need RAN to report some information to core NFs or to third party platform/applications via core, e.g., energy saving, network performance monitoring which require information exchange between RAN and 6GC. The principles of the new interface, are as follows:

-	the new interface shall be open;

-	the new interface shall support RAN information report to Core NFs, or to the third party applications via Core;

-	the new interface shall be friendly to flexible new services deployment and on-demand infrastructure, e.g., API.