3GPP TSG-RAN WG3 Meeting #129-bis	R3-256557

Prague, Czechia, 13 – 17 October 2025

Agenda item:	10.2.2

Source:	Nokia

Title:	Resilient 6G RAN Architecture

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

The new SID on 6G RAN was agreed by RAN#108 and further updated at RAN#109 [1].

The main objectives for RAN3 are listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This paper addresses the overall 6G RAN Architecture aspects including resiliency and aims at proposing requirements for the RAN logical architecture in 6G.

# 2	Overall architecture of 6G RAN

In line with the justification of 6G study [1], the 6G RAN architecture should be aimed at ensuring flexibility in deployment, interoperability and performance optimization. From our point of view, the 6G RAN architecture is depicted in Figure 1 below.

Figure 1: 6G RAN architecture overview

The 6G RAN architecture includes a vertical interface for interaction with the core network (NG-like) and a horizontal interface for interactions with peer RAN nodes / 6G NodeBs (Xn-like). These interfaces have proven to be beneficial and adequate to accommodate current use cases. Thus, the minimum requirement for 6G RAN Architecture is to support an interface between 6G NodeBs (aka. 6G Xn) and an interface to the core network (aka. 6G NG). Introduction of any new interface in 6G RAN as well as new entity/node in 6G RAN needs to be well justified with proven use case(s) and their requirements.

Observation 1: The NG and Xn interfaces are commercially viable multivendor interfaces with clear needs for standardization.

Proposal 1: RAN3 to support 6G NG and 6G Xn interfaces. FFS details on protocol stack.

Proposal 2: RAN3 to study and agree on the use cases and their requirements before discussing the need of additional interfaces and/or new entity/node in RAN.

# 3	Resilient 6G RAN Architecture

One of requirements from RAN plenary applicable to both the 6G RAN architecture and the 6G RAN as a whole relates to enhanced resiliency (TR 38.914 v0.2.0 section 5.2):

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

Before discussing details on the solution, it is important that RAN3 formulates the definition of resilience, especially from the service continuity point of view. In our view, the resilience of 6G RAN should be manifested by minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Observation 2: The resilience of 6G RAN is manifested as minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

One of the enhancements that RAN3 should study to enhance provisioning of resiliency for 6G RAN architecture is the introduction of a “RAN Set” concept inherited from the core network (i.e. Network Function Set). The Network Function (NF) set enables any NFs within the same set to access the same UE context. With the NF set, service interruption can be minimized in case NW performs maintenance for some of NFs in the set or NW experiences failure of some NFs in the set, as specified in [2]. The NF set has been a proven concept to offer 5GC resiliency from the commercial 5G SA network deployments over the world. Likewise, a RAN Set is composed of a group of 6G NodeBs as shown in Figure 2, that can cooperate with one another to increase the RAN resiliency and allow UE to maintain service continuity by sharing the UE context information within the RAN Set. For example, 6G NodeBs operating in an overlapping coverage area may be set up such that they can share the UE context and thus be able to overcome 6G NodeB failures by quickly switching the handling of a UE from the failed 6G NodeB to a cooperating one to reduce the interruption time perceived by the UE. Such mechanism avoids one 6G NodeB to appear as a single point of failure and if UE would need to re-establish a connection to any backup 6G NodeB after a failure of the serving 6G Node B, then the UE context is maintained and the connection can be resumed without the need of re-establishment.

Figure 2: 6G RAN set overview

In a similar fashion, 6G NodeBs operating in a non-overlapping coverage area may also be set up as RAN Set so that they can share the UE context. This is helpful in case of an RLF and/or if UE moves to the neighboring 6G NodeB where the interruption time is large, as the UE would need to send an RRC establishment request to the new/target 6G NodeB, whereas with the introduction of RAN Set, target 6G Node B would fetch the UE context from the previous serving 6G NodeB of the UE without the need of re-establishing the connection to the target 6G NodeB.

Observation 3: In 5G RAN Architecture, there is no specified mechanism to tackle the case of a 6G NodeB failure and efficient UE context fetching.

Observation 4: SA2 has already specified mechanism of NF Set for resiliency and it is a proven mechanism to enable UE context sharing.

For example, if we look at Figure 3, we may have the following scenarios:

Case a) 6G NodeB 4 outside a RAN SET 1 (6G NodeB 1, 6G NodeB 2) may not have a direct Xn interface to the previous source 6G NodeB of the UE, but may have at least an Xn interface with a 6G NodeB within the RAN Set.

Case b) 6G NodeB 4 outside a RAN SET 1 (6G NodeB 1, 6G NodeB 2) may have Xn interface established with all 6G NodeBs of the RAN Set.

For case a) the RAN Set, where the UE context is shared among 6G NodeBs of the RAN Set, is beneficial to the 6G NodeB 4 outside the RAN Set, as it enables 6G NodeB 4 to be able to fetch the UE context from/via 6G NodeB2 which otherwise would not be possible with today’s mechanisms. That said, if a target 6G NodeB does not have an Xn interface with the previous source 6G NodeB of the UE, it cannot fetch the UE context.

Similarly, for case b) the RAN Set,where the UE context is shared among 6G NodeBs of the RAN Set, is beneficial to the 6G NodeB 4 outside the RAN Set if the Xn interface of 6G NodeB 4 with 6G NodeB 1 would not work due to an interface failure. In that case, 6G NodeB 4 may still be able to fetch the UE context from/via 6G NodeB 2 knowing that 6G NodeB 2 has a shared UE context with 6G NodeB 1 within the RAN Set 1.

Those two cases infers another benefit of the RAN set that 1) Xn connectivity among all nodes is not a prerequisite deployment condition and that 2) the RAN network is tolerant to Xn connection failure.

Observation 5: The RAN set offers resiliency even w/o Xn connectivity or in case of Xn connection failure.

Figure 3: RAN set under failure scenarios

The above considerations enable us to extend the basic 6G RAN architecture presented in Figure 1 above. The enhancement consists in defining SETs that may group 6G NodeBs, as presented in the Figure 4 below.

Figure 4: Resilient 6G RAN Architecture

Proposal 3: RAN3 to study the concept of RAN Set for 6G RAN based on the proposed resilient 6G RAN architecture.

# 4	Conclusion

This paper has discussed key principles for RAN logical architecture and has made the following observations and proposed the following requirements:

Observation 1: The NG and Xn interfaces are commercially viable multivendor interfaces with clear needs for standardization.

Proposal 1: RAN3 to support 6G NG and 6G Xn interfaces. FFS details on protocol stack.

Proposal 2: RAN3 to study and agree on the use cases and their requirements before discussing the need of additional interfaces and/or new entity/node in RAN.

Observation 2: The resilience of 6G RAN is manifested as minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Observation 3: In 5G RAN Architecture, there is no specified mechanism to tackle the case of a 6G NodeB failure and efficient UE context fetching.

Observation 4: SA2 has already specified mechanism of NF Set for resiliency and it is a proven mechanism to enable UE context sharing.

Observation 5: The RAN set offers resiliency even w/o Xn connectivity or in case of Xn connection failure.

Proposal 3: RAN3 to study the concept of RAN Set for 6G RAN based on the proposed resilient 6G RAN architecture.

# 4	References

- RP-252912, 6G RAN study

- 3GPP TS 23.501, “System architecture for the 5G System (5GS); Stage 2,”