3GPP TSG-RAN WG3 Meeting #129bis	R3-256558

Prague, Czech Republic, October 13-17, 2025

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	Discussion on Network Architecture for ISAC

Document for:	Discussion

- Introduction

In the SID of ISAC [1], the  objective is given as below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we provide the initial consideration on network architecture for ISAC in R20.

- Discussion

- 2.1 Topology

In TS 22.137[2], the service requirement for ISAC has been captured, and the mono-static sensing operation where the sensing transmitter and sensing receiver are co-located in the same entity has been defined, one example is given as below.

Figure 1: Example of sensing with co-located sensing receiver and sensing transmitter

In R20, only the gNB-based mono-static sensing for UAV sensing target use cases are supported for ISAC, i.e., the sensing transmitter and sensing receiver are co-located in the same gNB. Therefore, the above topology can be selected as the baseline of sensing in R20. Therefore, the logical system architecture for ISAC can be given as below, while SF is logically located and defined in 5GC.

Figure 2. Logical system architecture for ISAC

Proposal 1: Capture the above logical system architecture for sensing in TR.

- 2.2 RAN Architecture

In R20, the basic scenario of ISAC and the sensing mode can be given as below. The sensing function should be supported in the gNB and the UAV is the Sensing target in R20.

Figure 3: Sensing mode for ISAC in R20

As given in the SID, RAN1 has started their study of ISAC in RAN1#122 meeting, and RAN1 shall focus on the metrics, measurements, and relevant measurement quantization for UAV use case. In addition, another SID related to ISAC has been discussed in SA2 in parallel [3], which has provided some consideration on the system architecture to sensing in R20.

Observation: RAN3 shall take the progress of other WGs (SA2 and RAN1) on ISAC into account.

Regarding the design of network architecture in RAN3, there are three candidate options on this issue.

Option 1: New interface solution

Option 2: Positioning like solution

Option 3: Ambient-IoT like solution

For Option 1, the gNB shall connect directly with the Sensing Function, and a new interface between the gNB and the Sensing Function needs to be defined. In this case, the new protocol stack should be introduced accordingly which could apply the similar design like the existing NG interface or new SBI interface.

For Option 2, the gNB shall connect with the Sensing Function like the way we designed for the Positioning feature. In this case, the existing NG interface can be reused, and a new application protocol, similar as the NRPPa, should be defined. Furthermore, the communication between the gNB and the Sensing Function can be achieved by the new application protocol on the top of NGAP. The ISAC specific information should be transferred transparently through the NAS-PDU over NG interface. In this option, the protocol stack can be designed like the Positioning as below, with the new application protocol on top over the NGAP.

Figure 4: Potential protocol stack for Positioning like solution

For Option 3, the gNB shall connect with the Sensing Function like the way we designed for the Ambient-IoT feature. In this case, both of the existing NG interface and NG-C protocol stack can be reused, and the ISAC specific information should also be transferred transparently via the AMF. In this option, the the protocol stack can be designed like the Ambient-IoT as below.

Figure 5: Potential protocol stack for A-IoT like solution

Among the three options, Option 1 introduces the most impact on the existing CN since a new interface and the corresponding protocol stack should be defined. As R20 is the last release in 5G, it would be better to specify the solution based on current 5GA architecture and minimize the standard impact as much as possible for rapid commercial deployment.

For Option 2 and Option 3, both of them introduce less impact on the existing CN than Option 1 since the NG interface can be reused. While, for Option 2, a new application protocol like NRPPa should be defined, which may introduce more standardization than Option 3. For Option 3, only the NGAP signalling enhancement is needed to support the ISAC, which has the least impact on the existing RAN architecture.

|  | Impact on CN | New interface to be introduced? | New protocol to be studied? |
| --- | --- | --- | --- |
| Option 1 | High | Yes | Yes |
| Option 2 | Medium | No☺ | Yes |
| Option 3 | Low | No☺ | No☺ |

Table 1: Comparison among the candidate options

Proposal 2: The above candidate options need to be studied and evaluated in RAN3 for 5GA sensing. Option3 has less standard impact compared with other two options.

Since only the gNB-based mono-static sensing for UAV is supported in R20, the Sensing Entity must be the gNB. In this case, the CU/DU split gNB for ISAC should be supported. To be more specific, the ISAC related procedures, e.g., the Sensing parameters configuration procedure and the Sensing data reporting procedure, should also be supported over F1. While, the discussion of the aggregated gNB should be prioritized in this release, and the details of split architecture on ISAC could be discussed later, depending on the progress of the aggregated gNB.

Proposal 3: CU/DU split gNB for sensing should be discussed based on the solution for aggregated gNB in R20.

3	Summary

RAN Architecture

Proposal 1: Capture the above logical system architecture for sensing in TR.

Observation: RAN3 shall take the progress of other WGs (SA2 and RAN1) on ISAC into account.

Proposal 2: The above candidate options need to be studied and evaluated in RAN3 for 5GA sensing. Option3 has less standard impact compared with other two options.

Proposal 3: CU/DU split gNB for sensing should be discussed based on the solution for aggregated gNB in R20.

4	Reference

[1] RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator)

- [2] TS 22.137 v19.1.0

- [3] SP-250809, Revised SID on Study on Stage 2 for Integrated Sensing and Communication, SA2

5	TP for TR 38.765

7	Network Architecture

Figure 7 depicts a logical system architecture for ISAC.

Figure 7. Logical system architecture for ISAC

The potential solutions of  NG-RAN architecture for Sensing are given as below.

7.x	New interface solution

In this solution, a new interface between the ISAC RAN and the Sensing Function and the corresponding new protocol stack should be defined.

XX in the new interface, the ISAC signalling is transported over the XXAP.

Figure 7.x shows the Protocol stack for New interface solution:

Figure 7.x. Protocol Stack for New interface solution

7.y	Positioning like solution

In this solution, a new application protocol like NRPPa should be defined.

NLX in the new interface between AMF and Sensing Function, NRxPa is the new application protocol, and the ISAC signalling is transported over the NRXPa on the top of NGAP.

Figure 7.y shows the Protocol stack for Positioning like solution:

Figure 7.y. Protocol Stack for Positioning like solution

7.z	Ambient-IoT like solution

In this solution, no new interface or new protocol stack needs to be defined.

Figure 7.z shows the Protocol stack for Ambient-IoT like solution:

Figure 7.z. Protocol Stack for Ambient-IoT like solution

7.xx	Split gNB Architecture for ISAC

Editor’s Note:  CU/DU split gNB for sensing should be discussed based on the solution for aggregated gNB.