3GPP TSG-RAN WG3 Meeting #129bis	R3-256573

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	[TPs to BL CRs] Consideration on Ambient IoT Topology 2

Agenda Item:	14.2

Source:	Huawei, China Unicom, China Telecom

Document for:	Agreement

# Introduction

For the Rel-20 Ambient IoT WI, RAN3 related objectives include:

- Objective 1 (D1T1, DO-A) - This objective begins after RAN#111 (March 2026) RAN3-led - Specify DO-A specific NGAP procedure. - Objective 2 (D2T2, DO-DTT, DT) - This objective begins after RAN#109 (September 2025) - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Specify RRC-based architecture, in coordination with SA2. RAN3-led - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

During the 3 RAN3 meetings in 2025 Q4 and 2026 Q1, we should focus on the support of Topology 2, and the discsussion can be classified into the following topics:

- UE Reader Authorization

- UE Reader Selection

- NGAP Signalling for Inventory and Command (also need to enable Reader id lever device location)

- Inter-gNB RRC-Connected UE Reader Mobility

- (Subject to checkpoint Sep 2026) DO-A support (also need to enable Reader id lever device location)

In this contribution, we further elaborate our considerations about the first 4 topics, and provides related TPs.

# Discussion

## General

In the latest version of TR 23.700-30, the following Architectural Assuptions can be found for T2:

Architectural assumptions for support of Topology 2 for Key Issue#1:

-	No Rel-19 AIoT Device impact is expected.

-	The following topology as defined in TR 38.848 [10] is assumed:

-	BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control

-	Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied.

-	The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline.

And in the updated WID, it is said that “Specify RRC-based architecture, in coordination with SA2”.

Conclusion 1: Rel-20 T2 discussion, based on the RRC-based solution, and only indirect connectivity is applied.

## UE Reader Authorization

In TR 38.769, the following can be found for UE Reader authorization:

| 6.3.6 | It is assumed that the intermediate UE authorization is performed by upper layers |
| --- | --- |
| 6.4.2.1.0 | Only if the UE is authorized to perform the A-IoT service, the UE can communicate with the A-IoT device and be configured with AIoT radio resources controlled by the A-IoT enabled gNB. NOTE 2: The NGAP protocol functions for the A-IoT-enabled UE authorization needs further discussions. |

Similar to the authorization handling for other features supported by the UEs, during several NGAP/F1AP/XnAP procedures, the UE authorization status is provided from the UE’s serving AMF to the gNB, from the gNB-CU to the gNB-DU, from the source/old gNB to the target/new gNB, it is straight forward to support the same handling for UE reader authorization.

As inter-gNB RRC-Connected UE Reader Moblity is supported, and RAN2 objective mentioned RLF scenario, it is better to also include the UE Reader Authorized IE also in XnAP Handover procedure (for HO) and Retrieve UE Context procedure (for RRC re-establishment after RLF).

The RAN3 objectives stated that “The objective includes specification of UE reader authorization, including F1AP support, ….”, considering that the A-IoT radio resources should be controlled by the gNB-DU in case of split architecture of the macro gNB, upon receiving the UE Reader authorization status from the CN, the gNB-CU should send it to the gNB-DU via UE Context Setup/Modification Request.

Proposal 1-1: UE Reader Authorization is performed at 5GC, and the UE Reader authorization status is provided from the 5GC to the gNB, from the gNB-CU to the gNB-DU, from the source/old gNB to the target/new gNB.

Proposal 1-2: Include the UE Reader Authorized IE ENUMERATED (authorized, not authorized, ...) in the following messages:

- NGAP: INITIAL CONTEXT SETUP REQUEST

- NGAP: UE CONTEXT MODIFICATION REQUEST

- NGAP: HANDOVER REQUEST

- NGAP: PATH SWITCH REQUEST ACKNOWLEDGE

- XnAP: HANDOVER REQUEST

- XnAP: RETRIEVE UE CONTEXT RESPONSE

- F1AP: UE CONTEXT SETUP REQUEST

- F1AP: UE CONTEXT MODIFICATION REQUEST

## UE Reader Selection

In Rel-19, the Reader Selection in T1 was concluded as:

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list). If the Requested Service Area Information contains: - neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves - the Requested AIoT Area List, the gNB shall select readers within the indicated AIoT Area(s) - the Requested Reader List, the gNB shall take the reader list into account to perform inventory.

In TR 38.769, the following can be found:

In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN. NOTE 3: How to perform reader selection needs further discussions. NOTE 4: In NAS/UP based solution, whether A-IoT CN selects the A-IoT-enabled UE for the selection needs further discussion.

Based on Rel-19 T1 discussion, the CN may provide a list of reader ids to the gNB, and the gNB shall take the reader list into account, therefore, the final reader selection is actually handed by the gNB, the information from the CN can be considered as important assistance information. For T2 RRC based solution, only the gNB is aware of the UE reader’s NR Uu radio status, it is better to follow the T1 design that the UE reader selection should be handled by the gNB.

Proposal 2-1: The gNB is responsible for the selection of UE readers in T2.

Currently the Requested Service Area Information IE in Inventory Request is designed as follows:

#### 9.3.3.71	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.276 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area ID | M |  | 9.3.1.277 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

In case an empty Requested Service Area Information IE is received, in Rel-19, the gNB selets all the served readers. In Rel-20 T2, it is straight forward to adopt the same solution, i.e., in case an empty Requested Service Area Information IE is received, the gNB selects all the connected mode UEs which were authorized as UE readers.

Proposal 2-2: In case an empty Requested Service Area Information IE is received, the gNB selects all the connected mode UEs which were authorized as UE readers.

Except the empty Requested Service Area Information IE case, for the selection of UE readers by the gNB, there could be several potential ways to go:

Solution 1: A-IoT Area id based

-	the Requested Service Area Information IE is unchanged;

-	the A-IoT CN provides the A-IoT Area id list to the gNB in Inventory Request;

-	the gNB is configured with the mapping relationship between the A-IoT Areas and the NR Uu Cells or TAIs;

-	the gNB selects the readers within the NR Uu Cells or TAIs which are relavant to the indicated A-IoT Areas;

Solution 2: NR Uu Cell id and/or TAI list based

-	enhance the Requested Service Area Information IE by adding a NR Uu Cell List and/or TAI list;

-	the AIOTF is configured with the information about NR Uu Cell List or TAI list;

-	the A-IoT CN provides the NR Uu Cell List and or TAI list to the gNB in inventory Request;

-	the gNB selects the UE readers within the NR Uu Cells and/or the TAI list;

Solution 3: UE Reader ID based

-	enhance the Requested Service Area Information IE by adding a UE Reader ID list;

-	the details of the UE Reader ID are FFS;

-	the gNB shall take the received UE Reader ID list into account;

Considering that the solution2 exposes the NR Uu Cell /TAI information to the AIOTF which is not expected, in our view, solution 2 should be deprioritized.

Proposal 2-3: RAN3 to further discuss A-IoT Area ID based, NR Uu Cell ID/TAI list based, UE Reader ID based solutions.

## NGAP Signalling for Inventory and Command

From our understanding, the Rel-19 introduced NGAP procedures should be reused to support T2 RRC based solution, the differences forseen include:

- may need to enhance the Requested Service Area Information IE, subject to UE Reader selection discussion

- may need to enhance the Reader Report List IE in inventory Report, subject to the discussion on the id of the UE reader.

Proposal 3-1: Reuse Rel-19 NGAP signallings to support Inventory and Command in T2.

Proposal 3-2: Further discuss which id of the UE can be used to represent the UE reader in Inventory Report Transfer.

## Inter-gNB RRC-Connected UE Reader Mobility

For following two bullets:

  - Inter-gNB resource coordination is not specified.

  - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

As the UE reader is selected by the gNB with or without assistance information from CN, once the UE changes gNB, the A-IoT service provided by the UE for source gNB should be stopped, and if the new/target gNB would like to use the UE as UE reader, the new/target gNB triggers the related actions to the UE after mobility procedure. With this consideration, except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs.

Proposal 4-1: Once the UE changes gNB during mobilty, the A-IoT service provided by the UE in source gNB should be stopped.

Proposal 4-2: Except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs

# Conclusion

In this contribution, we provide our considerations about the support of Topology 2, got the following observation and proposals:

General

- Conclusion 1: Based on SA2 progress, in Rel-20 T2 discussion, based on the RRC-based solution, and only indirect connectivity is applied.

UE Reader Authorization

- Proposal 1-1: UE Reader Authorization is performed at 5GC, and the UE Reader authorization status is provided from the 5GC to the gNB, from the gNB-CU to the gNB-DU, from the source/old gNB to the target/new gNB.

- Proposal 1-2: Include the UE Reader Authorized IE ENUMERATED (authorized, not authorized, ...) in the following messages:

  - NGAP: INITIAL CONTEXT SETUP REQUEST

  - NGAP: UE CONTEXT MODIFICATION REQUEST

  - NGAP: HANDOVER REQUEST

  - NGAP: PATH SWITCH REQUEST ACKNOWLEDGE

  - XnAP: HANDOVER REQUEST

  - XnAP: RETRIEVE UE CONTEXT RESPONSE

  - F1AP: UE CONTEXT SETUP REQUEST

  - F1AP: UE CONTEXT MODIFICATION REQUEST

UE Reader Selection

- Proposal 2-1: The gNB is responsible for the selection of UE readers in T2.

- Proposal 2-2: In case an empty Requested Service Area Information IE is received, the gNB selects all the connected mode UEs which were authorized as UE readers.

- Proposal 2-3: RAN3 to further discuss A-IoT Area ID based, NR Uu Cell ID/TAI list based, UE Reader ID based solutions.

NGAP Signalling for Inventory and Command

- Proposal 3-1: Reuse Rel-19 NGAP signallings to support Inventory and Command in T2.

- Proposal 3-2: Further discuss which id of the UE can be used to represent the UE reader in Inventory Report Transfer.

Inter-gNB RRC-Connected UE Reader Mobility

- Proposal 4-1: Once the UE changes gNB during mobilty, the A-IoT service provided by the UE in source gNB should be stopped.

- Proposal 4-2: Except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs

The corresponding TPs about UE reader authorization are provided in the following sections.

# Text Proposal for 38.300 BL CR (based on R2-2506490)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

//skip unchanged part

A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF.

A-IoT device: a device that supports A-IoT radio interface towards A-IoT reader, as defined in 16.x.

A-IoT MSG1: first D2R message transmission in the A-IoT CBRA procedure, as defined in 16.x.

A-IoT MSG2: R2D message in response to A-IoT MSG1 in the A-IoT CBRA procedure, as defined in 16.x.

A-IoT reader: reader providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.x.

//skip unchanged part

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

gNB-reader: an A-IoT reader that is deployed within the gNB, as defined in 16.x.

//skip unchanged part

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: a UE that provides functionality to support connectivity between two U2U Remote UEs.

U2U Remote UE: a UE that communicates with other UE(s) via a U2U Relay UE.

UE-reader: a UE that providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.x.

Upstream: direction toward parent node in IAB-topology.

Uu Relay RLC channel: an RLC channel between L2 U2N Relay UE or MP Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network Relay or for indirect path in case of MP.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.x	Support of Ambient IoT

### 16.x.1	General

A-IoT radio interface provides the communication between A-IoT device(s) and A-IoT reader, as illustrated in Figure 16.x.1-1. In this release, the A-IoT reader is a gNB-reader or a UE-reader. A-IoT radio interface can support both inventory procedure and command procedure as defined in TS 23.369 [zz]. An A-IoT device monitors the R2D message as long as it has sufficient energy.

Figure 16.x.1-1: Architecture supporting the A-IoT radio interface

### 16.x.2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple A-IoT readers.

-	A gNB is aware of the mapping relationship between A-IoT Areas and gNB-readers by means of OAM configuration.

-	An A-IoT Area may include gNB-readers belonging to the same or different gNB.

-	One gNB-reader only belongs to one gNB. One UE-reader only belongs to its serving gNB, the A-IoT related signalling is carried between UE-reader and it serving gNB via RRC.

-	One gNB-reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [zz], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF). A gNB is aware of direct connectivity with the AIOTF via OAM.

NOTE:	A deployment with both direct connectivity and indirect connectivity is not expected.

NOTE x:	Only indirect connectivity is supported in a deployment with UE-reader.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of gNB-readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the gNB-readers supported by the gNB.

The AIOTF may also be aware of the gNB-reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, gNB-readers and A-IoT areas by means of OAM configuration, as needed.

UE-reader authorization is performed at core network, the UE’s serving AMF provides UE-reader authorization information to the gNB during UE context management procedures.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for NGAP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

//skip unchanged part

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

//skip unchanged part

-	store the received PDU Set QoS parameters, if supported, in the UE context and use it as specified in TS 23.501 [9];

-	store the received A-IoT UE Reader Authorized Authorization information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

//skip unchanged part

If the Ranging and Sidelink Positioning Service Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, update the Ranging and Sidelink Positioning service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the A-IoT UE Reader Authorized IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, update the A-IoT UE Reader authorization information for the UE accordingly. If the A-IoT UE Reader Authorized IE includes and set to “not authorized”, the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

//skip unchanged part

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and the Area Scope of MDT IE.

If the A-IoT UE Reader Authorized IE is contained in the HANDOVER REQUEST message and set to "authorized", the NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

//skip unchanged part

If the AMF UE NGAP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the gNB shall, if supported, store this information in the UE context and use it for subsequent Xn handovers.

If the A-IoT UE Reader Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "authorized", the NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the AMF to provide UE Context information changes to the NG-RAN node.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| Target RAN Node ID | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| //skipunchanged part |  |  |  |  |  |  |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides the authorization status of the UE to act as an A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as A-IoT UE Reader or not. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for F1AP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-IoT	Ambient IoT

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

//skip unchanged part

For each DRB that has been successfully established and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform delay measurements on the successfully established DRBs.

If the A-IoT UE Reader Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it is set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized as A-IoT UE Reader.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

//skip unchanged part

For each DRB that has been successfully established or modified and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE or in the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform or update delay measurements on the successfully established or modified DRBs.

If the A-IoT UE Reader Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its authorization information for the UE accordingly. If the A-IoT UE Reader Authorized IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer acting as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| //skip unchanged part |  |  |  |  |  |  |
| LTM Information SN Addition |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| LTM Information SN Modification |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides information on the authorization status of the UE to act as A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as A-IoT UE Reader. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for XnAP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

A-IoT	Ambient IoT

AMF	Access and Mobility Management Function

A2X	Aircraft-to-Everything

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

//skip unchanged part

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the A-IoT UE Reader Authorized IE is included in the HANDOVER REQUEST message and set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission. The procedure can also be used to transfer the authorization status information of the mobile IAB-node.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

//skip unchanged part

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, and the Geographical Area IE contains the MDT PLMN List IE, the new NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the A-IoT UE Reader Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.2.3.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| //skip unchanged part |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.2.3.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	A-IoT UE Reader Authorized

This IE provides information on the authorization status of the UE to act as A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as A-IoT UE Reader. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>