3GPP TSG-RAN WG3 Meeting #129bis                                                             R3-256576

Prague, Czech Republic, 13 – 17 October 2025

Agenda item: 	10.3.2

Source:  	Qualcomm Inc, NTT Docomo Inc, Fibercop, Tejas Network Limited, KT Corp, Charter Communications Inc.

Title:  	Discussion on 6G RAN-CN Interface options

Document for:		Discussion

# 1	Introduction

TSG RAN#108 agreed on a new SI on 6G Radio. The SID contains the following objectives in RAN3 scope [1]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. … - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] … - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML … TSG#112 (June/2026): RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split.

TSG RAN#109 further agreed the following requirements for architecture and migration for 6G [2].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

This document discusses options for the 6G RAN-CN interface. The discussion assumes that the 6G RAN-CN interface supports:

- A user plane, which carries user data between UE and CN-based mobility anchor via a 6G PDU session.

- A control plane, which carries messages between RAN and CN.

The discussion assumes that the 6G RAN-CN functional split remains similar to that of 5G. The functions to be supported by the 6G RAN-CN interface protocols may be similar to that of 5G.

The 6G RAN-CN interface should allow for deployment of RAN NFs in cloud-native environments as discussed in a separate contribution [R3-256575]. The following discussion therefore assesses whether the legacy 5G RAN-CN U-plane and C-plane solutions can meet the requirements of cloud-native deployments and proposes alternative options in case these requirements cannot be met.

# 2	Discussion

## 2.1	P2P vs. SBI

Based on the SID agreed by TSG RAN [1], RAN3 needs to provide interim study results to allow TSGs to make a decision on whether the RAN-CN interface is P2P or SBI.

TSG RAN further agreed the following requirements for the 6G architecture and migration [2]:

The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

One way to lower CAPEX/OPEX of 6G RAN is to virtualize 6G RAN NFs and leverage the benefits of commercial cloud solutions such as containers, message brokers, load balancers, etc. Some operators have already gone this route for 5G RAN deployments and manage 3GPP specific, non-cloud friendly interfaces and protocols via additional OPEX. It can be expected that more operators may leverage commercial cloud solutions for 6G RAN over the timeline of 6G, i.e., the next 10-15 years. For this reason, the 6G RAN-CN interface should be compliant with commercial cloud-native solutions.

At the same time, some operators may choose to continue traditional deployment approaches for 6G RAN, at least for some timeframe. For this reason, the 6G RAN-CN interface needs to support both virtualized RAN deployments using commercial cloud-native solutions as well as traditional RAN deployment approaches.

Observation 1: Some operators may prefer to deploy 6G RAN NFs in cloud-native environments and leverage commercial cloud solutions (containers, message brokers, load balancers), while others may prefer deployment of 6G RAN NFs as appliances.

Proposal 1: The 6G RAN-CN interface should allow for virtualized RAN deployments that aim to leverage commercial cloud-native solutions (containers, message brokers, load balancers) and it should also support traditional RAN deployment approaches.

RAN3’s study on the nature of the 6G RAN-CN interface, e.g., whether it is P2P or SBI, should consider the requirement posed by proposal 1. The study should further identify other requirements that need to be met to allow for cloud-native RAN deployments and consider traditional RAN deployments.

Proposal 2: RAN3 to identify the relevant requirements for the 6G RAN-CN interface to support virtualized RAN deployments that leverage commercial cloud-native solutions.

## 2.2	User plane

In 5G, the RAN-CN interface uses NG-U for user plane traffic. NG-U supports a tunnel structure, which allows carrying the UE’s PDU-session over the IP layer of the RAN’s BH to the UPF. This tunnel has the following purposes:

- It allows the UPF to operate as the UE’s central PDU-session mobility anchor.

- It allows the UE to use different data types for the PDU-session than supported by the gNB-BH.

In 6G, it can be assumed that the 6G CN will support a UPF-equivalent CN function. A tunnel structure will also be needed for the same reasons as in 5G.

In 5G, the NG-U interface further supports the GTP-U layer, defined in TS 29.281. The GTP-U layer supports a header which carries tunnel-specific information, such as the TEID, and traffic-specific information, such as QFI, etc., as defined in TS 38.415.

In 6G, an equivalent protocol layer as GTP-U needs to be supported for the same purposes. To keep changes small, GTP-U can be reused in 6G. The information carried on the GTP-U header may have to be extended, as needed.

The NG-U interface further supports a UDP layer to carry GTP-U. The UDP layer ensures that NG-U can pass through middle-boxes (e.g., stateful firewalls). It further allows operating the GTP-U layer above the HLOS in user space.

In 6G, the UDP-layer will also have to be supported for the 6G RAN-CN U-plane interface for the same reasons as in 5G.

In summary, as the baseline, 6G can use the same user-plane protocol stack for the RAN-CN interface as 5G. Figure 1 shows this protocol stack.

Figure 1: Baseline protocols stack option for 6G RAN-CN U-plane interface

Observation 2: As in 5G, the 6G U-plane of the RAN-to-CN interface needs to support a tunneling mechanism with an additional protocol header to carry information, e.g., related to TEID, QoS, etc.

Proposal 3a: For the U-plane of the RAN-to-CN interface, RAN3 to consider GTP-U/UDP/IP as the default protocol stack.

In case protocols evolve over the time frame of 6G, it would be beneficial if the U-plane stack used by the RAN-CN interface could adapt to such evolution.

Proposal 3b: For the U-plane, RAN3 to consider how to be forward-compatible to support future protocol stacks.

For 5G, TS 33.501 mandates NG-U to be security-protected with IPsec. RAN3 should ask SA3 whether this security protection will also be mandated for the 6G RAN-CN C-plane interface.

Proposal 4: RAN3 to ask SA3 whether for the security protection of the 6G RAN-CN C-plane interface, support of IPsec and Datagram Transport Layer Security (DTLS) will be mandated.

## 2.3	Control plane

In 5G, the RAN-CN interface uses NG-C for control-plane traffic. NG-C carries NGAP over SCTP/IP.

NGAP represents an application-layer protocol, which carries C-plane messages using ASN.1 encoding.

The SCTP layer supports reliable in-order data delivery. SCTP provides other benefits such as head-of-line blocking by applying stream-based data delivery, and support of IP multi-homing.

While the legacy 5G C-plane protocol stack works well for appliance-like RAN deployments, it will invoke significant shortcomings when used for cloud-native RAN deployments for the following reasons:

- Many cloud-native solutions, such as Kubernetes, do not support SCTP.

  - Kubernetes is used to automate the deployment, scaling, and management of containerized applications, acting as a container orchestration platform. It enables organizations to run applications consistently across different environments, provides features like automatic self-healing and load balancing, and is crucial for managing microservices and cloud-native applications.

- Message brokers used in cloud-native environments for scalability typically operate on HTTP layer.

  - Message brokers are used to enable communication between different applications and microservices by acting as an intermediary that receives, routes, and delivers messages, promoting decoupling, scalability, and reliability in distributed systems.

Observation 3: SCTP is not well supported in cloud-native environments (e.g., to enable containerization using Kubernetes).

Observation 4: Many message brokers in cloud-native environments do not work without HTTP layer.

To address these issues, RAN3 should consider the following:

- Replace SCTP with a cloud-friendly transport protocol, such as TCP or QUIC.

- Add HTTP on top of the transport protocol to allow for cloud-native deployments where this protocol layer is required (e.g., as for message brokers).

Note that these protocol changes would still support appliance-like deployments if someone wants to deploy appliance based RAN.

Opposed to SCTP, TCP and QUIC do not provide native support for IP multi-homing. IP multi-homing, however, could be enabled by allowing the end hosts to establish multiple parallel TCP or QUIC connections for a given application layer association. This principle has already been supported for SCTP in 5G (TS 38.412). Alternatively, multipath extensions could be used for TCP. For QUC, it is expected that the IETF will standardize multi-path extensions rather soon.

TCP does not support multi-stream support, and therefore, head-of-line blocking cannot be avoided within a single TCP stream. It is possible, however, to establish separate TCP connections for each stream to achieve the same functionality. QUIC does natively support multiple streams, and it can therefore avoid head-of-line blocking.

HTTP supports ASN.1 and other encodings of the application layer information.

Observation 5: Replacing SCTP with HTTP/TCP or HTTP/QUIC enables support of both, appliance-like and cloud-native RAN deployments.

Observation 6: HTTP supports ASN.1 and other encodings of the application layer information.

Proposal 5a: For the C-plane of the 6G RAN-to-CN interface to use HTTP/TCP or HTTP/QUIC instead of SCTP as the default protocol stack.

As QUIC has been developed over the last 10-12 years, i.e., the time frame of 5G, it can be expected that protocols will evolve over the time frame of 6G. It would therefore be beneficial if the C-plane protocols used by the RAN-CN interface could adapt to future evolution.

Proposal 5b: For the C-plane, RAN3 to consider how to be forward-compatible to support future protocol stacks.

Figure 2 shows the three protocol stacks discussed.

Figure 2: Protocols stack options for 6G RAN-CN C-plane interface

For 5G, TS 33.501 mandates NG-C to be security-protected with D-TLS and IPsec. Both security solutions will work with the above protocol options proposed. QUIC already has TLS integrated.

Observation 7: For the C-plane of the 5G RAN-CN interface, TS 33.501 mandates support of IPsec as well as DTLS. Both security protocols can also be used HTTP/TCP. IPsec can be used for HTTP/QUIC. TLS is already integrated with QUIC.

Proposal 6: RAN3 to ask SA3 whether for the security protection of the 6G RAN-CN C-plane interface, support of IPsec and DTLS will be mandated.

Table 1 shows a comparison of the three protocol stack options for the RAN-CN C-plane interface.

|  | RNL - SCTP - IP | RNL - HTTP - TCP - IP | RNL - HTTP - QUIC - IP |
| --- | --- | --- | --- |
| Supports appliance-like RAN deployments | Yes | Yes | Yes |
| Support cloud-native RAN deployments | No | Yes | Yes |

# 3	Conclusion

This document discussed options for the 6G RAN-CN interface. The following proposals have been made:

On P2P vs. SBI:

Observation 1: Some operators may prefer to deploy 6G RAN NFs in cloud-native environments and leverage commercial cloud solutions (containers, message brokers, load balancers), while others may prefer deployment of 6G RAN NFs as appliances.

Proposal 1: The 6G RAN-CN interface should allow for virtualized RAN deployments that aim to leverage commercial cloud-native solutions (containers, message brokers, load balancers) and it should also support traditional RAN deployment approaches.

Proposal 2: RAN3 to identify the relevant requirements for the 6G RAN-CN interface to support virtualized RAN deployments that leverage commercial cloud-native solutions.

Proposal 2: RAN3 to identify the relevant requirements for the 6G RAN-CN interface to support virtualized RAN deployments that leverage commercial cloud-native  solutions.

On user plane:

Observation 2: As in 5G, the 6G U-plane of the RAN-to-CN interface needs to support a tunneling mechanism with an additional protocol header to carry information, e.g., related to TEID, QoS, etc.

Proposal 3a: For the U-plane of the RAN-to-CN interface, RAN3 to consider GTP-U/UDP/IP as the default protocol stack.

Proposal 3b: For the U-plane, RAN3 to consider how to be forward-compatible to support future protocol stacks.

Proposal 4: RAN3 to ask SA3 whether for the security protection of the 6G RAN-CN C-plane interface, support of IPsec and Datagram Transport Layer Security (DTLS) will be mandated.

On control plane:

Observation 3: SCTP is not well supported in cloud-native environments (e.g., to enable containerization using Kubernetes).

Observation 4: Many message brokers in cloud-native environments do not work without an HTTP layer.

Observation 5: Replacing SCTP with HTTP/TCP or HTTP/QUIC enables support of both, appliance-like and cloud-native RAN deployments.

Observation 6: HTTP supports ASN.1 and other encodings of the application layer information.

Proposal 5: For the C-plane of the 6G RAN-to-CN interface to select HTTP/TCP or HTTP/QUIC instead of SCTP as the default protocol stack.

Proposal 5b: For the C-plane, RAN3 to consider how to be forward-compatible to support future protocol stacks.

Observation 7: For the C-plane of the 5G RAN-CN interface, TS 33.501 mandates support of IPsec as well as DTLS. Both security protocols can also be used HTTP/TCP. IPsec can be used for HTTP/QUIC. TLS is already integrated with QUIC.

Proposal 6: RAN3 to ask SA3 whether for the security protection of the 6G RAN-CN C-plane interface, support of IPsec and DTLS will be mandated.

# 4	References

[1]  RP-251881, Study on 6G Radio, 3GPP TSG RAN #108, Prague, Czech Republic, June 2025. Further updates in RP-25912, 3GPP TSG RAN #109, Beijing, China, September 2025.

[2]  RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G, 3GPP TSG RAN #109, Beijing, China, September 2025.