3GPP TSG-RAN3 Meeting #129-bis	R3-256577

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item: 	10.4

Source: 	Qualcomm Incorporated

Title: 	6G RAN internal functional split and interfaces

Document for:	Other

# Background

6G RAN WG SID approved in [1], RP-252912, has the below objectives on 6G RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discuss the details on 6G RAN internal architecture, functional split and interfaces support.

# Discussion

## 6G RAN internal architecture

In 5G, both vertical (CU-DU) and horizontal (CU-CP, CU-UP) architecture splits are defined for the gNB as shown in Figure 1.

Figure 1: Overall architecture for separation of gNB-CU-CP and gNB-CU-UP

## Vertical architecture split

In 5G, the RAN architecture was designed with a clear functional and protocol layer split, along with standardized interfaces for inter-node communication. This enabled operators to choose between different deployment models based on their needs. For example, they could opt for an integrated gNB, where the Central Unit (CU) and Distributed Unit (DU) are deployed together, or a disaggregated gNB, where the CU (handling RRC/PDCP) and DU (handling RLC/MAC/High PHY) are deployed at separate sites.

Following are typical example deployment scenarios:

- CU can be deployed in cloud, DU can be deployed at the cell site and RF components at the top of the cell tower.

- CU can be deployed deep in the cloud, DU can be deployed in edge cloud to provide better scheduling latency and also allows to make use of cloud computing power by sharing common pool of hardware resources across multiple DUs and RF components can be deployed at cell site.

- In another deployment, both CU + DU can be deployed together in cloud as NFV and flexible sharing of cloud compute resources and RF components at the cell site.

Split gNB architecture allowed operators the flexibility to select from various deployment models based on their specific needs.  It also enables flexible usage of shared or distributed hardware resources across multiple CUs or DUs.

Observation 1: RAN internal functional split in 5G enables disaggregated and flexible RAN deployments.

Observation 2: RAN internal functional split allows for both integrated gNB implementations (e.g., merged CU/DU) ,and split gNB implementations (e.g., separate CU/DU across different vendors) and allowed operators to use shared or distributed pooling of hardware resources for CU/DU deployments.

Operators have the flexibility to choose RAN nodes from either a single vendor or multiple vendors. In current 5G rollouts worldwide, we see a mix of both approaches—some networks use single-vendor RAN solutions, while others adopt multi-vendor setups with disaggregated CU-DU components.

In 6G, it's essential to retain the flexibility and multi-vendor ecosystem that 5G has successfully introduced. Taking a step back from these advancements would restrict operator vendor choices and slow down technological progress. Instead, 6G should build on this strong foundation of 5G disaggregated RAN architecture, offering even greater modularity, openness, and deployment flexibility.

Observation 3: Commercial gNB deployments today include both single vendor and multi-vendor disaggregated CU-DU split as well as integrated gNB implementations.

Proposal 1: 6G RAN shall support standardized CU-DU functional split as in 5G to enable operators to deploy 6G RAN in the same manner as the 5G RAN.

## Horizontal architecture split

In addition to the CU-DU split, 5G introduced a further functional split within the CU dividing it into CU-CP (Control Plane) and CU-UP (User Plane). This design allows operators to scale control and user plane resources independently. It enabled independent evolution of control plane and user plane nodes.  It also enables flexible deployment strategies: for instance, CU-UP can be placed closer to the edge or tower to meet ultra-low latency requirements for URLLC, while a centralized CU-UP may be more suitable for eMBB services.

As 6G aims to support additional performance-centric services like immersive communications, integrated sensing and communications, and AI-native communications, the user data volumes and service diversity will be significantly higher compared to 5G. If large amounts of 6G user plane traffic are to be routed through a combined CP/UP node i.e. CU, it can lead to overload and performance bottlenecks, affecting service quality. Thus, keeping 6G CU-CP and CU-UP separate like 5G, helps avoid this issue by allowing each to scale and operate independently based on service demands. Further, the CU-UP and CU-CP separation allows the 6G UE to move the control plane and user plane sessions independently, i.e. CU-CP controlling the UE can be changed without changing the UE’s user plane anchor CU-UP and not impact the user plane performance.

In support of these arguments, RAN#109 plenary discussion included the below requirement in RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G:

-	The 6G RAN architecture shall allow for control plane and user plane separation.

Thus, it is essential for 6G RAN to support the CU-CP and CU-UP split.

Observation 4: The CU-UP (User plane) and CU-CP (Control plane) split was defined in 5G RAN to scale User plane and Control plane resources independently and allowed CP and UP nodes to evolve independently.

Observation 5: CU-UP and CU-CP split allows the operator to deploy multiple CU-UPs based on use case, e.g.CU-UP closer to edge/tower for URLLC or centralized CU-UP for eMBB.

Proposal 2: 6G RAN shall support standardized CU-UP and CU-CP functional split as in 5G so that operators can deploy 6G RAN in the same manner as the 5G RAN.

## Further architecture considerations

During the 5G standardization phase, multiple higher-layer and lower-layer split options for RAN functional disaggregation, Options 1 through 7 were evaluated, as documented in the 3GPP study report TR 38.801 [2]. The study concluded that Option 2, which defines a higher-layer split between the Central Unit (CU) and Distributed Unit (DU), would be supported in the specifications. This split has since been widely adopted in commercial gNB deployments across the world, with no significant impact observed on performance or scalability across various use cases.

In the context of 6G, the RAN Working Group Study Item Description (SID) [1] provides clear guidance as below:

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.

Given this direction, it is reasonable to consider that the same or a similar higher-layer CU-DU functional split can effectively support the diverse and demanding use cases of 6G. There is no compelling need to introduce additional disaggregation options, which could complicate implementations and reduce interoperability.

Proposal 3: Support standardization of 6G CU-DU higher layer functional split similar to 5G option 2.

In 5G, there is a strict restriction on the association between the gNB-CU, gNB-DU and gNB CU-CP and gNB CU-UPs as specified in TS38.401, [3]:

-	One gNB-DU is connected to only one gNB-CU-CP;

-	One gNB-CU-UP is connected to only one gNB-CU-CP;

While this simplifies the RAN architecture, it introduces limitations. Specifically, it restricts flexibility in CU selection during UE mobility.   We recommend that RAN3 study the possibility of introducing flexible association models in 6G—both between DUs and CUs, and between CU-UPs and CU-CPs.

Proposal 4: Study flexibility in the associations between 6G DUs and 6G CUs as well as between 6G CU-UPs to 6G CU-CPs.

## 6G RAN internal interfaces

The details of the 6G RAN internal interfaces should be discussed once RAN3 has settled onthe overall 6G RAN architecture and functional split.

Proposal 5: RAN3 to study the RAN internal interfaces after convergence has been achieved on the 6G RAN architecture and 6G RAN functional split.

# 3 Conclusion

Below are the observations and proposals discussed in this document.

Observation 1: RAN internal functional split in 5G enables disaggregated and flexible RAN deployments.

Observation 2: RAN internal functional split allows for both integrated gNB implementations (e.g., merged CU/DU) ,and split gNB implementations (e.g., separate CU/DU across different vendors) and allowed operators to use shared or distributed pooling of hardware resources for CU/DU deployments.

Observation 3: Commercial gNB deployments today include both single vendor and multi-vendor disaggregated CU-DU split as well as integrated gNB implementations.

Proposal 1: 6G RAN shall support standardized CU-DU functional split as in 5G to enable operators to deploy 6G RAN in the same manner as the 5G RAN.

Observation 4: The CU-UP (User plane) and CU-CP (Control plane) split was defined in 5G RAN to scale User plane and Control plane resources independently and allowed CP and UP nodes to evolve independently.

Observation 5: CU-UP and CU-CP split allows the operator to deploy multiple CU-UPs based on use case, e.g.CU-UP closer to edge/tower for URLLC or centralized CU-UP for eMBB.

Proposal 2: 6G RAN shall support standardized CU-UP and CU-CP functional split as in 5G so that operators can deploy 6G RAN in the same manner as the 5G RAN.

Proposal 3: Support standardization of 6G CU-DU higher layer functional split similar to 5G option 2.

Proposal 4: Study flexibility in the associations between 6G DUs and 6G CUs as well as between 6G CU-UPs to 6G CU-CPs.

Proposal 5: RAN3 to study the RAN internal interfaces after convergence has been achieved on the 6G RAN architecture and 6G RAN functional split.

# 4 References

[1] RP-252912, Revised SID: Study on 6G Radio