3GPP TSG-RAN WG3 Meeting #129bis	R3-256590

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	[TP to BL pCR] General principles and functions on RAN-CN interface

Agenda Item:	10.3.1

Source:	Huawei, Jio Platforms

Document for:	Agreement

# Introduction

In the 6G SID [1], the following bullet can be found:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide our analyses on the general principles and functions on RAN-CN interface.

# Discussion

## 2.1	RAN-CN interface principles

In 5G, a list of principles can be found for NG interface in TS 38.410 (similar principle as captured in TR 38.801), although there are few options of 6G RAN-CN interface mentioned during the RAN/SA discussion, from our view, most of these basic principles should be supported by default.

The general principles for the specification of the NG interface are as follows: - the NG interface is open; - the NG interface supports the exchange of signalling information between the NG-RAN and 5GC; - from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC; - the NG interface supports control plane and user plane separation; - the NG interface separates Radio Network Layer and Transport Network Layer; - the NG interface is future proof to fulfil different new requirements and support of new services and new functions; - the NG interface is decoupled with the possible NG-RAN deployment variants; - the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

For the NG interface principle #3, i.e., “from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC.”, we think it should still be applicable for 6G RAN-CN interface.

Proposal 1: Reuse the general principles for theNG interface for 6G RAN-CN interface as much as possible, in particlar:

- the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

- the 6G RAN-CN interface supports control plane and user plane separation;

- the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

- the 6G RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions;

- the 6G RAN-CN interface is decoupled with the possible NG-RAN deployment variants;

- the 6G RAN-CN interface, from locial standpoint, is a point-to-point interface between an 6G RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN node and CN node.

Besides the principles mentioned in proposal 1, regardless of legacy service/features or new 6G service/features, from our view, the 6G RAN-CN interface has to also support reliable signalling transmission, support the large scale of RAN nodes deployment, and support clear boundaries between RAN and CN.

Proposal 2: The 6G RAN-CN interface, supports reliable signalling transmission, large scale of RAN nodes deployment, and clear boundaries between RAN and CN.

The other additional principlies, depend on the further discussion of the 6G new services, such as sensing, etc.

About the “Data transfer design to support various types of data (e.g., AI/ML, Sensing, etc)” listed in the SID [1], whether there will be additional principle to be added for it is subject to further discussion of that topic.

Proposal 3: Other additional principles dedicated to 6G new services (e.g., sensing, etc) can be further discussed.

## 2.2	RAN-CN interface functions

For NG interface in 5G, there are lists of functions mentioned in TR 38.801 section 7.3.3, and TS 38.410 section 4.4 and 5.

NG-C interface supports following functions: - Interface management: The functionality to manage the NG-C interface; - UE context management : The functionality to manage the UE context between the New RAN and CN; NOTE 1: The UE context information may include roaming and access restriction and security information. NOTE 2: The UE context information may include the information related with network slicing. - UE mobility management: The functionality to manage the UE mobility for connected mode between the New RAN and CN; - Transport of NAS messages: procedures to transfer NAS messages between the CN and UE; - Paging: The functionality to enable the CN to generate Paging messages sent to the New RAN and to allow the New RAN to page the UE in RRC_IDLE state; - PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective New RAN resources that are made of data flows carrying UP traffic. - Configuration Transfer: the functionality to transfer the New RAN configuration information (e.g. transport layer addresses for establishment of Xn interface) between two New RAN nodes via the NGC. NOTE 3: It can be discussed on whether congestion and overload control function is needed or not in normative phase.

These listed functions are basic ones which should also be supported in 6G:

Proposal 4: RAN-CN interface supports the following basic functions:

-	Interface management: The functionality to manage the RAN-CN Control Plane interface;

-	UE context management: The functionality to manage the UE context between the RAN and CN;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE;

-	Paging: The functionality to send paging requests to the RAN nodes involved in the paging area;

-	PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective RAN resources.

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of Xn* interface) between two RAN nodes via the CN.

Similar as additional principles, other additional functions, depend on the further discussion of the 6G new services, such as sensing, etc.

Proposal 5: Other additional functions dedicated to 6G new services (e.g., sensing, etc) can be further discussed.

# Conclusion

In this contribution, we discuss the general principles and requirements on RAN-CN functional split and interface, and get the following proposals:

RAN-CN interface Principles

Proposal 1: Reuse the general principles for theNG interface for 6G RAN-CN interface as much as possible, in particlar:

- the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

- the 6G RAN-CN interface supports control plane and user plane separation;

- the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

- the 6G RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions;

- the 6G RAN-CN interface is decoupled with the possible NG-RAN deployment variants;

- the 6G RAN-CN interface, from locial standpoint, is be a point-to-point interface between an 6G RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN node and CN node.

Proposal 2: The 6G RAN-CN interface, supports reliable signalling transmission, large scale of RAN nodes deployment, and clear boundaries between RAN and CN.

Proposal 3: Other additional principles dedicated to 6G new services (e.g., sensing, etc) can be further discussed.

RAN-CN interface Functions

Proposal 4: RAN-CN interface supports the following basic functions:

- Interface management: The functionality to manage the RAN-CN Control Plane interface;

- UE context management: The functionality to manage the UE context between the RAN and CN;

- UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

- Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE;

- Paging: The functionality to send paging requests to the RAN nodes involved in the paging area;

- PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective RAN resources.

- Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of Xn* interface) between two RAN nodes via the CN.

Proposal 5: Other additional functions dedicated to 6G new services (e.g., sensing, etc) can be further discussed.

# Reference

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

# Text Proposal for 6G TR (based on R3-255517)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

## 5.3  		Overall RAN architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

### 5.3.1 	Functional Split RAN-CN

Editor’s note: The aim of this section is to describe functions which may be split between RAN and CN

### 5.3.2 	RAN Functions

Editor’s note: The aim of this section is to describe functions in RAN

# 6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

## 6.1	General Principles and Requirements

## 6.2 		RAN-CN interface

### 6.2.x	General principles

The general principles for the specification of the 6G RAN-CN interface are as follows, in particlar:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions;

-	the 6G RAN-CN interface is decoupled with the possible NG-RAN deployment variants;

-	the 6G RAN-CN interface, from locial standpoint, is a point-to-point interface between an 6G RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the 6G RAN node and CN node;

-	the 6G RAN-CN interface, supports reliable signalling transmission, large scale of RAN nodes deployment, and clear boundaries between RAN and CN.

NOTE x:	Principles for mobility and new services (e.g., sensing, etc) can be added after the discussion of these topics.

### 6.2.y	RAN-CN interface Functions

RAN-CN interface supports following functions:

-	Interface management: The functionality to manage the RAN-CN Control Plane interface;

-	UE context management	: The functionality to manage the UE context between the RAN and CN;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE;

-	Paging: The functionality to send paging requests to the RAN nodes involved in the paging area;

-	PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective RAN resources.

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of Xn* interface) between two RAN nodes via the CN.

NOTE y:	Functions for new services (e.g., sensing, etc) can be added after the discussion of these topics.

### 6.2.1	RAN-CN interface options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services and include control plane and user plane aspects considering interim milestone.

### 6.2.2	Evaluation of RAN-CN interface options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 5.2.1

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>