3GPP TSG-RAN WG3 Meeting #129bis	R3-256597

Prague, CZ, 13th – 17th October 2025

Agenda item:	10.5.1

Source: 	Samsung

Title: 	Discussion on AI/ML use cases for 6G

Document for:	Discussion and decision

Introduction

During the work of Rel-18 and Rel-19, RAN3 has established a data collection framework for AI/ML for NG-RAN, as per the discussions for the specific use cases:

- Rel-18: AI/ML-based Network Energy Saving, AI/ML-based Load Balancing, AI/ML-based Mobility Optimization

- Rel-19: AI/ML-based Slicing, AI/ML-based Coverage and Capacity Optimization

Since the 6G study has officially started, RAN3 is supposed to study on further AI/ML use cases according to the requirements and services in 6G. In general, we would consider AI/ML for mobility and AI/ML-based Network Energy Saving as the starting point for 6G use cases for AI/ML. This discussion section provides our detailed discussion on the potential 6G AI/ML use cases in our view.

Discussion

2.1 AI/ML-based Mobility Optimization

In Rel-18, data collection for Mobility Optimization was discussed and enabled by introducing corresponding Data Collection procedures and Cell based UE Trajectory Prediction for handover preparation procedure in XnAP. The collection of measured UE trajectory allows the training and inference of the AI model which generates the predicted UE trajectory in a per-cell manner, and the predicted UE trajectory would be useful for the RAN node to make appropriate mobility decisions, which could help to minimize the possibility of call drops, RLFs for users[1].

As for mobility scenarios in 5G, there are quite a few mobility types including L3 HO, LTM, CHO, DAPS, etc. When it comes to the mobility evolution in 6G, it is highly possible that the mobility related procedures could be further enhanced or simplified somehow, and also for the purpose of reducing the latency or interruption during mobility. It might seem a bit too early to make an assumption, but we tend to believe that the mobility framework would be reshaped in the 6G phase, along with the evolution of network architecture and new requirements and services in 6G.

Observation 1: It is highly possible that mobility framework would be reshaped or enhanced in 6G.

With this basic prospect of 6G mobility, the AI/ML-based mobility based on 6G mobility framework could also play a key role in the process of 6G evolution. The mobility of users would always be an important issue that the network would confront with, especially in the new generation when more types of mobile devices are connecting to the network. In 5G, AI/ML-based mobility optimization enables the cell-based UE trajectory prediction based on 5G network architecture. When it comes to 6G, the cell-based UE trajectory prediction, in our mind, can be taken as a start point for the 6G study on AI/ML for mobility, with the basic understanding that cell would remain a basic granularity for network resource management and radio access. The prediction of the cell that the UE might connect to after the HO would still be beneficial for the network performance, as is in 5G. The AI/ML-based mobility optimization in 5G focuses on the handover scenario for UEs in RRC_CONNECTED state, while in 6G, it depends on how the new mobility mechanism would be framed. For example, if LTM is considered as a dominant mobility mechanism for 6G mobility, the prediction for cell switch decision would be a main purpose in 6G AI/ML-based mobility, and correspondingly, the historical cell switch command decision shall be collected as the input for AI/ML model training/inference.

Proposal 1: The cell-based UE trajectory prediction can be taken as a start point for 6G AI/ML-based mobility optimization, based on 6G mobility framework.

To support the prediction of cell-based UE trajectory, taking R18 AI/ML data collection mechanism as a baseline, the UE history information from neighbour node shall be provided as one of the input information for AI/ML model training/inference. Information from local node, e.g., resource status, UE location and UE mobility history information are also needed for model training/inference, similar as in Rel18. Besides, the other kinds of information that may be collected via 6G radio interface could also be evaluated and considered as input for AI/ML model training/inference. For instance, with the sensing session performed in the RAN node or UE side, the RAN node may collect sensing data or reports, some information of which could also be applied to the AI/ML model training/inference for mobility optimization. Specifically, the sensing data could comprise the information related to the environment or objects in the environment which could impact the mobility of the UE, and using such information as input would help the AI/ML to generate a more accurate result for mobility optimization. Although it is too early to discuss the details of the sensing information for AI/ML model training, our view is that it could be beneficial to consider using potential data collected at RAN side for AI/ML mobility optimization. Similarly, since 6G new service/data plane has been put on the table for the discussion of 6G data transmission, we also think it possible to leverage the data collected for service/data plane as part of the input data for AI/ML mobility in 6G. A figure that depicts the potential input and output for mobility optimization use case in 6G is shown below:

Figure 1. Input and output of 6G AI/ML-based Mobility Optimization

Proposal 2: The input of AI/ML model training/inference for Mobility Optimization shall be based on the potential data collected at RAN side in 6G, e.g., considering sensing and other information via service/data plane.

As for the output of AI/ML model, which starts from the UE trajectory prediction, there could be more information related to the mobility strategy and so on. One aspect of mobility optimization can be associated with the resource preparation or pre-allocation. Assisted with the predicted cell that the UE would be connected to, the corresponding RAN node minimize the resource pre-allocation in candidate cells in advance before the UE connects to the target cell, which could help reduce the interruption of handover. If we consider native AI in 6G, the native AI function in the RAN node would enable more intelligent decisions, such as more intelligent resource allocation for the purpose of mobility optimization.

Proposal 3: Intelligent Resource allocation could be one of the outputs of 6G AI/ML-based mobility optimization, especially when native AI is deployed at RAN node.

2.2 AI/ML-based Network Energy Saving

In 6G, energy consumption would remain a crucial factor that would be concerned by the operators, as energy consumption always matters to the operational cost and the environment. As 6G aims to deliver higher data rates, ultra-low latency, and massive connectivity, and considering some new services like sensing which would require constant sensing signal transmission and measurements, the energy consumption of network infrastructure is expected to increase significantly. Although, there are quite some network energy saving mechanisms designed in previous releases, the involvement of AI/ML is not sufficient, if we consider the potential benefit that AI/ML could bring in monitoring energy consumption and making network energy saving optimization decisions. There is actually not much discussion about how the AI/ML could assist with the mechanisms of Network Energy Saving, since the AI/ML-base Network Energy Saving was only discussed in Rel-18 for 5G standalone architecture, and in Rel-19 for split architecture. In other words, Network Energy Saving has been a significant concern for quite a few releases, however the AI/ML assistance for the network energy saving was only introduced for two releases, with some basic discussion of energy consumption, not touching various energy saving mechanisms and scenarios.

Observation 2: Energy consumption would remain a crucial factor that is associated with operational expenses, and there was not enough discussion of network energy saving involving the assistance of AI/ML in previous releases.

With the above observation, it is in our mind the time to start a new page for the AI/ML-based Network Energy Saving in 6G, with more mechanisms and scenarios to take into account. As a start point, for example, the mechanism of cell activation and deactivation, which was a basic mechanism supported since Rel-15 for network energy saving, could be considered in 6G early stage. If a RAN providing coverage collects enough information related to a RAN node providing capacity, it can use the AI/ML function to generate an intelligent cell activation/deactivation strategy for network energy saving, and at the same time make sure the coverage and capacity provided by the serving cells are not impacted. With the assistance of AI/ML, more intelligent and efficient decisions and actions could be taken by the RAN node, to monitor the network energy, and also to provide guaranteed services to the users without degrading network performance.

Figure 2. cell activation and deactivation for NES

Proposal 4: AI/ML-based Network Energy Saving can be further considered in 6G, based on 6G framework.

Similar to AI/ML-based Network Energy Saving in R18, the input for AI/ML-based Network Energy Saving in 6G could include UE trajectory information, measured Energy Cost, resource status information and so on. As has been discussed above for AI/ML-based mobility optimization in 6G, for 6G AI/ML-based Network Energy Saving, we also think other information collected by the RAN node via sensing or service/data plane transmission shall be evaluated and considered as the input information for AI/ML-based Network Energy Saving.

Proposal 5: The input of AI/ML model training/inference for Network Energy Saving shall be based on the potential data collected at RAN side in 6G, e.g., considering sensing and other information via service/data plane.

For output information, Energy Saving related actions such as cell activation/deactivation, would also be reckon necessary in 6G. With the prediction of a well-trained AI/ML model, the network can generate fair strategies for cell activation/deactivation that maximizes the energy efficiency, minimizes the waste of resources, while provisioning sufficient coverage and capacity to UEs in need, without degrading the network performance.

A figure depicting the input and output of 6G AI/ML-based Network Energy Saving is shown below:

Figure 3. Input and output of 6G AI/ML-based Network Energy Saving

Aside from discussion above, other NES mechanisms, can also be considered in 6G and see how AI/ML could help for optimizations.

Proposal 6: Network Energy Saving strategies such as cell activation/deactivation can be considered as the output for AI/ML based Network Energy Saving in 6G.

Proposal 7: Other AI/ML-based NES optimizations can be discussed based on 6G framework.

The text proposals for the TR of 6G radio is provided in section 5 of this paper.

Proposal 8: Agree on the text proposals for AI use cases in section 5.

Conclusion

In this paper, the following proposals are provided:

Observation 1: It is highly possible that mobility framework would be reshaped or enhanced in 6G.

Proposal 1: The cell-based UE trajectory prediction can be taken as a start point for 6G AI/ML-based mobility optimization, based on 6G mobility framework.

Proposal 2: The input of AI/ML model training/inference for Mobility Optimization shall be based on the potential data collected at RAN side in 6G, e.g., considering sensing and other information via service/data plane.

Proposal 3: Intelligent Resource allocation could be one of the outputs of 6G AI/ML-based mobility optimization, especially when native AI is deployed at RAN node.

Observation 2: Energy consumption would remain a crucial factor that is associated with operational expenses, and there was not enough discussion of network energy saving involving the assistance of AI/ML in previous releases.

Proposal 4: AI/ML-based Network Energy Saving can be further considered in 6G, based on 6G framework.

Proposal 5: The input of AI/ML model training/inference for Network Energy Saving shall be based on the potential data collected at RAN side in 6G, e.g., considering sensing and other information via service/data plane.

Proposal 6: Network Energy Saving strategies such as cell activation/deactivation can be considered as the output for AI/ML based Network Energy Saving in 6G.

Proposal 7: Other AI/ML-based NES optimizations can be discussed based on 6G framework.

Proposal 8: Agree on the text proposals for AI use cases in section 5.

References

[1] TR 37.817 Study on enhancement for Data Collection for NR and EN-DC

TP to TR 38.8XX

<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>

- 7	AI/ML for RAN

7.1 	AI/ML use cases

7.1.1 AI/ML-assisted Mobility Optimization

7.1.1.1 Use case description

Mobility management is the scheme to guarantee the service-continuity during the mobility by minimizing the call drops, RLFs, unnecessary handovers, and ping-pong. The AI/ML-based mobility optimization is specified in 5G, which supports the cell-based UE trajectory prediction. Considering the potential evolution of 6G mobility framework, the AI/ML-based mobility optimization would still play a significant role for the mobility enhancement.

7.1.2 AI/ML-assisted Network Energy Saving

7.1.2.1 Use case description

Network energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales. As 6G aims to deliver higher data rates, ultra-low latency, and massive connectivity, and considering some new services like sensing which would require constant sensing signal transmission and measurements, the energy consumption of network infrastructure is expected to increase significantly. The AI/ML-based Network Energy Saving in 6G could further assist with the network to generate intelligent energy saving strategies, while providing guaranteed services to the users without degrading network performance

<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>