# 3GPP TSG-RAN WG3 Meeting #129-bis														R3-256605

# Prague, Czech Republic, 13th – 17th Oct,2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0176 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.483 to UE performance per DRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Unicom, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the reply LS S5-253854, UL PDCP SDU Loss Rate in split architecture and non-split architecture are defined in TS 28.558, and DL PDCP SDU Drop Rate in split architecture and non-split architecture are defined in TS 28.558. And SA5 confirms RAN3’s assumption that packet delay per UE level is derived from packet delay per DRB. Therefore, it is proposed to add reference to TS 28.558 for average packet delay and average packet loss. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the semantic description of Average Packet Loss DL IE to refer to DL PDCP SDU Drop Rate in TS 28.558. - Update the semantic description of Average Packet Loss UL IE to refer to UL PDCP SDU Loss Rate in TS 28.558. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of per DRB UE performance is not clear in the current specificaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.149 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>

9.3.1.149	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.150 |  |
| Average Packet Loss DL | O |  | INTEGER(0..1000000, …) | Corresponds to DL PDCP SDU Drop Rate as specified in TS28.558 [58] clause 6.3.1.6.2.1. |
| Average Packet Loss UL | O |  | INTEGER(0..1000000, …) | Corresponds to UL PDCP SDU Loss Rate as specified in TS28.558 [58] clause 6.3.1.3.1. |

<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>