3GPP TSG-RAN3 Meeting #129bis	R3-256609

Prague, Czech Republic, Oct 13 - 17, 2025

Agenda Item:	12.2.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Multi-hop UE Trajectory Prediction and Feedback

Document for:	Discussion/Decision

# Background

UE Trajectory feedback collection was discussed and specified in Release 18. However, only single hop (within the same gNB) feedback collection was specified in Release 18. Multi hop feedback (across gNBs) collection was left over to Release 19. Again, in Release 19 discussion on multi hop UE Trajectory feedback collection continued, however companies had no common understanding if the solution will be simple or not. There was a comparison between hop-by-hop feedback collection and direct feedback collection in terms of minimal impact. Due to no consensus, this topic was left out of Release 19.

For the sake of completion of UE Trajectory prediction and feedback collection specified in Release 18, Multi hop UE trajectory feedback collection is included in the scope of Release 20 NG-RAN AI/ML scope as shown below.

Release 20 RP-252867 The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts. Release19 RAN3#124 No consensus on the solution the multiple-hop UE trajectory. Multiple-hop UE trajectory is not supported in R19.

In this paper we discuss our views on hop by hop and direct feedback solution and comparisons between them to address Multi hop UE Trajectory feedback across gNBs.

# Discussion

## Direct UE Trajectory Feedback Collection

The source node which predicts the UE trajectory knows the cells and gNBs where the UE can possibly connect to in near future. Hence the node which predicts the UE trajectory and wants the feedback on the actual UE Trajectory can send Data collection Request to all the possible target nodes where the UE is predicted to connect to. The source node can also send Data Collection Request to all RAN nodes to which it is Xn connected to get feedback on UE Trajectory. UE stays in RRC Connected state for a short period of time and it is unlikely that UE moves beyond the Xn connected neighbouring gNBs. This is also very similar to RRC Inactive Context retrieval from the anchor node.

In the Handover Request along with the Data Collection ID, the UE context reference at the source node (UE ID at the source node) which collects the UE trajectory feedback from other nodes needs to be passed for UE identification at the Feedback collection node. Reference call flow is shown below.

The source node which predicts the UE trajectory knows the cells and gNBs where the UE is predicted to connect. Source gNB is also Xn connected to its neighbouring gNBs

UE stays in RRC Connected state for a short time. Hence it is unlikely that UE moves beyond the Xn connected neighbouring gNBs.

- The source gNB which collects the UE Trajectory Feedback configures all the predicted target gNBs or Xn Connected neighbouring gNBs with Data Collection Request for measured UE Trajectory data collection. The feedback collection is limited to only neighbouring gNBs which are directly connected to the source gNB via Xn interface. This is similar to RRC Inactive UE Context Retrieval over Xn.

- The source gNB (i.e which collects the UE Trajectory feedback) provides the Data collection ID at the source and the UE ID at the source in the Handover Request message to the target. There are only 2 new IEs which are needed in Handover Request for direct UE Trajectory feedback collection.

- When the UE goes to un-predicted target or to a target which is not Xn connected to the source or if the UE goes to RRC_Idle/inactive the feedback collection is terminated in the target, and the collected feedback is reported to the source.

## Comparison between hop-by-hop and direct Feedback

Hop by Hop Feedback

In hop by hop feedback collection every node in the UE Trajectory path configures the subsequent target node for UE Trajectory feedback. The feedback for UE Trajectory prediction is passed from current node to the previous node and the cycle continues until the feedback reaches the UE Trajectory prediction originating source.

Disadvantages:

- All the nodes in the UE Trajectory need to maintain UE context to forward the feedback to the UE Trajectory Prediction originating source node, which implies millions of UE context needs to be maintained until feedback collection period.

- It is unclear on how the Measured UE trajectory collection duration and number of visited cells is percolated from the UE Trajectory Prediction originating source node to the subsequent nodes.

- It is also unclear on how long each node in the UE Trajectory path needs to maintain the UE context.

Advantages:

- If the UE does not follow the predicted path, then the feedback collection can be triggered from the un-predicted node easily as it is hop by hop.

Big disadvantage in hop-by-hop feedback is all nodes in the UE trajectory path need to maintain the UE context for millions of UEs in the RAN node which is a huge overhead.

Direct Feedback

In direct feedback the UE Trajectory feedback is passed from the target node to the UE Trajectory prediction originating source node. The gNBs in the UE Trajectory path need not maintain the UE Context. The UE Trajectory prediction originating source node configures all the predicted targets with the necessary details to send the feedback directly to the source node.

Advantages:

- UE context needs to be retained only in the UE Trajectory Prediction originating source node and the subsequent nodes need not maintain the UE context which is a big advantage.

- Addition of the Data collection ID along with the Predicted node ID and source node UE ID in the Handover Request are the only stage 3 changes needed.

Disadvantages:

- If the UE goes to a gNB which is not Xn connected to the source gNB, then the feedback collection cannot continue dynamically and the feedback collection stops.

UE context need not be stored in all the nodes in the UE trajectory path and only the UE Trajectory Prediction originating source node needs to maintain the UE context.

- Direct feedback collection provides a huge advantage over hop-by-hop feedback collection by not maintaining many UE context(s) in all gNBs in the UE Trajectory path with minimal specification impact.

- RAN3 to agree to pursue direct feedback collection for multi-hop (across gNBs) UE Trajectory prediction and feedback for Rel-20.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The source node which predicts the UE trajectory knows the cells and gNBs where the UE is predicted to connect. Source gNB is also Xn connected to its neighbouring gNBs

Observation 2.	UE stays in RRC Connected state for a short time. Hence it is unlikely that UE moves beyond the Xn connected neighbouring gNBs.

Observation 3.	Big disadvantage in hop-by-hop feedback is all nodes in the UE trajectory path need to maintain the UE context for millions of UEs in the RAN node which is a huge overhead.

Observation 4.	UE context need not be stored in all the nodes in the UE trajectory path and only the UE Trajectory Prediction originating source node needs to maintain the UE context.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	The source gNB which collects the UE Trajectory Feedback configures all the predicted target gNBs or Xn Connected neighbouring gNBs with Data Collection Request for measured UE Trajectory data collection. The feedback collection is limited to only neighbouring gNBs which are directly connected to the source gNB via Xn interface. This is similar to RRC Inactive UE Context Retrieval over Xn.

Proposal 2:	The source gNB (i.e which collects the UE Trajectory feedback) provides the Data collection ID at the source and the UE ID at the source in the Handover Request message to the target. There are only 2 new IEs which are needed in Handover Request for direct UE Trajectory feedback collection.

Proposal 3:	When the UE goes to un-predicted target or to a target which is not Xn connected to the source or if the UE goes to RRC_Idle/inactive the feedback collection is terminated in the target, and the collected feedback is reported to the source.

Proposal 4:	Direct feedback collection provides a huge advantage over hop-by-hop feedback collection by not maintaining many UE context(s) in all gNBs in the UE Trajectory path with minimal specification impact.

Proposal 5:	RAN3 to agree to pursue direct feedback collection for multi-hop (across gNBs) UE Trajectory prediction and feedback for Rel-20.