3GPP TSG-RAN WG3 Meeting #129-bis	R3-256617

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	10.3.1

Source:	CATT

Title:	Discussion on general principles and requirements on RAN-CN

interface

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the RAN-CN function split, interface, protocol stack and procedures, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze the 6G RAN-CN interface aspects from RAN3 perspective, especially on the general principle and requirement. Base on the discussion, we will provide the observations and proposals accordingly.

2	Discussion

2.1 6G RAN-CN interface general principles and requirements

To make the discussion more efficient in the study phase, we should name the interface between 6G RAN and 6G Core network firstly. In following discussion, we simply named the interface between 6G RAN node and 6G Core network as NG+, which could be replaced if other name is adopted.

As discussed and proposed in the [6], the legacy NG-like way could be considered for the legacy functions (communication related functions as defined for NG). But for the new services (e.g. AI/ML, Sensing), we could consider direct connection and more efficient data transmission between 6G RAN node and corresponding function(s) in the 6GC.

Proposal 1: Consider legacy services/functions and new services separately when we define the 6G RAN-CN interface(s).

The general principles for the specification of the NG+ are as follows:

-	the NG+ interface is open;

-	the NG+ interface supports the exchange of signalling information between the 6G RAN and 6G CN;

-	for legacy functions, the NG+ is a point-to-point interface between a 6G RAN node and an 6GC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the 6G RAN and 6GC;

-    for new service functions, the NG+ is direct interface between a 6G RAN node and corresponding service function in 6G CN.

-	the NG+ interface supports control plane, user plane separation for legacy functions; [FFS for new service]

-	the NG+ interface separates Radio Network Layer and Transport Network Layer;

-	the NG+ interface is future proof to fulfil different new requirements and support of new services and new functions;

-	the NG+ interface is decoupled with the possible 6G RAN deployment variants;

-	the NG+ Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency for legacy functions. [FFS for new service]

Proposal 2: Adopt above principles and requirements for 6G RAN-CN interface.

2.3 6G RAN-CN function split

Compared to the RAN-CN function split in the NG interface, we assume the legacy function split as NG is still applicable for 6G communication related functions, e.g. RRM, RB control, Mobility Control, Measurement, etc. But for the new services, e.g. AI/ML, Sensing, new service functions will be introduced in the 6GC, and corresponding functions will also be included in 6G RAN node. The function split between 6G RAN and 6GC is illustrated with the figure below:

Figure 1. Functional Split between 6G RAN and 6GC

Observation 1: NG interface functions as defined for 5G could be taken as baseline for 6G RAN-CN interface design.

According to the figure above, the 6G RAN hosts the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in both uplink and downlink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF+ at UE attachment when no routing to an AMF+ can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF+(s);

-	Routing of Control Plane information towards AMF+;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state; [FFS]

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity; [FFS]

-	Interworking between 6GR and NR; [FFS]

-	 Functions for Sensing, e.g. Sensing related radio resource control, sensing TRP selection, sensing data collection, sensing data processing and report towards the Sensing Function in 6G CN.

-	Functions for AI/ML, e.g. data collection and data transmission between 6G RAN and the AI/ML related function in 6G CN.

-	Support of Positioning, including positioning resource allocation, positioning measurement, positioning report, between 6G RAN and Positioning related function in 6G CN.

It’s assumed that the legacy functions of 6G CN are similar to the functions in 5GC, as hosted by AMF, SMF, UPF. While the functions of AI/ML related function(s), Sensing Function, Positioning related function (e.g. LMF+) in 6GC to be further discussed, coordination with SA2 is needed.

Proposal 3: Take above function split between 6G RAN and 6GC as the start point for the RAN3 study.

2.4 6G RAN-CN Interface architecture

NG+ interface shall support both legacy communication based functions and new service functions. For the legacy functions, it could be supported via the control plane and user plane, just like NG-C/NG-U separation in 5G.

Proposal 4: Control Plane and User Plane separation should be adopted to support the legacy functions for 6G RAN-CN interface.

For new services such as AI/ML and ISAC, there are distinct characteristics that differentiate them from traditional communication functions/services. For instance, sensing typically involves tracking targets not served by the cellular network. Similarly, AI/ML is generally aimed at optimizing overall system performance rather than serving a specific UE. In such cases, there is no need for AMF to be involved throughout the entire process. Therefore, establishing a direct interface between RAN and the core network for these new services appears to be a more efficient approach.

Proposal 5: For the new services in 6G, it’s feasible and more efficient to support direct connection between 6G RAN and new service function(s) in 6GC.

In 5G, on support of the Positioning, NG-RAN connects to LMF via the AMF indirectly, as shown in the figure below:

Figure 2: UE Positioning Overall Architecture applicable to NG-RAN

Base on this architecture, positioning related procedures are exchanged between NG-RAN and LMF through AMF transparently. For 6G, we can further simplify the Positioning architecture, e.g. support direct connection between 6G RAN and the “LMF” (Positioning function in 6GC), which could make the Positioning related procedures more straightforward and efficient.

Proposal 6: For Positioning service, also feasible and beneficial to use direct connection between 6G RAN and related positioning function in 6GC (e.g. LMF+).

As above, the overall architecture for 6G RAN-CN interface could be illustrated with the figure below:

Figure x. Overall architecture for 6G RAN-CN interface

As shown in the figure above:

- 6G RAN directly connect to AMF+ for legacy control plane functions;

- 6G RAN directly connect to UPF+  for legacy user plane data transmission;

- 6G RAN directly connect to Positioning function in 6GC (maybe evolved LMF, maybe new function);

- 6G RAN which holds AI/ML function, directly connect to AI/ML function(s) in 6GC;

- 6G RAN which holds Sensing function, directly connect to Sensing function(s) in 6GC;

- For the other new services, if any, 6G RAN directly connect to the related new service functions in 6GC.

Note: the direct interface between 6G RAN and new service functions in the 6GC support both signalling and data transmission.

Proposal 7: Discuss and adopt the overall architecture for 6G RAN-CN interface.

3	Conclusion

Base on the discussion above, we provide the following observations and proposal:

Proposal 1: Consider legacy services/functions and new services separately when we define the 6G RAN-CN interface(s).

Proposal 2: Adopt above principles and requirements for 6G RAN-CN interface.

Observation 1: NG interface functions as defined for 5G could be taken as baseline for 6G RAN-CN interface design.

Proposal 3: Take above function split between 6G RAN and 6GC as the start point for the RAN3 study.

Proposal 4: Control Plane and User Plane separation should be adopted to support the legacy functions for 6G RAN-CN interface.

Proposal 5: For the new services in 6G, it’s feasible and more efficient to support direct connection between 6G RAN and new service function(s) in 6GC.

Proposal 6: For Positioning service, also feasible and beneficial to use direct connection between 6G RAN and related positioning function in 6GC (e.g. LMF+).

Proposal 7: Discuss and adopt the overall architecture for 6G RAN-CN interface.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- 3GPP TR38.801 v14.0 Study on new radio access technology: Radio access architecture and interfaces;

- 3GPP TS38.410 v18.6.0 NG general aspects and principles

- 3GPP TS38.413 v18.6.0 NG Application Protocol (NGAP)

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

- R3-256616 RAN Logical Architecture