3GPP TSG-RAN WG3 Meeting #129-bis	R3-256618

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	10.3.2

Source:	CATT

Title:	Considerations on 6G RAN-CN interface options

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the RAN-CN function split, interface, protocol stack and procedures, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze the 6G RAN-CN interface options, considering different requirements from the legacy services and new services. Considering how to make support of legacy NG-like functions and how to make support of the new services e.g. AI, Sensing, over the 6G RAN-CN interface separately.

Base on the discussion, we will provide the observations and proposals accordingly.

2	Discussion

2.1 Support of legacy functions

The legacy functions (NG-like functions) of the 6G RAN-CN interface (called “NG+” for short below), NG-like interface design could be taken as the baseline, e.g. control plane and user plane separation.

Proposal 1: Support Control Plane and User Plane separation for the legacy functions of the 6G RAN-CN interface.

The NG+ control plane interface is defined between the 6G RAN and 6GC entity. It’s assumed the control plane protocol stack of the NG+ interface could base on the NG-C (refer to [5]), the transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG+ Application Protocol.

Figure 1: NG+ Interface Control Plane

The SCTP layer provides the guaranteed delivery of application layer messages.

In the transport IP layer point-to-point transmission is used to deliver the signalling PDUs.

SCTP/IP is the agreed transport protocol for the control plane of NG+.

Proposal 2: It’s proposed to use SCTP/IP as transport protocol for Control plane of 6G RAN and CN interface (NG+). The application layer signalling protocol is referred to as NG+ Application Protocol.

The NG+ user plane interface is defined between the 6G RAN and the UPGW. The user plane interface provides non-guaranteed delivery of user plane PDUs between the 6G RAN and the UPGW.

The XG-U interface shall support per PDU Session tunneling.

NOTE 1:	Support of other type tunneling e.g. per node tunneling may be decided at WI phase, and may pending to the progress of SA2.

The protocol stack for NG+ user plane is shown in Figure below.

Figure 2: NG+ User Plane protocol structure

NOTE 2:	GTP-U is a baseline for NG-U, however, later some enhancements or introduction of a new alternative protocol is still possible if justified.

Proposal 3: It’s proposed to take GTP-U/UDP/IP as transport protocol for the user plane between 6G RAN and 6G CN (similar to NG-U).

2.2 Support of new services

As discussed and proposed in [6], it’s feasible and more efficient to support direct connection between 6G RAN and new service function(s) in 6GC. If a new direct interface is established between the RAN and a new service function in 6GCore Network, it should adhere to a principle similar to the separation of control plane and user plane of NG interface. This is because signalling transmission and data transmission entail fundamentally different requirements in terms of latency, reliability, and data volume. Applying such a separation would facilitate independent scaling of resources, and contribute to more efficient end-to-end service delivery for emerging applications such as AI/ML and ISAC.

Proposal 4: Separate the signalling and data transmission for RAN-CN interface for new services.

2.2.1 Signalling aspects

Unlike legacy functions, for 6G network, many new types of services need to be introduced, e.g., AI, Sensing, etc. To adapt to the new services scenarios, the requirements for new network interfaces and protocols should be elastic scalability, cloud-native, independence, and so on [2]. Considering that the 6GC will continue to adopt the SBI-based network architecture, if the new service types also take the SBI-based interface and protocol into account, it can further realize the unified architecture design of 6G Network in the field of new services.

For the SBI-based interface and protocol on which new types of services can rely, there are the following technical advantages:

- Extensibility of Network Functions

By defining NF Services (SBI APIs), modularization and extensibility can be realized. This is mainly due to service-based architecture principle: adding a new service to an existing Network Function (NF) or introducing a new NF with additional services has minimal impact on other NFs. Network Repository Functions provide registration and discovery services, enabling service providers to announce their new services (or service modifications) in near real-time, while service consumers can receive immediate notifications of such modifications.

- Cloud-Friendly Features

The architecture based on SBI can adapt to and leverage cloud-native technologies, enabling virtualization and containerization of NFs. This in turn brings advantages, e.g., flexible scaling, rapid iteration, improved resource utilization, and enhanced openness. The SBI-based architecture also introduces cloud-friendly protocols, applicable to subscription and notification mechanisms as well as authentication scenarios, e.g., protocols like HTTP/2, HTTP/3, TCP or QUIC.

- Independence of Network Functions

For legacy N2 protocol, all UE-related signalling and non-UE-related signalling are bound to SCTP associations. In contrast, an SBI-based architecture supports resource-level granularity: service operations and created resources can be addressed via URIs. This means message routing can be executed without parsing the message payload, thereby achieving decoupling between service functions and the transport layer. Additionally, the SBI-based architecture enhances flexibility in encryption deployment by providing a method to flexibly control encryption implementation (e.g., HTTP over TLS) on demand.

- Resilience of Network Functions

The service-based architecture natively supports auxiliary services, e.g., load balancing. Message destinations can be identified using the target URI in HTTP headers, and load balancing can be performed without parsing the message payload. This realizes system scalability without relying on specific application protocol. Furthermore, the definition of NF (service) sets provides increased resiliency, e.g., in case of overload or failure of NF instances.

- Efficiency of Transmission

For new types of services, direct communication between different NFs should be permitted. On one hand, this reduces transmission latency. On the other hand, it eliminates redundant binding relationships between different NFs. The SBI-based architecture adopts a bus-based topology design, which ensures that direct communication connections between different NFs can be established on demand. This simplifies the overall network design.

In summary, regarding new types of services, especially in terms of control signalling design, it is recommended to consider the technical advantages of interfaces and protocols based on SBI.

Proposal 5: For new types of services, in terms of control signalling design，RAN3 to consider the technical advantages of interfaces and protocols based on SBI.

2.2.2 Data Transmission

For data transmission for new services, traditional user plane based solution is not suitable with the following reasons:

- Different QoS requirements

For legacy 5G services, DRBs are used to carry user plane data, such as voice, video, and application data, which have diverse QoS profiles tailored to user services. DRBs are more flexible in quantity—multiple DRBs can be established for different services of a single UE, each with unique QoS configurations. For AI/ML data in different scenarios/applications, there may be significant differences in QoS requirements, e.g., training data is not sensitive to latency and allows packet loss; model data does not allow packet loss and has a large data volume, etc.

- Routing and Delay

The UP tunnel can only be established between UE and UPF, the fixed start/terminate node may not fulfil the data transmission requirements of diverse AI/ML and sensing services. For example, in some scenario, it is only necessary to exchange data between RAN and CN, without involvement of UE. We understand it’s beneficial to establish a direct data transmission connection between the RAN and the corresponding Core Network function for new services, rather than relying on the current user plane architecture, which always routes traffic through the UPF.

Proposal 6: Establish a direct data transmission connection between the RAN and the corresponding Core Network function for new services rather than route traffic through UPF.

3	Conclusion

Base on the discussion above, we provide the following observations and proposal:

Proposal 1: Support Control Plane and User Plane separation for the legacy functions of the 6G RAN-CN interface.

Proposal 2: It’s proposed to use SCTP/IP as transport protocol for Control plane of 6G RAN and CN interface (NG+). The application layer signalling protocol is referred to as NG+ Application Protocol.

Proposal 3: It’s proposed to take GTP-U/UDP/IP as transport protocol for the user plane between 6G RAN and 6G CN (similar to NG-U).

Proposal 4: Separate the signalling and data transmission for RAN-CN interface for new services.

Proposal 5: For new types of services, in terms of control signalling design，RAN3 to consider the technical advantages of interfaces and protocols based on SBI.

Proposal 6: Establish a direct data transmission connection between the RAN and the corresponding Core Network function for new services rather than route traffic through UPF.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- 3GPP TR38.801 v14.0 Study on new radio access technology: Radio access architecture and interfaces;

- 3GPP TS38.410 v18.6.0 NG general aspects and principles

- 3GPP TS38.413 v18.6.0 NG Application Protocol (NGAP)

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

- R3-256617 General principles and requirements on RAN-CN interface, CATT