3GPP TSG-RAN3 Meeting #129-bis	R3-256623

Prague, CZ, 13 - 17 October, 2025

Agenda item:	10.2.2

Source:	Xiaomi

Title:	Initial views on RAN logical architecture

Document for:	Discussion

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to progress Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc).

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] Overall RAN architecture aspects [RAN3, RAN2]

In this paper, we provide our initial views about the RAN architecture aspects with the following guidance from RAN3 chair.

Based on standalone architecture to support the agreed existing and new services. Similar scope to TS 38.300 sections 4.1 and 4.2, focusing on RAN functions and high-level logical architecture. Proposals should be motivated by requirements, where appropriate.

2	Discussion

2.1 	Standalone RAN Architecture

An agreed draft TR [2] at RAN plenary #109 meeting has identified the following requirements for architecture and migration.

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Based on the above requirements, a new Radio Access Technology (RAT), referred to as 6GR, will be introduced. While it remains uncertain whether a new 6G core network will be developed, support for new services will necessitate the introduction of new or enhanced core network functions. These aspects are subject to further discussion in the RAN plenary and SA groups. For the RAN3 architecture discussions, given the introduction of a new RAT, it is assumed that new interface(s) between the RAN and core network functions will be required.

Observation 1, For the RAN3 architecture discussions, with the introduction of a new RAT, it is assumed that new interface(s) between the RAN and core network function(s) will be required—regardless of whether a new core network or an enhanced existing core is adopted in SA2.

Lessons learnt from 5G

The following is the overall architecture description for 5G as defined in TS 38.300 [3]

An NG-RAN node is either: - a gNB, providing NR user plane and control plane protocol terminations towards the UE; or - an ng-eNB, providing E-UTRA user plane and control plane protocol terminations towards the UE. The gNBs and ng-eNBs are interconnected with each other by means of the Xn interface. The gNBs and ng-eNBs are also connected by means of the NG interfaces to the 5GC, more specifically to the AMF (Access and Mobility Management Function) by means of the NG-C interface and to the UPF (User Plane Function) by means of the NG-U interface (see TS 23.501 [3]).

- Experience from 5G shows that integrating both gNB and ng-eNB within NG-RAN significantly increased architectural complexity, operational costs, and slowed the adoption of NR-native capabilities. Maintaining dual protocol stacks and interworking mechanisms introduced signaling overhead and constrained innovation. For 6G, a clean-slate approach is essential: the 6G RAN should exclusively support the new 6G RAT only, while legacy inter-working should be handled at the core network level. This separation minimizes complexity, reduces cost, and accelerates the deployment of advanced 6G features such as AI-native optimization and integrated sensing, ensuring that backward compatibility does not hinder forward progress.

| - 5G | - 6G |
| --- | --- |

- Lesson learnt from 5G (1), Integrating gNB and ng-eNB in 5G NG-RAN led to increased complexity, higher costs, and hindered NR-native innovation due to dual protocol stacks and signaling overhead.

- Proposal 1, RAN3 starts standalone RAN logical architecture study as baseline.

2.2 	6G RAN functions

According to TS 38.300 [3], the RAN-CN function split is summarized on the figure below where yellow boxes depict the logical nodes and white boxes depict the main functions.

Figure 4.2-1: Functional Split between NG-RAN and 5GC

- While 5G primarily focused on communication aspects such as data, control, and mobility—with AI/ML and sensing treated as add-ons, 6G RAN–CN function split should build on 5G principles but also natively support for emerging functionalities like AI/ML and sensing.

- In general, the functionalities for AI and sensing may be potentially distributed between RAN and CN, so the following figure shows the main function split between 6G RAN and 6G CN.

- RAN3 can first focus on the RAN side functionalities and discuss the CN functions according to SA2 progress. The following table shows our views on 6G RAN potential functionalities comparing to 5G.

- Legacy RAN functions

- According to 5G RAN functionalities, e.g., function for Radio resource management, L2 functionalities (e.g., IP header compression), session management, etc, 6G can follow similar functionalities in 6G, but the details are pending to other WGs’ discussion. From RAN3 perspective, we can focus on the aspects related RAN3 and architecture design, e.g., selection of core network functions, routing the signalling/data to the core network functions.

- New functions for new services

- To enable intelligent and adaptive network operations in 6G, the introduction of a dedicated AI Control Function within the RAN is essential. This entity would manage the full LCM of AI/ML models at the air interface, including training, deployment, and updates, while also handling data collection configuration and reporting to ensure high-quality datasets. Furthermore, it would support efficient processing and transfer of model parameters and datasets, which are critical for real-time AI-driven decision-making.

- To support ISAC, the sensing related RAN functionalities should also be considered. It was agreed in RANP#109 meeting, sensing use cases are agreed in [2] on Requirements of new and existing services for 6G, and the sensing mode are agreed during the online discussion.

### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

- The sensing related functionalities at least includes sensing resource allocation, measurement configuration, sensing processing and report can be further considered. From RAN3 perspective, a general new function to support sensing should be considered, and the details sensing related functions split study should align and coordinate with other working groups (e.g., RAN2 and SA2).

- Observation 2, legacy RAN functionalities related to other working group need coordination with WG’s discussion.

- Observation 3, RAN3 related RAN functionalities e.g., NF selection and routing depends on the RAN architecture design.

- Proposal 2, RAN3 starts the RAN functionalities related to the selection of NF and routing of signalling/data to the core network functions, based on the architecture options.

- Proposal 3, RAN3 study the new RAN functionalities of AI and sensing related RAN function in coordination with RAN1 and RAN2.

2.3	 New RAN logical entity for AI

- As we discussed in section 2.2, a new AI control function may be needed to enable intelligent and adaptive network operations in 6G. Given the complexity and diversity of AI-native services, this function could be deployed either centrally or in a distributed manner, offering flexibility to meet varying performance and scalability requirements across different network scenarios. When AI function is outside of a RAN node, a new AI control entity can be considered, on the other hand, this AI control entity can also be realized by implementation.

| - AI Control Function is inside RAN node | - AI Control Function is outside of RAN node |
| --- | --- |
| UE RAN Node 6GC Lower layer AI Control function | UE AI control FUNCTION 6GC RAN Node RAN Node |

- Observation 4, 6G RAN architecture support an AI Control Function, deployed in either centralized or distributed manner, to enable intelligent air interface management and efficient handling of AI-related data.

- Observation 5, when deployed outside of a RAN node, the AI control entity can be a new logical node or realized by implementation.

- Proposal 4, RAN3 study the need to introduce a new logical node for AI control function.

- In the Annex, we provide TP to 6G TR to capture the general 6G standalone architecture.

- Proposal 5, RAN3 agrees the TP in the Annex.

3	Conclusion

In this contribution, we discussed the 6G general RAN architecture, 6G RAN functions, and whether to have a logical new RAN entity for AI and have the following observations and proposals.

Standalone RAN Architecture

Observation 1, For the RAN3 architecture discussions, with the introduction of a new RAT, it is assumed that new interface(s) between the RAN and core network function(s) will be required—regardless of whether a new core network or an enhanced existing core is adopted in SA2.

- Lesson learnt from 5G (1), Integrating gNB and ng-eNB in 5G NG-RAN led to increased complexity, higher costs, and hindered NR-native innovation due to dual protocol stacks and signaling overhead.

- Proposal 1, RAN3 starts standalone RAN logical architecture study as baseline.

6G RAN functions

- Observation 2, legacy RAN functionalities related to other working group need coordination with WG’s discussion.

- Observation 3, RAN3 related RAN functionalities e.g., NF selection and routing depends on the RAN architecture design.

- Proposal 2, RAN3 starts the RAN functionalities related to the selection of NF and routing of signalling/data to the core network functions, based on the architecture options.

- Proposal 3, RAN3 study the new RAN functionalities of AI and sensing related RAN function in coordination with RAN1 and RAN2.

New RAN logical entity for AI

- Observation 4, 6G RAN architecture support an AI Control Function, deployed in either centralized or distributed manner, to enable intelligent air interface management and efficient handling of AI-related data.

- Observation 5, when deployed outside of a RAN node, the AI control entity can be a new logical node or realized by implementation.

- Proposal 4, RAN3 study the need to introduce a new logical node for AI control function.

- Proposal 5, RAN3 agrees the TP in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC

[3] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description, stage2.

5	Annex (TP to 6G TR on RAN logical architecture)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

5.3.x	Standalone RAN Architecture

A 6G RAN node is providing 6GR user plane and control plane protocol terminations towards the UE.

The 6G RAN node is connected by means of the Nx interfaces to the NF(s) in 6GC.

Editor’s Note, the NF(s) could be 6G AMF, 6G UPF, other NFs, that are pending to further discussion.

The 6G RAN architecture is illustrated in Figure 5.3.x-1 below.

Figure 5.3.x-1: Overall Architecture

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>