3GPP TSG-RAN3 Meeting #129-bis	R3-256624

Prague, CZ, 13 - 17 October, 2025

Agenda item:	10.3.1

Source:	Xiaomi

Title:	General principles and requirements on RAN-CN interface

Document for:	Discussion

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to study the RAN-CN interface and protocol stack.

(6)Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a)Overall RAN architecture aspects [RAN3, RAN2] b)RAN-CN functional split, interface, protocol stack and procedures [RAN3] c)RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discussed the general requirements on RAN-CN interface based on the lessons learnt from 5G and new requirements for sensing and AI.

2	Discussion

2.1	Transmission efficiency and complexity requirements for new services

- Lesson learnt from 5G

- Taking location service as an example, the positioning signaling between the RAN node/UE and the LMF is relayed via either the AMF or UPF. This indirect routing increases transmission latency and adds complexity to both the AMF and UPF, as shown in Figure 1.

- Figure 1 Logical Architecture for Positioning in 5G

- From the RAN3 perspective on RAN-CN interface design, the NRPPa protocol is relayed by the AMF, while LPP messages are routed through both the NG-RAN and the AMF. The current indirect protocol stacks used for location services requires additional routing functionalities within the AMF. These routing requirements could be minimized in the 6G RAN-CN interface design to improve efficiency and reduce complexity.

| - LPP protocol stack | - NRPPa protocol stack |
| --- | --- |

Lesson learnt from 5G (1), the 5G indirect interface and protocol stacks are relayed by AMF, which increases transmission latency and adds complexity.

On the other hand, in the context of AI/ML, both RAN3 and SA2 have studied and specified data collection for AI/ML use cases in 5G. While some data and features could be mutually beneficial, there is currently no efficient interworking between RAN and NWDAF, primarily due to complexity, as AMF involvement remains unavoidable, as shown in Figure 2.

Figure 2 No-interworking between RAN and CN in 5G for AI

- Lesson learnt from 5G (2), there’s no efficient inter-working between RAN node and NWDAF for AI related use cases due to complexity of the 5G RAN-CN interface design, as AMF involvement remains unavoidable.

- Considering the data amount of sensing and AI services, if based on the legacy architecture, there may be signalling storms for AMF to handle and route the signalling/data between RAN and other NFs, which are not relevant to UE context and mobility.

- Observation 1, The AMF is designed to manage UE context and mobility. However, signaling storms may occur if it processes and routes signaling and data traffic for sensing and AI-driven services.

- In 6G RAN-CN interface design, for the new services (e.g., Sensing, AI), we prefer direct interface between 6G RAN node and network functions (e.g., SF, 6G AIF) can be supported. In our views, it could be realized either by SBI or P2P (the transport could be STCP/QUIC/GTP-U based).

- For 5G Ambient IoT, an direct interface between gNB and AIOTF has been introduced for topology 1, which is one of the example to realize direct interface via P2P interface (e.g., SCTP-based protocols).

- Figure 3 Direct connectivity between 6G RAN and NFs for new services

- Proposal 1, 6G RAN-CN interface between RAN node and network functions for new services (e.g., Sensing, AI) shall support low latency and less complexity transmission and avoid singalling storms for irrelevant NF (e.g., 6G AMF).

2.2	Data transfer requirements for new services

In 5G, RAN architecture supports user plane and control plane separation as they experience different load and have different characteristics, as shown in Figure 4.

Figure 4 5G system architecture and interfaces

To support emerging 6G services such as AI and sensing, RAN-CN interface should consider the data transmission requirements. We summarized the following data transmission requirements especially for RAN3 to consider, for the data transmission from UE to network, we can wait for RAN2 progress.

| Use case | Data path | Data volumes and delay requirements | Data types (UE associated or not) | Fit for existing CP or UP |
| --- | --- | --- | --- | --- |
| Training data/inference data for RAN-side/CN-side model | RAN<->CN | Similar to existing signalling but with large amount of data Low or high latency requirements depending on the purpose | UE associated or non UE associated (depending on use cases) | No CP cannot afford large volume of data UP is not designed for non-UE associated data |
| Model parameters/data set for two-sided model | RAN->CN->AF (OTT server) | ~ tens of KBs to hundreds of MBs Low latency requirements | Non UE associated | No Same as above |
| Sensing data (e.g., row data, spectrum measurements) from TRP | RAN->CN | ~ tens of KBs to hundreds of MBs, depends on the data type High latency requirements | Non UE associated | Yes or No depends on the data type Same as above |

Unlike traditional user data, AI-related traffic—such as model parameters, training datasets, requires high-throughput, relax-latency transmission but does not fit within the scope of existing user or control plane functions. The 5G control plane is not designed for large data transport, and non-UE-associated data is not well-suited for transmission via the current user plane mechanisms.

Observation 2, existing control plane and user plane between RAN and CN does not fit for non-UE associated large-volume data transmission for AI and sensing (e.g., AI model parameters/dataset, some types of the sensing data).

Currently, SA2 is discussing whether to introduce a new data plane or enhancing CP/UP to support the data framework in 6G, but there’s no progress in the last SA2 meeting, the following is the draft scope for data framework in SA2 in [2].

- Design end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.: - data collection - data transfer and distribution - data processing e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs - data storage within 6G system - data access - data exposure within 6G system and to outside of 6G system NOTE 1: Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure. NOTE 2: Coordination with RAN WG is required for data collection from or via RAN. NOTE 3: Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

Observation 2, SA2 is discussing the data framework in 6G system architecture.

Proposal 2, 6G RAN-CN interface shall support efficient transmission of large-volume data for both UE-associated and non-UE-associated cases, with the ability to handle different latency requirements relevant to AI/ML and sensing, to meet such requirements, coordinated with SA2 is needed.

According to above discussion, we provide a TP to pCR to include the principle and requirements for 6G RAN-CN interface.

Proposal 3, RAN3 agrees the TP in the Annex.

3	Conclusion

In this contribution, we discussed the general requirements on RAN-CN interfac and have the following observations and proposals.

Transmission efficiency and complexity requirements for new services

Lesson learnt from 5G (1), the 5G indirect interface and protocol stacks are relayed by AMF, which increases transmission latency and adds complexity.

- Lesson learnt from 5G (2), there’s no efficient inter-working between RAN node and NWDAF for AI related use cases due to complexity of the 5G RAN-CN interface design, as AMF involvement remains unavoidable.

- Observation 1, The AMF is designed to manage UE context and mobility. However, signaling storms may occur if it processes and routes signaling and data traffic for sensing and AI-driven services.

- Proposal 1, 6G RAN-CN interface between RAN node and network functions for new services (e.g., Sensing, AI) shall support low latency and less complexity transmission and avoid singalling storms for irrelevant NF (e.g., 6G AMF).

Data transfer requirements for new services

Observation 2, existing control plane and user plane between RAN and CN does not fit for non-UE associated large-volume data transmission for AI and sensing (e.g., AI model parameters/dataset, some types of the sensing data).

Observation 2, SA2 is discussing the data framework in 6G system architecture.

Proposal 2, 6G RAN-CN interface shall support efficient transmission of large-volume data for both UE-associated and non-UE-associated cases, with the ability to handle different latency requirements relevant to AI/ML and sensing, to meet such requirements, coordinated with SA2 is needed.

Proposal 3, RAN3 agrees the TP in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] S2-2508132, [WT#5 Data framework] Summary of WT scope on 6G Data framework

5	Annex (TP to pCR)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

Proposal 1, 6G RAN-CN interface between RAN node and network functions for new services (e.g., Sensing, AI) shall support low latency and less complexity transmission and avoid singalling storms for irrelevant NF (e.g., 6G AMF).

6G RAN-CN interface shall support efficient transmission of large-volume data for both UE-associated and non-UE-associated cases, with the ability to handle different latency requirements relevant to AI/ML and sensing.

Editor’s note: to meet above requirements, coordinated with SA2 is needed.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>