3GPP TSG-RAN3 Meeting #129-bis	R3-256625

Prague, CZ, 13 - 17 October, 2025

Agenda item:	10.3.2

Source:	Xiaomi

Title:	RAN-CN interface and protocol options

Document for:	Discussion

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to study the RAN-CN interface and protocol stack.

(6)Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a)Overall RAN architecture aspects [RAN3, RAN2] b)RAN-CN functional split, interface, protocol stack and procedures [RAN3] c)RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discussed the RAN-CN interface and protocol options based on the requirements discussed in our paper in [2], and provide the evaluation considerations between SBI and P2P.

- Discussion

The 6G RAN architecture must support control and user plane separation. For legacy services, the RAN-CN control and user planes could be the same as 5G. However, for new services (e.g., sensing, AI), given their different transmission requirements as we discussed in [2], the need for an additional plane (e.g., a "data plane") may need to be considered in coordination with SA2. This paper assumes the signaling/data for new services will utilize an enhanced control plane which can be regarded as “data plane” in the future if agreed by SA2.

Figure 1 Transmission planes for signalling/data between 6G RAN and 6G Core

Observation 1, the need for an new plane (e.g., a "data plane") for the new services requires further discussion and coordination with SA2.

Proposal 1, RAN3 assumes the signalling/data for new services (e.g., AI, Sensing) can be transferred via a “enhanced” control plane, and align with SA2 when they have clear progress.

2.1	User Plane Interface and protocol

Same as specified in 5G NG-U, a Nx user plane interface (Nx-U) can be defined between the 6G RAN node and the 6G UPF as shown on Figure below. The transport network layer is built on IP transport and GTP-U is re-used on top of UDP/IP to carry the user plane PDUs between the 6G RAN node and the 6G UPF.

Proposal 2, A Nx-U interface between the 6G RAN and the 6G UPF, NG-U provides the delivery of user plane PDUs between the 6G RAN node and the 6G UPF.

2.2	(enhanced) Control Plane Interface and protocol

- In 5G, the core network introduced the Service-Based Architecture (SBA) with Service-Based Interfaces (SBIs) to enable flexible and modular communication between network functions like AMF and SMF. However, the interface between the RAN and Core Network—the NG interface—retained a traditional point-to-point protocol (NGAP) to prioritize efficiency, reliability, and backward compatibility. While SBA brought service agility to the core, NGAP remained in place to ensure stable and performant RAN–CN interaction.

- In this section, the P2P and SBI interface and protocols for (enhanced) control plane are discussed separately.

2.2.1	P2P Architecture and protocol stacks

- As we discussed in [2], two potential P2P architecture options exist:

- Option 1 (Direct Connectivity): Establishes direct links between the RAN node and relevant NFs requiring coordination. Its protocol resembles NG-C.

- Option 2 (Indirect Connectivity): Relies on 5G's indirect model via AMF/UPF for all RAN-NF coordination. As noted in [2], this is less preferred but remains open for study. Protocol design offers two sub-options for sensing/AI coordination:

- - Option 2-1: Uses a new protocol over NxAP, similar to the NRPPa model for positioning.

- - Option 2-2: Embed service-related information within NxAP messages, similar to SM info transport in NGAP.

- For NFs in core network shown in the table below, the final definitions depend on SA2 outcomes. However, it is foreseen that some NFs (e.g., NF1, NF2) need tight RAN coordination, particularly for sensing/AI use cases, whereas NF3 and NF4 will operate without RAN coordination.

| - Option 1 direct connectivity with new NFs (sensing, AI) | - Option 2 indirect connectivity with new NFs (sensing, AI) |
| --- | --- |
| - Protocol stacks between 6G RAN node and AMF/SF/AIF |  |
| Option 1 6G Nx control plane interface (Nx-C) | - Option 2-1, new protocol layer above NxAP. - Option 2-2, service container included in NxAP. |
| - For both options, the transport layer protocol could be SCTP, QUIC or GTP-U, which requires more study based on operator deployment and service requirements. |  |

- Proposal 3: RAN3 is to study two P2P architectural options for 6G:

- - Direct connectivity

- - Indirect connectivity (via AMF/UPF)

- Proposal 4: For indirect architecture, RAN3 study the signaling/information transfer protocols between the 6G RAN nodes and the new network functions (e.g., AI, sensing):

- - Option 2-1: Implementation of a new protocol layer above NxAP

- - Option 2-2: Encapsulation of service-container information within NxAP

- Proposal 5: For both direct and indirect P2P architecture, RAN3 shall analyze the following transport protocols considering operational deployment and service-specific demands:

- - SCTP

- - QUIC

- - GTP-U

2.2.2	Service-based Architecture and protocol stacks

- Enable RAN to function(s) as both a service producer (exposing RAN service to CN) and consumer (utilizing CN services),there may be two implementation options:

- Option 1 (Full SBA): Implement service-based interfaces for all RAN functions.

- Option 2 (Partial SBA): Apply service-based interfaces only to new services (e.g., AI/sensing), while maintaining traditional point-to-point interfaces for legacy access functions per 5G principles.

| Option1, full SBA | Option 2, partial SBA |
| --- | --- |
| The SBI protocol can refer to the protocol stack defined in TS 29.500, if new protocol stack for 6G is defined in SA2, SBI for RAN-CN need to align with SA2. |  |

Proposal 6, RAN3 study the two options for service-based RAN architecture.

- full RAN service-based architecture

- partial RAN service-based architecture

Proposal 7, the protocol stack for SBI should be aligned with SA2.

2.2.3	Evaluation consideration

We think the following factors must be considered when evaluating Service-Based Interfaces (SBI) versus Point-to-Point (P2P) architectures with the reasons.

| Key factors | Why |
| --- | --- |
| Performance Efficiency | RAN-CN interfaces handle critical signaling (e.g., mobility management, session setup, sensing, AI). Latency or throughput degradation directly impacts user experience and network efficiency, and operational costs. |
| Deployment Flexibility & Cloud-Native Alignment | Operators seek scalable, software-defined deployments |
| Support for Emerging Services (e.g., AI, Sensing) | 6G must natively support AI-driven services (e.g., predictive mobility) and sensing (e.g., environmental perception), which introduce new data/signalling transmission requirement. |
| Standardization Effort | SBI is new for RAN-CN interface, how to specify it and how to split the work between RAN and SA are not clear, on the other hand, P2P focuses on protocol-level specs but needs extensions for service awareness. |

- Proposal 8, RAN3 considers the following evaluation factors to compare SBI with P2P

- - Performance Efficiency

- - Deployment Flexibility & Cloud-Native Alignment

- - Support for Emerging Services (e.g., AI, Sensing)

- - Standardization Effort

- In the Annex, we provide a TP to 6G TR on the RAN-CN interface and protocol options.

- Proposal 9, RAN3 agrees the TP in the Annex

3	Conclusion

In this contribution, we discussed RAN-CN interface and protocol options, and have the following observations and proposals.

Observation 1, the need for an new plane (e.g., a "data plane") for the new services requires further discussion and coordination with SA2.

Proposal 1, RAN3 assumes the signalling/data for new services (e.g., AI, Sensing) can be transferred via a “enhanced” control plane, and align with SA2 when they have clear progress.

User Plane Interface and protocol

Proposal 2, A Nx-U interface between the 6G RAN and the 6G UPF, NG-U provides the delivery of user plane PDUs between the 6G RAN node and the 6G UPF.

(enhanced) Control Plane Interface and protocol

- Proposal 3: RAN3 is to study two P2P architectural options for 6G:

- - Direct connectivity

- - Indirect connectivity (via AMF/UPF)

- Proposal 4: For indirect architecture, RAN3 study the signaling/information transfer protocols between the 6G RAN nodes and the new network functions (e.g., AI, sensing):

- - Option 2-1: Implementation of a new protocol layer above NxAP

- - Option 2-2: Encapsulation of service-container information within NxAP

- Proposal 5: For both direct and indirect P2P architecture, RAN3 shall analyze the following transport protocols considering operational deployment and service-specific demands:

- - SCTP

- - QUIC

- - GTP-U

Service-based Architecture and protocol stacks

Proposal 6, RAN3 study the two options for service-based RAN architecture.

- full RAN service-based architecture

- partial RAN service-based architecture

Proposal 7, the protocol stack for SBI should be aligned with SA2.

Evaluation consideration

- Proposal 8, RAN3 considers the following evaluation factors to compare SBI with P2P

- - Performance Efficiency

- - Deployment Flexibility & Cloud-Native Alignment

- - Support for Emerging Services (e.g., AI, Sensing)

- - Standardization Effort

- Proposal 9, RAN3 agrees the TP in the Annex

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2]R3-256624, General principles and requirements on RAN-CN interface, Xiaomi

5	Annex (TP to 6G TR on RAN-CN interface options)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

6.1.3.x1	User Plane Interface and protocol

The Nx user plane interface (Nx-U) is defined between the 6G RAN node and the 6G UPF. The user plane protocol stack of the Nx interface is shown on Figure 6.1.3.x1-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs between the 6G RAN node and the 6G UPF.

Figure 6.1.3.x1-1: Nx-U Protocol Stack

6.1.3.x2	Control Plane Interface and protocol

In case of P2P architecture, The Nx control plane interface (Nx-C) is defined between the 6G RAN node and the NF(s). The control plane protocol stack of the Nx interface is shown on Figure 6.1.3.x2-1. The transport network layer is built on IP transport. In the transport, IP layer point-to-point transmission is used to deliver the signalling PDUs.

Editor’s Note x-1, the transport layer could be SCTP, QUIC, GTP-U, which need further study.

Editor’s Note x-2, FFS on whether the NF is only 6G AMF, or multiple NFs.

Editor’s Note x-3, FFS on whether there is a data plane for new services (e.g., sensing, AI)

Figure 6.1.3.x2-1: Nx-C P2P Protocol Stack

In case of service-based architecture, service-based interface is defined between 6G RAN node and Core network NFs, as shown on Figure 6.1.3.x2-2.

Editor’s Note x-4, the transport layer of SBI should be aligned with SA2.

Figure 6.1.3.x2-2: Service-based Protocol Stack

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>