3GPP TSG-RAN3 Meeting #129-bis	R3-256626

Prague, CZ, 13 - 17 October, 2025

Agenda item:	10.4

Source:	Xiaomi

Title:	RAN internal functional split and interfaces

Document for:	Discussion

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to study the RAN internal functional split, interfaces.

(6)Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a)Overall RAN architecture aspects [RAN3, RAN2] b)RAN-CN functional split, interface, protocol stack and procedures [RAN3] c)RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we discussed the RAN-RAN interface and RAN internal function split.

2	Discussion

2.1 	RAN-RAN interface

- Per TS 38.300 [2], the 5G Xn interface comprises Xn-C (control plane) and Xn-U (user plane). For 6G, the user plane protocol should follow 5G's Xn-U. The control plane protocol and underlying transport, however, should align with those used for the 6G RAN-CN interface (e.g., P2P or SBI)

- Proposal 1, for 6G Xn interface, RAN3 agrees the following:

- - User Plane : same as 5G Xn-U

- - Control Plane : align with 6G RAN-CN interface (P2P or SBI)

- Legacy Xn functions

- For 6G Xn functions, the existing 5G Xn functionalities serve as the baseline, including:

-  Data forwarding (Xn-U);

-	Flow control (Xn-U);

-	Xn interface management (Xn-C);

-	UE mobility management, including context transfer and RAN paging(Xn-C);

-	Dual connectivity ((Xn-C)).

The table below analyzes whether the 6G RAN-RAN interface requires functions similar to 5G Xn.

| 5G Xn function | Analysis | To be considered as 6G Xn function |
| --- | --- | --- |
| Data forwarding; Flow control. | Data forwarding and flow control are still required in 6G RAN-RAN interface | Yes |
| Xn interface management; | Should align with RAN-CN interface (P2P/SBI); SBI adoption may necessitate new/common interface management | Pending to P2P/SBI discussion |
| UE mobility management, including context transfer and RAN paging; | UE mobility management is needed, but context transfer/RAN paging depend on UE-state discussions | Yes, details are pending to UE state/paging discussion |
| Dual connectivity | To be addressed at RAN Plenary level, and in our view, Dual Connectivity is recommended for exclusion due to its complexity | No |

Observation 1, User-plane data forwarding and flow control remain essential for the 6G RAN-RAN interface.

Observation 2, The requirement for Xn interface management functionality depends on P2P/SBI protocol decisions.

Observation 3, UE mobility specifics (e.g., context transfer and paging) require are pending to UE state and paging mechanism discussions.

Observation 4, Dual connectivity complexity warrants further evaluation at the RAN Plenary level.

Proposal 1, RAN3 should investigate RAN-to-RAN interface functional aspects, using these legacy capabilities as starting point:

- Data forwarding

- Flow control

- UE mobility management

- AI related functions

As with legacy systems, supporting AI in the network requires data collection and forwarding between RAN nodes to enable AI lifecycle management (LCM). Furthermore, to realize native-AI operations, AI/ML for air interface (e.g., AI-based CSI, Beam management,...) and 6G RAN (e.g., AI-based load balancing, mobility, CCO,...) are expected to be extensively deployed on the network side. Given this trend, energy efficiency for AI-related operations within the network side must also be addressed, for the following reasons:

-High Computational Demand: AI/ML processing tasks performed by base stations (e.g., for channel state prediction, beamforming optimization, traffic forecasting, network slicing management, anomaly detection) are computationally intensive and consume significant power.

-Growing Contribution to Network Energy: As AI/ML becomes integral to optimizing RAN performance , its power consumption will constitute an increasingly large portion of the total energy usage of cellular networks.

- Operational Cost and Sustainability: Energy efficiency is paramount for network operators to reduce operational expenditure (OPEX) and meet environmental sustainability goals. Optimizing AI/ML processing power directly impacts the network's overall energy efficiency.

Energy-efficient AI operations in the network may involve strategies such as computational resource sharing and load balancing across gNBs to reduce power consumption during AI-driven tasks. However, the trade-off between energy savings and overall system performance must be carefully evaluated. To support these optimizations, the 6G Xn interface should accommodate AI-related data exchange—potentially including non-UE-associated information—between 6G RAN nodes, specifically for energy-saving purposes.

- Observation 5: Inter-node data collection remains essential for 6G Xn, consistent with legacy system requirements.

- Observation 6: AI/ML deployment in 6G RAN/air interfaces faces critical energy challenges due to rising computational demands.

- Proposal 2: RAN3 should investigate these Xn functionalities for Native AI:

- - Data collection and forwarding

- - AI resource sharing for network side model

- Sensing related functions

- As agreed in RAN#109 meeting in [3], all the sensing modes are agreed to be studied in 6G.

- Moderator proposal 4: - -6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - -6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs.

- When TRPs reside on separate RAN nodes, sensing coordination via the 6G Xn interface may be required for efficient resource configuration and updates. Direct inter-base station coordination is preferred over core-network-mediated approaches due to the time-sensitive nature of sensing services. Consequently, introducing an Xn-based sensing coordination function should be considered, the details should be based on the progress in RAN2/SA2.

- Observation 7: Direct inter-base station coordination is preferable to core-network-mediated approaches for sensing services due to time-sensitivity requirements.

- Proposal 3: RAN3 should investigate potential sensing coordination functions for 6G Xn, based on the RAN2/SA2 progresses on sensing.

2.2 	RAN internal function split

5G NR supports CU-DU split as well as CP-UP split (i.e. CU-CP/CU-UP split). For CP-UP split, considering its complexity and no commercial deployment, it is proposed to not consider CP-UP split in 6G. A simplified 6G function split at RAN is shown in below.

| 5G RAN split | Proposed 6G RAN split |
| --- | --- |

Proposal 4: The 6G RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

Lessons learnt from 5G

The following drawbacks are identified for 5G RAN CU-DU split

- Control Plane Latency and Overhead, DU-related L1/L2 configurations require CU-RRC involvement, adding latency and signaling load unnecessarily. (e.g., the signalling has to go through the route of (CU request)->DU->CU->DU->UE)

- MAC Layer Inefficiency: Slow RRC procedures force unnecessary control information to be sent over the MAC layer as a workaround.

Lesson learnt from 5G (1), The 5G RAN CU-DU split introduces control plane latency and MAC layer inefficiencies due to centralized RRC handling, resulting in excessive signaling and suboptimal resource management.

Proposal 5: The RAN internal architecture shall support low control signalling latency and efficient transmission.

In the Annex, we provide a TP to 6G TR on general principles, RAN-RAN interface and RAN internal function split.

Proposal 6, RAN3 agrees the TP in the Annex.

3	Conclusion

In this contribution, we discussed the RAN-RAN interface and RAN internal function split, and have the following observations and proposals.

RAN-RAN interface

- Proposal 1, for 6G Xn interface, RAN3 agrees the following:

- - User Plane : same as 5G Xn-U

- - Control Plane : align with 6G RAN-CN interface (P2P or SBI)

- Legacy 5G functions

Observation 2, The requirement for Xn interface management functionality depends on P2P/SBI protocol decisions.

Observation 3, UE mobility specifics (e.g., context transfer and paging) require are pending to UE state and paging mechanism discussions.

Observation 4, Dual connectivity complexity warrants further evaluation at the RAN Plenary level.

Proposal 1, RAN3 should investigate RAN-to-RAN interface functional aspects, using these legacy capabilities as starting point:

- Data forwarding

- Flow control

- UE mobility management

- AI related Functions

- Observation 5: Inter-node data collection remains essential for 6G Xn, consistent with legacy system requirements.

- Observation 6: AI/ML deployment in 6G RAN/air interfaces faces critical energy challenges due to rising computational demands.

- Proposal 2: RAN3 should investigate these Xn functionalities for Native AI:

- - Data collection and forwarding

- - AI resource sharing for network side model

- Sensing related Functions

- Observation 7: Direct inter-base station coordination is preferable to core-network-mediated approaches for sensing services due to time-sensitivity requirements.

- Proposal 3: RAN3 should investigate potential sensing coordination functions for 6G Xn, based on the RAN2/SA2 progresses on sensing.

RAN internal function split

Proposal 4: The 6G RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

Lesson learnt from 5G (1), The 5G RAN CU-DU split introduces control plane latency and MAC layer inefficiencies due to centralized RRC handling, resulting in excessive signaling and suboptimal resource management.

Proposal 5: The RAN internal architecture shall support low control signalling latency and efficient transmission.

Proposal 6, RAN3 agrees the TP in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description, stage2.

[3]RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC (Moderator)

5	Annex (TP to pCR)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

6.2.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

The 6G RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

The 6G RAN architecture shall support internal function split with low control signalling latency and efficient transmission.

6.2.x1	RAN-RAN interface

6.2.x1.1	6G Xn User Plane

The 6G Xn User plane (6G Xn-U) interface is defined between two 6G RAN nodes. The user plane protocol stack on the 6G Xn interface is shown in Figure 6.2.x1.1-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs.

Figure 6.2.x1.1-1: 6G Xn-U Protocol Stack

Xn-U provides non-guaranteed delivery of user plane PDUs and supports the following functions:

-	Data forwarding;

-	Flow control.

6.2.x1.2	6G Xn Control Plane

The 6G Xn control plane interface (6G Xn-C) is defined between two 6G RAN nodes.

Editor’s note 1: the protocol stacks for 6G Xn control plane should be aligned with 6G RAN-CN control plane interface, which need further discussion.

The 6G Xn-C interface supports the following functions:

-	Xn interface management;

-	UE mobility management;

-	Data collection and forwarding

-  AI resource sharing

-  	Sensing measurement coordination

Editor’s note 2: the details of sensing measurement coordination is pending to RAN2/SA2 discussion.

6.2.x2	RAN Internal Function Split

A 6G RAN node may consist of a 6GNB-CU and one or more 6GNB-DU(s). A 6GNB-CU and a 6GNB-DU is connected via Fx interface, as shown in Figure 6.2.x2-1.

One 6GNB-DU is connected to only one 6GNB-CU.

Figure 6.2.x2-1: 6G RAN Internal Function Split

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>