3GPP TSG-RAN WG3 Meeting #129bis	R3-256639

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Consideration on A-IoT dedicated Cause values

Agenda Item:	9.2.7

Source:	Huawei, CMCC, Futurewei, Lenovo, China Unicom, China Telecom

Document for:	Discussion

# Introduction

In R19 Ambient IoT discussion, there are many failure cases in which the gNB or the A-IoT CN node needs to send cause to inform the reason of the failure to the peer node.

Currently, NGAP is able to provide many cause valuess but they are not able to cover some A-IoT dedicated failure cases, in this contribution, we propose to add several A-IoT dedicated Cause values.

# Discussion

Cause IE has to be included in the following A-IoT related Messages and IEs:

- INVENTORY FAILURE message

- COMMAND FAILURE message

- Inventory Failure Transfer IE

- Command Failure Transfer IE

- AIOT Session Release Command Transfer IE

- AIOT Session Release Request Transfer IE

If we check exisiting cause values supported by the Cause IE, the following may be reused in case of A-IOT:

| >>Radio Network Layer Cause | Unspecified, Release due to NG-RAN generated reason, Release due to 5GC generated reason, Radio resources not available, Failure in the radio interface procedure, Interaction with other procedure, |
| --- | --- |
| >>Transport Layer Cause | Transport resource unavailable, Unspecified, |
| >>NAS Cause | Normal release, Unspecified, |
| >>Protocol Cause | Transfer syntax error, Abstract syntax error (reject), Message not compatible with receiver state, Semantic error, Abstract syntax error (falsely constructed message), Unspecified, |
| >>Miscellaneous Cause | Control processing overload, Hardware failure, O&M intervention, Unknown PLMN or SNPN, Unspecified |

But the above cause values are not sufficient, for example:

- Case 1: upon receiving a Command Request from the A-IoT CN node, the included RAN A-IoT Device NGAP ID is unknown within this A-IoT session, the gNB shall fail the Command Request procedure.

- Case 2: upon receiving the Inventory Request with either the A-IoT Area ID list and or the Reader List, the gNB found that neither of them are served by itself, the gNB shall fail the Inventory Request procedure.

- Case 3: upon receiving the Inventory Response/report, the combination of the AIOTF Identifier and the A-IoT Correlation Identifier is not known by the A-IoT CN node, the A-IoT CN node shall trigger Error Indication to inform this.

- Case 4: after inventory report, upon receiving the Command Request towards a device, the gNB found that the device is not reachable, the gNB shall fail the Command Request procedure.

- Case 5: upon receiving an Inventory Request, the gNB finds that the session (identified by the combination of the AIOTF Identifier and the A-IoT Correlation Identifier) has already been established, the gNB shall fail the incoming proceduce.

- Case 6: after sending A-IoT NAS PDU received from Command Request to the device, the device did not provide the A-IoT NAS PDU in response.

Based on the above analyese, we need to at least add the following cause values:

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unknown RAN A-IoT Device NGAP ID | The action failed because the receiving node does not recognise the RAN A-IoT Device NGAP ID. |
| Requested A-IoT service area information not served by the gNB | The action failed because neither the AIoT Area(s) nor the reader(s) indicated in the Requested Service Area Information are served by the NG-RAN node. |
| Unknow A-IoT session | The action failed because the AIOT Session (identified by the combination of AIOTF Identifier and A-IoT Correlation Identifier) is unknown. |
| Device not reachable | The action failed because the device is not reachable. |
| Multiple A-IoT Session ID instances | The action failed because multiple instances of the same A-IoT Session had been provided to the NG-RAN node. |

| NAS cause | Meaning |
| --- | --- |
| No A-IoT NAS response from the device | The action failed because there is no A-IoT NAS response in the received D2R message(s) from the device. |

Proposal: Extend NGAP: Cause IE with the following new cause values: Unknown RAN A-IoT Device NGAP ID, Requested A-IoT service area information not served by the gNB, Unknow A-IoT session, Device not reachable, Multiple A-IoT Session ID instances, No A-IoT NAS response from the device.

# Proposal

In this contribution, we discuss the missing A-IoT dedicated cause values, and get the following proposal:

Proposal: Extend NGAP: Cause IE with the following new cause values: Unknown RAN A-IoT Device NGAP ID, Requested A-IoT service area information not served by the gNB, Unknow A-IoT session, Device not reachable, Multiple A-IoT Session ID instances, No A-IoT NAS response from the device.

The corresponding NGAP CR is provided in [1].

# Reference

[1]	R3-256640 Correction on A-IoT dedicated Cause values, R19 NGAP CR