3GPP TSG-RAN WG3 Meeting #129bis	R3-256641

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Consideration on A-IoT leftovers on IE details

Agenda Item:	9.2.7

Source:	Huawei, CMCC, China Unicom, China Telecom

Document for:	Disc

# Introduction

The Rel-19 Ambient IoT WI was completed in last RAN3 meeting, with few small leftovers on IE details which were remarked by magic sentences in asn.1 part:

AIoT-CorrelationIdentifier ::= OCTET STRING-- needs to be replaced by a constrained type definition --

AIoT-DeviceIdentificationRequested ::= CHOICE { singleDevice BIT STRING (SIZE(1..1024, ...)), groupDevices BIT STRING (SIZE(1..1024, ...)), allDevices NULL, choice-Extensions ProtocolIE-SingleContainer { { AIoT-DeviceIdentificationRequested-ExtIEs} } }-- the detailed encoding needs to be refined --

AIOTFIdentifier ::= OCTET STRING -- needs to be replaced by a constrained type definition --

AIoT-InventoryAssistanceInformation ::= SEQUENCE { expectedD2RMsgSize INTEGER (1..256, ...), approximateNoofTargetDevices INTEGER (1..65535, ...) OPTIONAL, timeInterval INTEGER (1..256, ...) OPTIONAL, -- the detailed value needs to be revisited -- iE-Extensions ProtocolExtensionContainer { { AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL, ... }

InventoryReportTransfer ::= SEQUENCE { correlationIdentifier AIoT-CorrelationIdentifier, globalgNB-ID GlobalGNB-ID, readerReportList AIoT-ReaderReportList,--the presence of the IE may be revisited-- inventoryCompleteIndication ENUMERATED {true, ...} OPTIONAL, iE-Extensions ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} } OPTIONAL, ... }

In this contribution, we further discuss these leftovers on IE details.

# Discussion

  - Encoding of A-IoT Correlation Identifier IE and AIOTF Identifier IE

Following can be found in CT4 agreed CR C4-253537:

The A-IoT Correlation Identifier is defined as Unit16 (Unsigned 16-bit integer), therefore in NGAP, we should update encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...), and remove the related comment;

Proposal 1: Update the encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...).

The AIOTF Identifier is defined by CT4 as NfInstanceId, the following can be found in TS 29.571:

| Type Name | Type Definition | Description |
| --- | --- | --- |
| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| NOTE 3: NFs shall be able to receive a NF Instance Id in any UUID format. |  |  |

Based on IETF RFC 9562, a UUID is 128 bits long, we need to update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)), and remove the related comment.

Proposal 2: Update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)).

  - Presence of Device Report List IE in the Inventory Report Transfer IE

In current NGAP spec, the Device Report List IE is mandatorily included in the Inventory Report Transfer IE, therefore it requires the gNB to send the Inventory Complete Indication together with the last Reader Report. The gNB will need to buffer the last received A-IoT NAS PDU(s) and send them together with the Inventory Complete Indication, this will lead to unnecessary delay for the report. It is proposed to change it from mandatory to optional.

Proposal 3: Update the presence of Device Report List IE in the Inventory Report Transfer IE from mandatory to optional.

  - Encoding of Time Interval IE

The Time Interval IE contained in the Inventory Assistance Information IE is defined as INTEGER (1..256, ...) Unit: 0.1 second, it seems to be a reasonable design and no strong need to update.

Proposal 4: Keept the current encoding of the Time Interval IE.

  - Encoding of A-IoT Device Identification Requested IE

In current NGAP spec, the A-IoT Device Identification Requested IE is designed as:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the AIoT device Identifier defined in TS 23.369 [z]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the Filtering Information defined in TS 23.369 [z]. |
| >All Devices |  |  | NULL |  |

It allows difference inventory cases, unless requested by other WGs, we do not see the need to change these three branches structure.

Furthermore, based on TS 23.369, “The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.” Therefore, for single device inventory and group devices inventory, the current semantics descriptions are all right.

Proposal 5: Keep the current encoding of the A-IoT Device Identification Requested IE.

# Conclusion

In this contribution, we further discussed the A-IoT leftovers on IE details, get the following proposals:

Proposal 1: Update the encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...).

Proposal 2: Update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)).

Proposal 3: Update the presence of Device Report List IE in the Inventory Report Transfer IE from mandatory to optional.

Proposal 4: Keept the current encoding of the Time Interval IE.

Proposal 5: Keep the current encoding of the A-IoT Device Identification Requested IE.

The corresponding NGAP CR is provided in [1].

# Reference

[1]	R3-256642 Correction on A-IoT leftovers on IE details, R19 NGAP CR