3GPP TSG-RAN WG3 #129bis                                                 R3-256656

Prague, CZ, 13th – 17th Oct, 2025

Agenda Item:	11.2.1

Source:	CATT

Title:	Discussion on SON enhancement for inter-CU LTM

Document for:	Discussion

Introduction

According to the WI [1], the following objective is captured:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

Here we provide some analysis on MRO enhancement for inter-CU LTM.

Discussion

In release 19, MRO enhancement for intra-CU LTM is specified to support following objectives:

- Failure type detection and optimization;

- Beam optimization;

- TA optimization;

For the first bullet, it is based on RLF Report to optimize handover triggering point and candidate cell selection. As a legacy solution, in release 20, MRO enhancement for inter-CU LTM also should take it as baseline.

For the other two bullets, RAN3 introduce network solution to perform MRO on beam and TA for intra-CU LTM because suitable beam and valid TA can improve LTM performance. For the same reason, optimization on beam and TA should also be discussed in case of inter-CU LTM.

Proposal 1: It is proposed to take MRO for intra-CU LTM as the baseline for MRO for inter-CU LTM. The work may include:

- Legacy MRO solution based on RLF Report for execution condition and candidate cell list optimization;

- Beam optimization and TA optimization, by network based or UE based solution.

As for sub-optimal successful LTM event, SHR may be enhanced for analysis of near failure handover. In release 19, SHR is not discussed in RAN3. For the time reason, we propose to discuss successful LTM after the real LTM failure.

Proposal 2: It is proposed to prioritize MRO for LTM failure over successful case.

In R19 mobility, L3 triggered LTM is supported. Based on L1 or L3 measurement report, network triggers LTM. The information is necessary for MRO to decide whether to optimize L3 or L1 configuration. It can be recorded by network as a kind of LTM configuration.

Proposal 3: It is proposed to use network based solution to identify L1 or L3 triggered LTM.

In release 19 MRO, Ping-Pong issue in case of intra-CU LTM has been discussed. The solution is CU detects LTM Ping-Pong occurs and then inform DU to solve the issue. In release 20, Ping-Pong issue should be considered in inter-CU LTM scenarios.

Proposal 4: It is proposed for RAN3 to discuss Ping-Pong issue in inter-CU LTM scenarios.

Subsequent LTM is supported in case of inter-CU LTM. So, MRO for subsequent LTM scenarios should be considered. For example, gNB1 triggers LTM preparation, candidate cells in gNB2 and gNB3 is configured. Later, UE move from gNB1-> gNB2-> gNB3. If LTM failure occurs from gNB2 to gNB3, the failure may be cause by gNB1 (wrong candidate cell selection) or gNB2(wrong handover triggering point). Not only last serving gNB2 but also gNB1 as initiating node is involved in MRO.

Proposal 5: It is proposed for RAN3 to discuss the subsequent inter-CU LTM scenario in which not only last serving gNB but also the initiating gNB should be involved to perform optimization.

In R19 mobility, CSI-RS configuration is supported. CSI-RS measurement should be considered in R20 MRO. For example, RAN3 considers CSI-RS level target selection.

Proposal 6: It is proposed to consider CSI-RS level target selection for inter-CU LTM.

Conclusions

Proposal 1: It is proposed to take MRO for intra-CU LTM as the baseline for MRO for inter-CU LTM. The work may include:

- Legacy MRO solution based on RLF Report for execution condition and candidate cell list optimization;

- Beam optimization and TA optimization, by network based or UE based solution.

Proposal 2: It is proposed to prioritize MRO for LTM failure over successful case.

Proposal 3: It is proposed to use network based solution to identify L1 or L3 triggered LTM.

Proposal 4: It is proposed for RAN3 to discuss Ping-Pong issue in inter-CU LTM scenarios.

Proposal 5: It is proposed for RAN3 to discuss the subsequent inter-CU LTM scenario in which not only last serving gNB but also the initiating gNB should be involved to perform optimization.

Proposal 6: It is proposed to consider CSI-RS level target selection for inter-CU LTM.

Reference

[1] RP-251869 “Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5” 3GPP TSG RAN Meeting #108