3GPP TSG-RAN WG3 Meeting #129bis	R3-256675

# Prague, Czech Republic, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1325 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NG Suspend and Resume during Hard FLSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For hard feeder link switchover, interruption time is not avoidable between the break of the old feeder link and the activate of the new feeder link. It’s beneficial to sync between the gNB on-board and the 5GC on ground, then 5GC could suspend the signalling/data transmission towards the gNB when the NG is suspended due to old feeder link broken, and transmission of DL signalling/data could be resumed as soon as possible when the new feeder link is ready. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the support for NG suspend/resume. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The packet loss during hard FLSO cannot be avoided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.2, 9.2.6.4, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

8.7.2	RAN Configuration Update

8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

8.7.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the NG Connection Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the NG connection accordingly.

-----------------Next Change-------------------

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |  |
| RAN Node Name | O |  |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |  |
| Supported TA List |  | 0..1 |  |  | Supported TAs in the NG-RAN node. | YES | reject |  |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  |  | - |  |  |
| >>TAC | M |  |  | 9.3.3.10 | Broadcast TAC | - |  |  |
| >>Broadcast PLMN List |  | 1 |  |  |  | - |  |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  |  | - |  |  |
| >>>>PLMN Identity | M |  |  | 9.3.3.5 | Broadcast PLMN | - |  |  |
| >>>>TAI Slice Support List | M |  |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |  |
| >>>>NPN Support | O |  |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |  |
| >>>>Extended TAI Slice Support List | O |  |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |  |
| >>>>TAI NSAG Support List | O |  |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |  |
| >>Configured TAC Indication | O |  |  | 9.3.3.50 |  | YES | ignore |  |
| >>RAT Information | O |  |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |  |
| Default Paging DRX | O |  |  | Paging DRX 9.3.1.90 |  | YES | ignore |  |
| Global RAN Node ID | O |  |  | 9.3.1.5 |  | YES | ignore |  |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  |  | YES | reject |  |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  |  | - |  |  |
| >>TNL Association Transport Layer Address | M |  |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |  |
| >>TNL Association Transport Layer Address at AMF | O |  |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |  |
| NB-IoT Default Paging DRX | O |  |  | 9.3.1.137 |  | YES | ignore |  |
| Extended RAN Node Name | O |  |  | 9.3.1.193 |  | YES | ignore |  |
| NG Connection Control | O |  |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the NG connection during hard feeder link switchover. | YES | ignore |  |
| Range bound |  |  | Explanation |  |  |  |  |
| maxnoofTACs |  |  | Maximum no. of TACs. Value is 256. |  |  |  |  |
| maxnoofBPLMNs |  |  | Maximum no. of Broadcast PLMNs. Value is 12. |  |  |  |  |
| maxnoofTNLAssociations |  |  | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |  |  |  |  |

-----------------Next Change-------------------

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

WUS-Assistance-Information,

XrDeviceWith2Rx,

NGConnectionControl

UNCHANGED PART OMITTED

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo,

id-ExtendedOldAMF,

id-Aerial-UE-FlightInformationOngoingReporting-failed,

id-NGConnectionControl

UNCHANGED PART OMITTED

-- **************************************************************

--

##### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	}|

{ ID id-AdditionalULI							CRITICALITY ignore	TYPE AdditionalULI 									PRESENCE optional	}|

{ ID id-NGConnectionControl						CRITICALITY ignore	TYPE NGConnectionControl							PRESENCE optional	},

...

}

-----------------Next Change-------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

NGAPIESupportInformationRequestList ::= SEQUENCE (SIZE(1.. maxnoofNGAPIESupportInfo)) OF NGAPIESupportInformationRequestItem

NGAPIESupportInformationRequestItem ::= SEQUENCE {

ngap-ProtocolIE-Id		ProtocolIE-ID,

iE-Extensions		ProtocolExtensionContainer { { NGAPIESupportInformationRequestItem-ExtIEs} }	OPTIONAL,

...

}

NGAPIESupportInformationRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGAPIESupportInformationResponseList ::= SEQUENCE (SIZE(1.. maxnoofNGAPIESupportInfo)) OF NGAPIESupportInformationResponseItem

NGAPIESupportInformationResponseItem ::= SEQUENCE {

ngap-ProtocolIE-Id				ProtocolIE-ID,

ngap-ProtocolIESupportInfo		ENUMERATED {supported, not-supported, ...},

ngap-ProtocolIEPresenceInfo		ENUMERATED {present, not-present, ...},

iE-Extensions		ProtocolExtensionContainer { { NGAPIESupportInformationResponseItem-ExtIEs} }	OPTIONAL,

...

}

NGAPIESupportInformationResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGConnectionControl ::= ENUMERATED {

suspend,

resume,

...

}

NgENB-ID ::= CHOICE {

macroNgENB-ID			BIT STRING (SIZE(20)),

shortMacroNgENB-ID		BIT STRING (SIZE(18)),

longMacroNgENB-ID		BIT STRING (SIZE(21)),

choice-Extensions		ProtocolIE-SingleContainer { {NgENB-ID-ExtIEs} }

}

NgENB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

-----------------Next Change-------------------

9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

id-DLPDUSetInformationMarkingSupportIndication			ProtocolIE-ID ::= 488

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= 489

id-MMSID												ProtocolIE-ID ::= 490

id-Indication-of-bitrate-adaptation						ProtocolIE-ID ::= 491

id-SCGActivationTime									ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingContol			ProtocolIE-ID ::= 494

id-NGConnectionControl					            	ProtocolIE-ID ::= a

-----------------End of the Change-------------------