3GPP TSG-RAN WG3 Meeting #129-bis	R3-256676

Prague, Czech Republic, 13th - 17th Oct, 2025

Title: 	(TP for ISAC TR) Network architecture for ISAC

Source: 	Huawei

Agenda item:	13.2

Document Type:	pCR

# 1. Introduction

The new study item on Study on Integrated Sensing and Communication (ISAC) for NR was agreed in [1], with the following contents.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution provides our views on the network architecture for gNB-based mono-static for UAV use case. The companion paper on the sensing procedures and signalling can be found in [2].

# 2. Discussion

## 2.1 SA2 progress on the system architecture

SA2 already initiated the study on ISAC, and captured the key issues as well as potential solutions in TR 23.700-14 [3]. Especially the key issue #1 is to study the System Architecture to Support Sensing, where any new and/or existing functions in the Core Network and their interactions would be studied for Sensing.

So far overall 39 solutions were discussed in SA2, where multiples solutions proposed the RAN/CN network architecture covering the following two connection options between the gNB and the Sensing Function (Note that the exact terminology of sensing function would be finally decided by SA2).

- Option 1: Direct connection

- Option 2: Indirect connection by AMF

Below table provides the exemplary summary on the two options mentioned in [3], not in an exhaustive way.

| Solutions | Descriptions in TR23.700-14 | Direct connection between gNB and Sensing function | Indirect connection via the AMF |
| --- | --- | --- | --- |
| #1 | Support of Sensing operations with direct interfaces | Y |  |
| #3 | Figure 6.3.1.2-1: Reference Architecture NOTE: Whether the control signalling to Sensing Entity/Entities is sent via AMF or directly is addressed by the other solutions. | - | - |
| #4 | <The AMF is in the Figure 6.4.2-1, but no clear descriptions> | - | - |
| #5 | AMF: - Supports interaction with the SF for the reception of sensing requests from the SF and for provisioning of the sensing data to the SF. - Supports selection of sensing entity/entities and interaction with the selected sensing entity/entities for requesting execution of sensing and for reception of sensing data. |  | Y |
| #6 | <No AMF is in the Figure 6.6.3-1> | Y |  |
| #7 | - AMF: The AMF interface with the SSMF is supported to send the sensing configuration request for sensing entities and the sensing data response from the sensing entities to the SSMF. |  | Y |
| #13 | AMF: - According to sensing entity subscription/request from the SEMF to response the sensing entity info. Sensing Entity: - Provides its sensing related info to the AMF (ID, sensing capability). |  | Y |
| #16 | Impacts to the AMF: - In case of Indirect Connectivity when the SE communicates with the SF via the AMF, the AMF supports forwarding of signalling between the SE and SF. | - | - |
| #27 | <No AMF in the Figure 6.27.2.2-1> | Y |  |
| #28 | < No AMF in the Figure 6.28.4-1> | Y |  |
| #30 | < No AMF in the Figure 6.30.2-1> | Y |  |
| #31 | < No AMF in the Figure 6.31.2-1> | Y |  |
| #32 | < No AMF in the Figure 6.32.2-1> | Y |  |
| #34 | < No AMF in the Figure 6.34.2-1> | Y |  |
| #35 | AMF: - Expose services to receive sensing parameters from SSF and forwarding to NG-RAN. |  | Y |
| #39 | AMF: - For supporting sensing, the AMF needs to support additional services to transport configurations related to sensing. |  | Y |

From the above table, it can be observed that those SA2 solutions already proposed either the direct or indirect connection between the gNB and the Sensing Function.

- Observation 1:  The SA2 key issue#1 focuses on the system architecture for sensing. And multiple solutions were proposed in TR 23.700 the direct connection and indirect connection between the gNB and the Sensing Function.

## 2.2 Network architecture for gNB-based mono-static sensing

As observed from RAN SID [1], one main objective is to study the network architecture for gNB-based mono-static sensing treating the UAV as the use case. As made in the above observation, SA2 already discussed the network architecture. It is expected that SA2 can make initial conclusion in Oct. meeting, and make final in their last SI meeting. From RAN3 perspective, it is better to leave the network architecture conclusion (including the direct connection and indirect connection) to SA2. This is not only beneficial to avoid any potential parallel work, but also allows RAN3 to concentrate on the RAN own aspects.

The network architecture for ISAC (including the direct connection and indirect connection between the gNB and the Sensing Function) is left to SA2 conclusion.

To support ISAC, RAN3 should discuss the applicable interface and the protocol stack between the RAN and the CN. To facilitate the discussion during the study item before SA2 final decision, RAN3 can define and use the Sensing CN as the terminology, which may include the AMF and Sensing function, or Sensing Function only, up to SA2 decision.

To facilitate RAN3 discussion on the applicable interface, and protocol stack, RAN3 can define and use the Sensing CN during the SI, which may include the AMF and Sensing function, or Sensing Function only, up to SA2 decision.

For the applicable interface between the gNB and Sensing CN, the R19 Ambient IoT already went through very thorough analysis during the SI and WI phase, and finally concluded that the NG interface is reused. We envision that the NG-interface can be also reused for R20 ISAC with necessary enhancement. That means, between the gNB and the Sensing CN,

- the sensing specific control plane information is transported via the NG-C interface,

- the sensing data is delivered over the NG-U interface if necessary.

Then for the protocol stack, the existing NG stack can be reused for sensing. Below the Fig.1 and Fig.2 depict the network system architecture, control-plane and user-plane Protocol Stack respectively. And two editor’s notes are provided.

Figure 1 system architecture for sensing Figure 2. Control-plane Protocol Stack for sensing Figure 3. User-plane Protocol Stack for sensing Editor’s Note 1: The Sensing CN may include the AMF and Sensing Function, or Sensing Function only, which is up to SA2 decision. Editor’s Note 2: The terminology and the detailed functions for the Sensing Function is up to SA2 decision.

Based on the above, we have the following proposals.

RAN3 to capture the system architecture, where the NG interface is between the sensing gNB and sensing CN, with an editor’s note indicating that the Sensing CN could include AMF and Sensing function, or sensing function only up to SA2 decision.

RAN3 to capture the protocol stack, where the NGAP is used to transport the sensing specific information, and the GTP-U is to deliver the sensing data, if UP report is agreed, between the gNB and the sensing CN.

For the RAN split architecture for ISAC, we consider that there is no need to study it during the study item. First this is not covered in the RAN SID [1], second, it is not important considering the fact that the RAN split architecture can be studied during the normative phase when the whole RAN/CN signalling procedures are clear, or can be left out totally.

No need to study the sensing for RAN split architecture during the study item.

- Last, for the procedures and signalling for sensing, we think that the basic procedures and signalling discussed in RAN3 should be generic, and have no dependent on the two architecture options. The details can be found in our companion paper [2].

The basic procedures and signalling discussed in RAN3 should be generic, without any dependent on the network architecture options which would be concluded in SA2.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

- SA2 progress on the system architecture

- Observation 1:  The SA2 key issue#1 focuses on the system architecture for sensing. And multiple solutions were proposed in TR 23.700 the direct connection and indirect connection between the gNB and the Sensing Function.

- Network architecture for gNB-based mono-static sensing

- The network architecture for ISAC (including the direct connection and indirect connection between the gNB and the Sensing Function) is left to SA2 conclusion.

To facilitate RAN3 discussion on the applicable interface, and protocol stack, RAN3 can define and use the Sensing CN during the SI, which may include the AMF and Sensing function, or Sensing Function only, up to SA2 decision.

RAN3 to capture the system architecture, where the NG interface is between the sensing gNB and sensing CN, with an editor’s note indicating that the Sensing CN could include AMF and Sensing function, or sensing function only up to SA2 decision.

RAN3 to capture the protocol stack, where the NGAP is used to transport the sensing specific information, and the GTP-U is to deliver the sensing data, if UP report is agreed, between the gNB and the sensing CN.

No need to study the sensing for RAN split architecture during the study item.

The basic procedures and signalling discussed in RAN3 should be generic, without any dependent on the network architecture options which would be concluded in SA2.

The Annex 5 provides the pCR on the network architecture for the sensing.

# 4. Reference

- RP-251861 New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- R3-256677	(TP for ISAC TR) RAN-CN procedures and signaling for ISAC, Huawei

- TR 23.700-14 Study on Integrated Sensing and Communication

# 5. Annex – pCR for sensing

CHANGES START

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause attempts to identify and describe the system architecture to support the sensing in the overall 5G system architecture.

This clause also attempts to identify a functional split between RAN and the CN.

Figure 7.1 depicts a system architecture, where the NG interface is between the gNB and the sensing CN.

Figure 7.1 system architecture for sensing

Figure 7.2 and Figure 7.3 depict the control-plane and user plane Protocol stack between the gNB and Sensing CN respectively.

Figure 7.2. Control-plane Protocol Stack for sensing

Figure 7.3. User-plane Protocol Stack for sensing

Editor’s Note 1: The Sensing CN may include the AMF and Sensing Function, or Sensing Function only, which is up to SA2 decision.

Editor’s Note 2: The terminology and the detailed functions for the Sensing Function is up to SA2 decision.

CHANGES END