3GPP TSG-RAN WG3 Meeting #129bis                                                        R3-256684

Prague, CZ, 13th–17th October, 2025

Agenda item	: 12.2.2

Source		: Tejas Networks

Title			: AI/ML Enhancements for Intra gNB CU LTM

Document Type	: Discussion

______________________________________________________________________

1. Introduction

The finalized objectives for 5GA REL 20 are following[1]:

In this TDoc, we propose the introduction of an AI/ML model to optimize the Intra gNB CU LTM use case.

______________________________________________________________________

2. Discussion

2.1. Need for AI/ML Model for the Intra gNB CU LTM use case

In 5G NR, inter-gNB DU LTM often involves allocating resources to multiple candidate cells for potential handovers, leading to signaling overhead and inefficient utilization of radio and processing resources. An AI/ML model can address this by predicting the most probable target cell(s) based on input features such as UE mobility history, radio measurements (RSRP, RSRQ, SINR), UE location, etc. Such predictive models can enhance decision-making in mobility management by learning from past handover outcomes and adapting to real-time conditions. By accurately determining candidate cell(s), the model minimizes redundant resource allocation and signaling. This enhances mobility especially in dense deployments.

Proposal 1: RAN3 to consider the introduction of the AI/ML model for Intra gNB-CU LTM, aimed at significantly reducing the number of candidate cells involved in LTM preparation phase.

2.2. Location of the model

Since the initial list of candidate cells is selected by the gNB-CU in intra gNB CU LTM, it is most appropriate to introduce the AI/ML model at the gNB-CU level(gNB-CU-CP). This approach not only aligns with the functional responsibilities of the gNB-CU but also ensures consistency with existing standards.

Proposal 2: RAN3 to consider Introducing the model in gNB-CU-CP where both model training and model inference are performed.

2.3 Inputs, outputs and feedback for AI/ML-Based Intra gNB CU LTM

2.3.1 Inputs for AI/ML-Based Intra gNB CU LTM

Based on the inputs and outputs defined in Technical Report [2] for AIML-based mobility optimization, the following data can be considered as input:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

-    Radio measurements that indicate the UE's mobility speed, eg., DL timing drift.

-	UE Mobility History Information of at least last 3 hops.

From the neighbouring gNB DUs:

-	Current resource status

From the serving gNB DU:

-	Current resource status

From the gNB CU-UP:

-     Current/Predicted UE traffic

From the gNB CU-CP:

- UE history information

- Predicted Resource status

- Predicted Cell level UE trajectory

2.5. Output for AI/ML based Intra gNB CU LTM

AI/ML based Intra gNB CU LTM can generate the following information as output:
      -     Predicted candidate cell

-     Model output validity time

2.6. Feedback for AI/ML based Intra gNB CU LTM

The following data is required as feedback data for mobility optimization.

-	UE performance of the hand overed UE’s.

-	Resource status information updates from target gNB DU.

The inputs required from the UE side can be obtained via RRC measurement reports, while the gNB Distributed Units (DUs) can provide their current resource status through RESOURCE STATUS messages.For Current /predicted UE traffic, utilize the DATA COLLECTION procedure in E1AP to report these measurements from the gNB-CU-UP to the gNB-CU-CP. Additionally, the performance metrics of hand overed UEs(Feedback) can be conveyed from the gNB-CU-UP to the gNB-CU-CP through the same DATA COLLECTION procedure, as introduced in earlier 3GPP releases.

Proposal 3:RAN3 to consider enhancing existing signaling procedures to support the transmission of inputs required for both training and inference of machine learning models.

The above method aligns well with the existing AI/ML framework. However, enabling gNB Distributed Units (DUs) to host, train, and infer machine learning models introduces several enhancements—most notably, a reduction in workload on the gNB Central Unit (CU). This decentralized approach as shown below allows for more efficient processing, faster decision-making, and improved scalability across the network.

Distributed Predictive Mobility Management

This proposed method aims to enhance mobility management efficiency in 5G networks by reducing unnecessary signaling and enabling intelligent, data-driven handover decisions. It introduces a predictive approach where in each gNB-DU, there are local AI/ML models designed to estimate its predicted resource status etc.

The Predicted Resource status along with other parameters like UE location and movement trends, current traffic conditions etc, is sent to the gNB-CU (Central Unit). The CU acts as the central decision-maker and collects this information from all neighboring DUs that could potentially serve the UE. Based on this data, the CU evaluates the suitability of each candidate DU by the use of a central AI/ML model and select the most appropriate target for a potential handover. Traditionally, a mobility event might require the CU to initiate signaling exchanges with multiple candidate DUs to gather resource and capability information. However, with predictive data already available, the CU can avoid these redundant messages and directly choose the optimal DU. This results in faster decision-making, reduced latency, and lower traffic between the CU and DUs.

Proposal 4: RAN3 to reconsider the introduction of model training and inferencing capabilities within the gNB DU.

______________________________________________________________________

5. Conclusions

Proposal 1: RAN3 to consider the introduction of the AI/ML model for Intra gNB-CU LTM, aimed at significantly reducing the number of candidate cells involved in LTM preparation phase.

Proposal 2: RAN3 to consider Introducing the model in gNB-CU-CP where both model training and model inference are performed.

Proposal 3:RAN3 to consider enhancing existing signaling procedures to support the transmission of inputs required for both training and inference of machine learning models.

Proposal 4: RAN3 to reconsider the introduction of model training and inferencing capabilities within the gNB DU.

__________________________________________________________________

6. References

[1] RP-251868-New SI: Study on AI/ML for NG-RAN Phase 3 (RAN3-led)

[2] TR 37.817-study on enhancements for Data Collection for NR and EN-DC