3GPP TSG-RAN WG3 Meeting #129-bis	R3-256686

Prague, Czech Republic, 13th–17th October, 2025

Title:	Discussion on RAN-CN procedures and signaling for ISAC

Source:	Tejas Networks

Agenda Item:	13.3

Document for:	Discussion

# Introduction

A new study on integrated sensing and communications (ISAC) has been approved in RAN #108 [1], where the objectives of the study for 5GA-based ISAC are as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we discuss the signaling procedures between RAN and CN to support ISAC. SA2 has agreed that a new Network Function (NF), namely, Sensing Function (SF) will be introduced in the Core Network to handle sensing related functionality. A new protocol namely NR Sensing Protocol (NRSP) is needed between RAN and the SF to facilitate sensing procedures.

# Discussion

## 2.1 TRP Information Exchange

The purpose of the TRP Information Exchange procedure is to allow the SF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure is used to support sensing capabilities and sensing related configuration exchange. The SF initiates the procedure by sending a TRP Information message with TRP list included. The NG-RAN node responds with a TRP Information Response message that contains the requested TRP information.

## 2.2 Sensing Configuration Exchange

Sensing configuration procedure is responsible for configuring, modifying or releasing sensing configuration, The Sensing Configuration Exchange procedure is initiated by the SF to request the NG-RAN node to configure or modify or release Sensing Reference Signal configuration. The SF initiates the procedure by sending a Sensing Reference Signal (SeRS) Configuration request message to the NG-RAN. If SeRS transmission is successfully configured or modified or released for at least one of the TRPs, the NG-RAN node shall respond with a Sensing Configuration Response message.

## 2.3 Sensing Measurement

The Sensing Measurement procedure allows the SF to request one or more TRPs in the NG-RAN node to perform and report sensing measurements. The LMF initiates the procedure by sending a Sensing Measurement Request message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. Along with TRP List, SF will include following IEs:

- TRP Measurement Request List

- Report Characteristics (On demand, Periodic, Event based)

- Measurement Periodicity

- TRP Measurement Quantities

  - TRP Measurement Type (Location, Velocity, type etc.)

If Report Characteristics is set to “On demand”, NG-RAN node will perform sensing and report the sensing result to SF immediately in Sensing Measurement Response. If Report Characteristics is set to “Periodic” or “Event based”, NG-RAN will perform sensing and report sensing result in Sensing Measurement report message as per Measurement Periodicity or when event is triggered.

The purpose of the Sensing Measurement Abort Procedure is to enable the SF to abort an on-going measurement.

Proposal 1: Above sensing and TRP procedures need to be studied and specified to support sensing

Above procedures need a protocol (name FFS, a temporary name NRSP (NR Sensing Protocol) is used for ease of reference in this document) between RAN and SF to exchange the messages. This new Protocol carries information between the NG-RAN Node and the Sensing Function in Core Network.  The NRSP protocol may support the following sensing functions:

- Request sensing-related information and measurements from the gNB to the SF

- Transfer of information between the SF and the gNB for sensing configuration

Alternative 1: Transport of NRSP over NGAP shall be based on the encapsulation of NRSP PDUs into appropriate NGAP messages (NRSP TRANSPORT messages), in both “downlink” (from the AMF to the gNB) and “uplink” (from the gNB to the AMF). The below figure shows the protocol layering used to support transfer of NRSP PDUs between SF and NG-RAN Node.

Figure 1 Protocol Layering for SF to NG-RAN Signaling

- Alternative 2: A new interface between RAN and SF to exchange NRSP PDUs

- Observation 1: RAN to CN specification effort and complexity can be reduced if Alternative 1 is selected

- Observation 2: Alternative 2 provides direct interface between RAN and SF dedicated for sensing without involving AMF

- Proposal 2: The study should cover both the Alternative 1 (via AMF) and Alternative 2 (Without AMF)

In case of Alternative1, for exchange of NRSP PDUs, non-UE-associated NRSP message transport is needed in NGAP.

- Proposal 3 Support non-UE-associated NRSP message transport in NGAP.

Both “uplink” (from the gNB to the AMF) and “downlink” (from the AMF to the gNB) transport messages are needed.

- Proposal 4:  Define both “uplink” (from the gNB to the AMF) and “downlink” (from the AMF to the gNB) NRSP transport messages in NGAP.

# 3	Conclusion

To support gNB-based sensing, TRP Information exchange, Sensing Configuration exchange, Sensing Measurement procedures needs to be defined. we need a mechanism to transport NRSP messages between the gNB and the SF.

Proposal 1: Above sensing and TRP procedures need to be studied and specified to support sensing

- Proposal 2: The study should cover both the option 1 (via AMF) and Option 2 (Without AMF)

Proposal 3: Support non-UE-associated NRSP message transport in NGAP

Proposal 4: Define both “uplink” (from the gNB to the AMF) and “downlink” (from the AMF to the gNB) NRSP transport messages in NGAP

# 4 Annex

## NRSP transport

### General

The purpose of the NRSP Transport procedure is to carry NRSP signalling between gNB and SF over the NG Interface. The procedure uses non-UE associated signalling.

#### DOWNLINK NON-UE ASSOCIATED NRSP TRANSPORT

DOWNLINK NON-UE ASSOCIATED NRSP Transport Procedure

The AMF initiates the procedure by sending the DOWNLINK NON-UE ASSOCIATED NRSP TRANSPORT message to gNB.

#### UPLINK NON-UE ASSOCIATED NRSP TRANSPORT

UPLINK NON-UE ASSOCIATED NRSP TRANSPORT Procedure

The gNB initiates the procedure by sending the UPLINK NON-UE ASSOCIATED NRSP TRANSPORT message to AMF.

# 5 References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication".

- TS 38.413: NG Application Protocol (NGAP)