3GPP TSG-RAN WG3 Meeting #129bis		R3-256699

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	Discussion on shared CFRA resource for inter-CU LTM

Agenda Item:	9.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the signaling enhancements needed to support CFRA resources indicated in the LTM Cell Switch Command MAC CE for inter-CU LTM.

- Discussion

In RAN2#129 meeting [1], RAN2 agreed that CFRA resource configuration could be included in LTM cell switch command MAC CE for inter-CU LTM

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, RAN3 reached the following agreement in RAN3#128 meeting [2] for the Xn interface:

RAN3#128 Agreement

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

In our review, the CFRA resource(s) provided by the candidate cell can be shared by multiple UEs in the source DU.

Compared with providing CFRA resources via pre-configured LTM candidate configurations, including CFRA resources in the LTM Cell Switch Command MAC CE is more efficient in terms of CFRA resource utilization. This is because the source DU can allocate a specific CFRA resource from the pool to a UE when it needs to perform an LTM cell switch. This approach is similar to the early-RACH resource sharing mechanism supported in Rel-18 LTM. The source DU is responsible for allocating specific CFRA resources for each UE to perform a RACH-based LTM cell switch while simultaneously avoiding CFRA resource conflicts. As a result, resource wastage can be significantly reduced.

When a dedicated CFRA resource is allocated by the source DU from the shared pool to a UE, the target cell needs to know which CFRA resource has been allocated and start monitoring the corresponding Msg1. Therefore, the source DU needs to indicate the preamble index and RA-RNTI to the target DU.

Proposal 1: Preamble index and RA-RNTI should be transmitted from source DU to the target DU after source DU sends an LTM cell switch command MAC CE to UE with CFRA resource included. RNA3 to discuss whether to reuse existing Xn and F1 interface messages or introduce new interface messages to support this.

- Conclusion

In this contribution, we discuss the signaling enhancement that are needed for supporting CFRA resource indicated in LTM cell switch command MAC CE for inter-CU LTM. We have the following proposal:

Proposal 1: Preamble index and RA-RNTI should be transmitted from source DU to the target DU after source DU sends an LTM cell switch command MAC CE to UE with CFRA resource included. RNA3 to discuss whether to reuse existing Xn and F1 interface messages or introduce new interface messages to support this.

- References

- RAN2#129_ChairNotes.

- RAN3#128_ChairNotes.