3GPP TSG-RAN WG3 Meeting #129bis	R3-256701

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on Average Window for Alternative QoS

Document for:	Discussion and Decision

# 1. Introduction

RAN3 received reply LS from SA2 on the Average Window for Alternative QoS in S2-2508118. However, SA2 didn’t give any clear instructions on how to use the averaging window for alternative QoS. So, in this paper, we provide our view on the necessity of introducing averaging window in Alternative QoS profiles.

# 2. Discussion

According to TS 23.501 [1], the Alternative QoS Profile represents a combination of QoS parameters PDB, PER, GRBR and the Averaging Window. In addition, it may optionally include the MDBV for delay-critical traffic. But in the stage 3 specifications (e.g., NGAP spec in TS38.413[2]), as excerpted below, the alternative QoS profile does not include the Averaging Window.

------ Excerption from TS 23.501 ------ - 5.7.1.2a Alternative QoS Profile The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. If the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL (described in clause 5.7.7), the Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s); otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER. NOTE 1: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER (or PSDB or PSER) monotonically increase as the Alternative QoS Profiles become less preferred. For a GBR QoS Flow using the Delay-critical resource type, an Alternative QoS Profile may also include the QoS parameter MDBV. For the NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled (i.e. for all Notification control scenarios described in clause 5.7.2.4), the NG-RAN shall also apply the value of the optional QoS parameter present in an Alternative QoS Profile instead of the corresponding QoS parameter's value in the QoS Profile (which is either provided explicitly or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4) for every optional QoS parameter present in an Alternative QoS Profile (e.g. MDBV). ------ Excerption from TS 38.413 ------ - 9.3.1.151 Alternative QoS Parameters Set List This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Alternative QoS Parameters Set Item; 1..<maxnoofQoSparaSets>. >Alternative QoS Parameters Set Index; M; 9.3.1.152; -. >Guaranteed Flow Bit Rate Downlink; O; Bit Rate 9.3.1.4; -. >Guaranteed Flow Bit Rate Uplink; O; Bit Rate 9.3.1.4; -. >Packet Delay Budget; O; 9.3.1.80; -. >Packet Error Rate; O; 9.3.1.81; -. >Maximum Data Burst Volume; O; 9.3.1.83; Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise.; YES; ignore.

------ Excerption from reply LS S2-2508118 ------

- 1	Overall description

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] SA2 has discussed the LS and there are other understandings beyond Understanding 1 and 2. There is no consensus on how to select and the impacts of using the averaging window of current activated QoS Profile by NG-RAN. SA2 would like to ask RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

------ Excerption from reply LS S2-2508118 ------

- 1	Overall description

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] SA2 has discussed the LS and there are other understandings beyond Understanding 1 and 2. There is no consensus on how to select and the impacts of using the averaging window of current activated QoS Profile by NG-RAN. SA2 would like to ask RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

In RAN3-127bis meeting, the paper [3] proposed to add the averaging window in the Alternative QoS Parameters Set List contained in NGAP specs, to align with TS23.501. But some companies in RAN3 have concern on how can the RAN use different averaging window in different Alternative QoS parameters Set. So, RAN3 sent the LS to SA2 (R3-252397[4]) to ask clarification on the motivation of introducing averaging window in the alternative QoS profile. In this meeting, we get response from SA2 in S2-2508118[5], the main content is copied below:

From the reply LS, SA2 cannot reach consensus on how to select and use the averaging window when the AQP are present in NG-RAN, so they decided to leave this problem to be solved by RAN3, and SA2 cannot provide any clarification on the motivation for introducing averaging window in the Alternative QoS profile.

In fact, SA2 has discussed this issue based on the contribution S2-2502110[6] several months ago. Some companies suggested to remove the averaging window from the Alternative QoS profile, but such issue looks very controversial and there was no consensus.

Observation 1: SA2 introduced the averaging window in the Alternative QoS profile, but without consensus on the motivation, and some companies in SA2 suggested to remove this parameter.

Observation 2: How to use the averaging window for the presence of alternative QoS is up to RAN.

To proceed, RAN3 needs to discuss the following two issues:

- 1) Whether the averaging window is needed in alternative QoS parameter? In other words, whether the Notification control with Alternative QoS Profiles can work well without such parameter in AQP?

- 2) If the averaging window is included as one parameter in the alternative QoS parameter set, how will the NG-RAN node perform the QoS monitoring to decide whether the QoS profile can be fulfilled, and/or which alternative QoS profile can be fulfilled?

About the first problem, RAN3 has the understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. So, we do not see any issue if the alternative QoS parameters do not contain averaging window. In contrast, this will simplify the QoS monitoring handling at NG-RAN node, i.e., the NG-RAN node can decide whether the QoS profile/which Alternative QoS profile can be fulfilled using one measurement process within the averaging window contained in the QoS profile.

About the second problem, it is still unclear and hard to reach consensus on how to perform at RAN. If RAN follows the understanding 2, the NG-RAN node needs to initiate multiple measurement processes with different averaging window for one QoS flow. For example, for one QoS flow, if the averaging window in QoS profile is 2000ms, while this value is 1000ms in the first Alternative QoS parameter set and 1500ms in the second Alternative QoS parameter set, the NG-RAN node needs to have 3 different measurement processes with 3 different windows, to decide whether the QoS profile can be fulfilled and which Alternative QoS profile can be fulfilled. Such implementation will increase the complexity for the RAN.  If RAN use the averaging window as included in the current fulfilled (alternative) QoS profile, RAN needs to adjust the measurement process frequently to align with different configuration of alternative QoS. Besides, it also makes the evaluation criteria for the QoS profile changes from time to time. Still taking the above example as an illustration, when the first alternative QoS profile is fulfilled, RAN can evaluate whether the original QoS profile is fulfilled based on the 1500ms averaging window. But when the second alternative QoS profile is fulfilled, RAN has to evaluate whether the original QoS profile is fulfilled based on the 1000ms averaging window. This may lead to different results on determination of whether the QoS profile/which alternative QoS profile can be fulfilled.

Observation 3: If the averaging window is included as one parameter in the alternative QoS parameter set, the RAN may need to initiate multiple measurement processes with different value of window size, or need to adjust the measurement process frequently to align with different configuration of alternative QoS. It is hard to foresee any benefits apart from increasing the complexity of RAN implementation.

Observation 4: The Notification control with Alternative QoS Profiles works well with understanding 1, which facilitates the simplest RAN implementation for determining the fulfilled (alternative) QoS profile.

Based on the above analysis, we propose that

Proposal 1: RAN3 to adopt understanding 1 if the AQP are present in the NG-RAN, and ask SA2 to remove the averaging window from the alternative QoS profile.

# 3. Conclusion

In this contribution, we analyzed the issue regarding the necessity and usage of the averaging window in alternative QoS, and provide the following observations and proposals:

Observation 1: SA2 introduced the averaging window in the Alternative QoS profile, but without consensus on the motivation, and some companies in SA2 suggested to remove this parameter.

Observation 2: How to use the averaging window for the presence of alternative QoS is up to RAN.

Observation 3: If the averaging window is included as one parameter in the alternative QoS parameter set, the RAN may need to initiate multiple measurement processes with different value of window size, or need to adjust the measurement process frequently to align with different configuration of alternative QoS. It is hard to foresee any benefits apart from increasing the complexity of RAN implementation.

Observation 4: The Notification control with Alternative QoS Profiles works well with understanding 1, which facilitates the simplest RAN implementation for determining the fulfilled (alternative) QoS profile.

Proposal 1: RAN3 to adopt understanding 1 if the AQP are present in the NG-RAN, and ask SA2 to remove the averaging window from the alternative QoS profile.

# 4. Reference

[1]. 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]. 3GPP TS 38.413: " NG-RAN; NG Application Protocol (NGAP)".

[3]. R3-251773, Correction on Alternative QoS parameter, CATT, Nokia, Qualcomm.

[4]. R3-252397, LS on Average Window for Alternative QoS, RAN3(CATT).

[5]. S2-2508118, LS on Average Window for Alternative QoS, SA2(CATT).

[6]. S2-2502110, Discussion on Averaging Window in AQP, Ericsson.