3GPP TSG-RAN WG3 Meeting #129bis	R3-256716

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:		10.2.1

Source:	T-Mobile USA, Qualcomm, Nokia

Title:	6G Service Aware RAN Architecture Option

Document for:		Discussion and Approval

# Introduction

3GPP wireless technology advancement has been the catalyst for the emergence of new services. A service aware RAN (SAR) approach is proposed here to address the real time user experience challenge. The current 5G network QoS is static, and there is no real-time observability of the services in RAN. The introduction of new 6G use cases such as Robotics, ISAC, AI applications etc. will require such dynamic support from service aware RAN since RAN is typically the bottleneck in wireless networks.

In this contribution, we discuss the importance of Service Aware RAN and propose that RAN3 should look into 6G RAN architecture for supporting SAR.

# Discussion

## Background

In the current 5G system, 5QI is used to define the specific quality profile for a service by the core network. Different 5QI value correspondent to a set of service level thresholds such as throughput, latency, error rate etc. 5QI index is passed to the RAN in which the 5QI value, along with other RAN domain parameters, is used by the MAC scheduler to calculate the MAC scheduling priority, weights etc. for each data flow. Such mechanism works for a system carrying data applications that do not require real-time observability of application QoE for the RAN to manage the resource efficiently, such as web browsing and non-real time video streaming, however, RAN will struggle to support applications, such as XR, which require both stringent throughput and latency adjustments in real time unless the RAN is over dimensioned to ensure the quality of service for such applications. This makes it very challenging for operators to launch such new services without compromising the overall system spectrum efficiency and legacy eMBB service performance.

3GPP made the efforts to address this issue by introducing RvQoE which allows the application attributes such as video buffer level to be transferred to the AS layer through RRC containers, however, in commercial network, operators ran into challenges from device ecosystem players who normally prevent the OS from sending the application information/APP IDs to the AS layer for various reasons. Operators have very limited visibility and control over the massive and diversified applications that are going through the network and are not able to effectively meet all applications’ performance targets while maintaining healthy network spectrum and operation efficiency. To solve this problem, fundamental changes are needed to make RAN be more adaptive to the applications. Service Aware RAN for 6G aims to address the issue from the basic protocol and architecture levels.

The current 5G system uses the same protocol stacks regardless of the type of services. Service aware RAN seeks to understand the services requirements, e.g. FWA, IoT, robotics, skipping protocols in mobility, user plane or retransmission and adeptly enable RAN features based on services and user RF conditions in a dynamic manner.

Observation 1: Current 5G RAN does not have visibility of real-time application-level performance attributes to address the type of requirements that are expected for new service types, therefore it’s less effectively able to allocate radio resources and features that meet the applications’ minimum requirements while maintaining healthy overall system spectrum efficiency.

Service aware RAN addresses RAN ability to identify service types, map them to appropriate QoS profiles, detect user experience/SLA and trigger corrective actions in real-time

Proposal 1: Support RAN service characteristics and awareness (service-aware RAN) for 6G that adapts to application/service requirements

Current 5G RAN does not have the visibility of real-time application-level performance attributes and there is no mechanism in RAN for dynamic QoS. Therefore, RAN is less effectively able to allocate the radio resources to meet the applications minimum requirements while maintaining healthy overall system spectrum efficiency.

Proposal 2: 6G RAN shall support real-time service characteristic performance observability

Standard interface between RAN-CN is an important enabler to make RAN-service aware, expose and make dynamic RAN QoS/QoE possible. While eMBB continues to be an important service provided by operators, new services, such as robotics, XR or AI traffic, have significantly differentiated traffic profiles and performance criteria required to be treated as primary services. To meet QoE targets of specific service and real-time KPIs, dynamic changes may be needed in scheduler rules.

Proposal 3: 6G RAN shall be designed to provide service specific performance adaptation based on dynamic service performance observability, as close to real-time as possible

# Proposal

Based on the discussion in the previous sections, we propose the following:

Observation 1: Current 5G RAN does not have visibility of real-time application-level performance attributes to address the type of requirements that are expected for new service types, therefore it’s less effectively able to allocate radio resources and features that meet the applications’ minimum requirements while maintaining healthy overall system spectrum efficiency.

Proposal 1: Support RAN service characteristics and awareness (service-aware RAN) for 6G that adapts to application/service requirements

Proposal 2: 6G RAN shall support real-time service characteristic performance observability

Proposal 3: 6G RAN shall be designed to provide service specific performance adaptation based on dynamic service performance observability, as close to real-time as possible

# References

[1] RP-251986, T-Mobile USA, AT&T, Nokia, Ericsson, Telstra, Verizon, “Requirements for Service Aware 6G RAN”