3GPP TSG-RAN WG3 #129bis	R3-256785

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		13.3

Title:		Discussion on ISAC procedures and signaling

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the ISAC SID [1], the following objects should be studied in RAN that:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution will discuss the procedures and signaling regarding the ISAC.

# 2. Discussion

2.1 Discussion on procedures and signaling

Sensing RAN node information reporting:

For each network architecture mentioned in another contribution, prior to the Sensing Function (SF) transmitting any sensing-related signaling to the RAN, the RAN should first provide its sensing-related information to the SF. This enables the SF to become aware of the RAN node and to perform sensing RAN node discovery and selection based on the requirements of the sensing service requirement. The reported sensing information shall at least include the RAN node ID (e.g., Global gNB ID), the location of the sensing RAN node, and the sensing service area supported by the RAN node.

Since the RAN-SF interface may not be established when the sensing information is reported, the SF can acquire this information via OAM, and no RAN3 signaling is required.

Proposal 1: The SF can acquire the sensing-related RAN node information via OAM, and no RAN3 signaling is required.

Proposal 2: The reported sensing information shall at least include the RAN node ID, the location of the sensing RAN node, and the sensing service area supported by the RAN node.

RAN-SF Interface establishment procedure

For the direct connection between RAN and SF (Architecture 1 & 2) mentioned in another contribution, the corresponding interface needs to be established before any signaling is transmitted between RAN and SF. For Architecture 4 (i.e., the indirect connection between RAN and SF via the AMF, but with a new RAN-to-SF interface similar to NRPPa), the RAN–SF interface should also be established prior to any signaling transmission between them.

When a sensing service request arrives at the SF, the SF selects the appropriate sensing RAN node based on the available RAN node information and the sensing service requirements. The SF then sends an Interface Setup Request message to the selected sensing RAN node. If the interface is successfully set up, the sensing RAN node responds with an Interface Setup Response message. The signaling procedure is illustrated in Figure 1.

Figure 1: Interface establishment procedure for direct connection and indirect connection.

For Architecture 3 (i.e., the indirect connection between RAN and SF via the AMF without introducing a new interface), the existing N2 interface is reused to convey control signaling, and sensing data will be transported explicitly over N2 interface. In this case, no interface establishment procedure is required.

Observation 1: The interface establishment procedure is triggered after the SF selects the sensing RAN node, based on the sensing RAN node information and the sensing service requirements.

Observation 2: An interface establishment procedure is required if a new interface between RAN and SF is defined in RAN3.

Proposal 3: The need for an interface establishment procedure depends on whether a new interface between RAN and SF is defined.

Proposal 4: A Class 1 interface establishment procedure shall be defined for any new interface between RAN and SF, and the procedure shall be initiated by the SF.

Sensing service configuration

Following the sensing RAN node selection and interface establishment (if applicable), the SF shall send a Sensing Service Request message to the selected sensing RAN node to configure the sensing service (this transmission may occur via the AMF in the case of an indirect connection). If the sensing service is configured successfully (e.g., with the necessary resources allocated), the sensing RAN node shall send a Sensing Service Response message to the SF. An example signaling procedure is illustrated in Figure 2 &3.

Alternatively, the sensing RAN node may send the Sensing Service Response after it has collected some sensing data, in which case the response includes the collected sensing data.

Proposal 5: It is necessary to define a Class 1 procedure for sensing service initiation. This procedure consists of a Sensing Service Request (from the SF or AMF to the sensing RAN) and a Sensing Service Response (from the sensing RAN to the SF or AMF).

Proposal 6: The sensing RAN may send the Sensing Service Response message under the following conditions:

The sensing service has been successfully configured.

The sensing RAN node has collected some sensing data.

Figure 2: Sensing service trigger procedure if new interface with direct connection and indirect connection

Figure 3: Sensing service trigger procedure for N2 interface is reused

Furthermore, the Sensing Service Request message could also be used for RAN-SF interface establishment (if applicable), similar to the interface setup request message mentioned previously. In this case, the necessary parameters for interface setup would be included in the Sensing Service Request message.

Proposal 7: The Sensing Service Request/Response messaging may be used to initiate the RAN-SF interface setup.

Additionally, support for parallel sensing services towards a single sensing RAN node should be considered in RAN3. The sensing node could differentiate these services by allocating distinct sets of resources, and no further technical issues are foreseen. Therefore, from a technical perspective, a sensing RAN node should be capable of supporting parallel sensing services. These parallel services could be configured via separate Sensing Service Request messages or a single combined request.

Proposal 8: A sensing RAN node should support parallel sensing services, which may be configured via either multiple individual Sensing Service Request messages or a single Sensing Service Request message.

Sensing data reporting

As indicated in Proposal 6, the Sensing Service Response message can carry sensing data. However, a separate Class 2 Sensing Data Report message is still required. This is necessary for sensing services configured with periodic sensing service, where the sensing RAN node needs to report collected data for each measurement period. In such scenarios, using the Sensing Service Response message is not applicable.

Proposal 9: A Class 2 Sensing Data Report message is required for reporting the collected sensing data to the SF.

Sensing service revocation

Sensing service revocation is a key issue in TR 23.700-14 [2], and several solutions are captured in the TR (e.g., Solution #8, #9, #11, #12). The AF may decide to revoke an ongoing sensing service and send a sensing revocation request to the SF. The SF then initiates a Sensing Revocation Request to the sensing RAN node, which shall include the corresponding sensing session ID. Upon receiving the request, the sensing RAN node shall release the ongoing sensing session context and release the associated sensing resources. The RAN node subsequently sends a Sensing Revocation Response message to the SF to confirm the successful revocation. The signaling procedure is illustrated in Figure 4&5.

Figure 4: Sensing Revocation procedure for procedure if new interface with direct connection and indirect connection

Figure 5: Sensing service trigger procedure if N2 interface is reused

Proposal 10: A Class 1 sensing service revocation procedure shall be triggered by the SF. The sensing session ID shall be included in both the sensing revocation request and response messages.

Proposal 11: Upon receiving a Sensing Revocation Request, the sensing RAN node shall release the ongoing sensing session context and the allocated sensing resources.

# 3. Conclusion

Observation 1: The interface establishment procedure is triggered after the SF selects the sensing RAN node, based on the sensing RAN node information and the sensing service requirements.

Observation 2: An interface establishment procedure is required if a new interface between RAN and SF is defined in RAN3.

Proposal 1: The SF can acquire the sensing-related RAN node information via OAM, and no RAN3 signaling is required.

Proposal 2: The reported sensing information shall at least include the RAN node ID, the location of the sensing RAN node, and the sensing service area supported by the RAN node.

Proposal 3: The need for an interface establishment procedure depends on whether a new interface between RAN and SF is defined.

Proposal 4: A Class 1 interface establishment procedure shall be defined for any new interface between RAN and SF, and the procedure shall be initiated by the SF.

Proposal 5: It is necessary to define a Class 1 procedure for sensing service initiation. This procedure consists of a Sensing Service Request (from the SF or AMF to the sensing RAN) and a Sensing Service Response (from the sensing RAN to the SF or AMF).

Proposal 6: The sensing RAN may send the Sensing Service Response message under the following conditions:

The sensing service has been successfully configured.

The sensing RAN node has collected some sensing data.

Proposal 7: The Sensing Service Request/Response messaging may be used to initiate the RAN-SF interface setup.

Proposal 8: A sensing RAN node should support parallel sensing services, which may be configured via either multiple individual Sensing Service Request messages or a single Sensing Service Request message.

Proposal 9: A Class 2 Sensing Data Report message is required for reporting the collected sensing data to the SF.

Proposal 10: A Class 1 sensing service revocation procedure shall be triggered by the SF. The sensing session ID shall be included in both the sensing revocation request and response messages.

Proposal 11: Upon receiving a Sensing Revocation Request, the sensing RAN node shall release the ongoing sensing session context and the allocated sensing resources.

# References

[1] RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR

[2] TR 23.700, Study on Integrated Sensing and Communication