3GPP TSG-RAN WG3 Meeting #129bis	R3-256789

Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1338 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cyphering indication for A-IoT NAS Command Request PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.3.2 of TS 33.369, there is the following statement on information protection during the command procedure that: “3. The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN. 4. The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 5. The device shall derive the KCommand_enc, KCommand_int and verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KCommand_enc and KCommand_int key using the same algorithms.” The AIoTF should send an indication on whether the Command Request NSA PDU is cyphered or not to the NG-RAN. Therefore, a cyphering indication is needed in the COMMAND REQUEST message from AIoT CN to NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding an IE A-IOT NAS PDU cyphering indication in the COMMAND REQUEST message from AIoT CN to NG-RAN. Adding some text description about A-IOT NAS PDU cyphering IE in the Command Request Successful Operation section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is a misalignment between SA3 and RAN3 in terms of Command Request PDU cyphering. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.20.3.2, 9.3.6.5, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 38.410 CR 0047 TS 38.423 CR 1069 TS 38.473 CR 1070 TS 38.401 CR 0267 TS 38.420 CR 0038 TS 38.470 CR 0120 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 8.20.3	Command Request

#### 8.20.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.20.3.2	Successful Operation

Figure 8.xx.3.2-1: Command Request, successful operation.

The A-IoT CN node triggers the procedure by sending a COMMAND REQUEST message to the NG-RAN node. In case of successful operation, the NG-RAN node responds with the COMMAND RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE, the A-IoT Correlation Identifier IE and the RAN A-IoT Device NGAP ID IE in the COMMAND RESPONSE message as received from the COMMAND REQUEST message.

If the Command Assistance Information IE is included in the COMMAND REQUEST message, the NG-RAN node, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

If the A-IOT NAS PDU Cyphering IE is included in the COMMAND REQUEST message, the NG-RAN node shall follow the behaviours as described in TS 33.369 [z].

Next Change

#### 9.3.6.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| A-IOT NAS PDU Cyphering | O |  | ENUMERATED (activated, ...) | Indication on whether cyphering is activated defined in TS 33.369 [z]. | YES | reject |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Omit unrelated part

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AIoT-NASPDUCyphering,

Omit unrelated part

AIoTAreaID ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

nID					NID							OPTIONAL,

aIoTAreaCode		OCTET STRING (SIZE (3)),

iE-Extensions		ProtocolExtensionContainer { { AIoTAreaID-ExtIEs} } OPTIONAL,

...

}

AIoTAreaID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-CorrelationIdentifier ::= OCTET STRING-- needs to be replaced by a constrained type definition --

AIoT-FollowonCommandIndication ::= ENUMERATED {true, ...}

AIoT-NASPDUCyphering ::= ENUMERATED {true, ...}

RAN-AIOT-Device-NGAP-ID ::= INTEGER (0..4294967295)

Omit unrelated part

CommandRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { CommandRequestTransferIEs} },

...

}

CommandRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier				PRESENCE	mandatory	}|

{ ID id-RAN-AIOT-Device-NGAP-ID					CRITICALITY reject	TYPE RAN-AIOT-Device-NGAP-ID					PRESENCE	mandatory	}|

{ ID id-AIoT-NASPDU								CRITICALITY reject	TYPE AIoT-NASPDU								PRESENCE mandatory	}|

{ ID id-AIoT-CommandAssistanceInformation		CRITICALITY reject	TYPE AIoT-CommandAssistanceInformation	PRESENCE	optional		}|

{ ID id-AIoT-NASPDUCyphering					CRITICALITY reject	TYPE AIoT-NASPDUCyphering					PRESENCE	optional		},

...

}

CommandResponseTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

rAN-AIOT-Device-NGAP-ID			RAN-AIOT-Device-NGAP-ID,

aIoT-NASPDU						AIoT-NASPDU,

criticalityDiagnostics			CriticalityDiagnostics												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CommandResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

CommandResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CommandFailureTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

rAN-AIOT-Device-NGAP-ID			RAN-AIOT-Device-NGAP-ID,

cause							Cause,

criticalityDiagnostics			CriticalityDiagnostics												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CommandFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

CommandFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Next Change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Omit unrelated part

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

Omit unrelated part

id-InventoryRequestTransfer							ProtocolIE-ID ::= 500 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 501 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 502 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 503 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 504 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 505 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 506 --to be allocated

id-AIOTSessionReleaseCommandTransfer					ProtocolIE-ID ::= 507 --to be allocated

id-AIOTSessionReleaseCompleteTransfer					ProtocolIE-ID ::= 508 --to be allocated

id-AIOTSessionReleaseRequestTransfer				ProtocolIE-ID ::= 509 --to be allocated

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 510 --to be allocated

id-AIOTFIdentifier									ProtocolIE-ID ::= 511 --to be allocated

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 512 --to be allocated

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 513 --to be allocated

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 514 --to be allocated

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 515 --to be allocated

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 516 --to be allocated

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 517 --to be allocated

id-AIoT-NASPDU										ProtocolIE-ID ::= 518 --to be allocated

id-AIoT-Support											ProtocolIE-ID ::= 519 --to be allocated

id-AIOTFName											ProtocolIE-ID ::= 520 --to be allocated

id-AIoTNASPDUCyphering									ProtocolIE-ID ::= 5xx --to be allocated

End of Change