3GPP TSG-RAN WG3 Meeting #129 bis	R3-256794

Prague, Czech Republic, 13-17 October 2025

Agenda Item:		12.2.2

Source:		NEC

Title:			(pCR for TR38.745) AI/ML based intra-CU LTM

Document for:		Discussion and decision

# Introduction

A new Rel-20 SI on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 [1] was approved for 5G-A and the following objectives are included.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

- multi-hop UE trajectory across gNBs;

- intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In this contribution, we provide our initial considerations on AI/ML based intra-CU LTM and the draft version TP to TR38.745.

# Discussion

2.1	Use case description

L1/L2 Triggered Mobility (LTM) was introduced in Rel-18 to reduce overhead and interruption time through enabling mobility using lower-layer signaling. The basic procedures of intra-CU LTM are shown as figure 1 below.

Figure 1: Intra-CU LTM

In order to further improve reliability, reduce latency and optimize resource allocation of legacy intra-CU LTM mechanism, AI/ML based intra-CU LTM can be adopted to address potential mobility issues through predictive and data-driven approaches.

2.2	Solutions and standard impacts

2.2.1	Locations for AI/ML Model

In Rel-18 and Rel-19, high-level principles and functional framework for AI/ML for NG-RAN were agreed and two procedures, i.e., Data Collection Reporting Initiation procedure and Data Collection Reporting procedure were adopted for AI/ML data collection. In Rel-20, we need to take the agreements made in previous releases into account and agree to apply the following as the default deployment scenario for the locations for AI/ML Model Training and AI/ML Model Inference:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NB-CU.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB-CU.

Besides, for intra-CU LTM use case, since L1 measurements are collected by gNB-DU and it is gNB-DU to make LTM decision and trigger LTM cell switch towards the UE, it is also possible that AI/ML Model Inference is directly locates in gNB-DU. Therefore, for this use case, AI/ML Model Inference located in gNB-DU should also be considered.

Proposal 1: Study AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-DU.

Below is an example procedure for AI/ML based intra-CU LTM when AI/ML Model Inference located in gNB-CU:

Figure 2: AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-CU

Step 0. AI/ML Model Training function locates, collects the input data for training. For intra-CU LTM, the input data for training includes L1 and L3 measurements, UE location information, UE mobility history information, resource status (including cell level and beam level). After (re-)training, the AI/ML model is deployed in the gNB-CU or gNB-DU.

Step 1. gNB-CU configures UE with radio measurements information.

Step 2. UE collects the indicated measurement, e.g., L3 measurements of serving cell and neighbouring cells and sends measurement report message, including the required measurement, to gNB-CU.

Step 3. Model Inference. Required measurements are leveraged into Model Inference to output the LTM prediction, including e.g., predicated LTM candidate cells, predicted LTM configuration for each candidate cells.

Step 4. gNB-CU sends the LTM prediction to serving gNB-DU for LTM configuration, and gNB-CU configures the UE with LTM configuration of the predicted candidate cells.

Step 5. UE performs early synchronization with the predicted LTM candidate cells in both UL and DL. UE collects the required L1 measurements of serving cell and neighbouring cells and sends to serving gNB-DU. Serving gNB-DU can send the received L1 measurements to gNB-CU for LTM prediction.

Step 6. Serving gNB-DU makes intra-CU LTM decision and triggers intra-CU LTM cell switch towards the UE.

Step 7: UE executes intra-CU LTM cell switch towards the LTM target cell.

One step further, gNB-CU can also generate predicted TA value and predicted activated TCI state as LTM prediction in order to simplify UE behavior on early synchronization in both UL and DL.

When AI/ML Model Inference located in gNB-DU, gNB-DU can perform model inference and directly generate one predicted LTM target cell, based on the required input data, e.g. L1 measurements.

2.2.2	Input, output and feedback

As described above, a gNB may need the following information as input data for AI/ML based intra-CU LTM:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

-	Early synchronization information, including TCI state, RACH configuration.

From the gNB-DU:

-	UE L1 measurements

From the local gNB-CU:

-	Current/predicted resource status, including cell level and beam level

-	UE trajectory prediction

-	Current/predicted UE traffic

The output generated by AI/ML model inference is the LTM prediction, including predicated LTM candidate cells, predicted LTM configuration for each candidate cells, predicted TA value, predicted activated TCI state, predicated LTM target cell.

Legacy UE performance feedback, beam level UE performance feedback for those UEs that performed intra-CU LTM cell switch and SON reports (e.g., RLF, CEF, RA) can also be used as feedback data.

2.2.3	Potential standard impacts

According to the analysis above, following standard impacts are foreseen for subsequent Rel-20 normative work:

F1 interface:

-	Enhanced existing procedure or defined new procedure to collect information between gNB-CU and gNB-DU:

-	LTM prediction

-	UE L1 measurements

TP to TR38.745 which captures the above information is in Annex.

Proposal 2: Agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Proposals

In this contribution, we provide our initial considerations on AI/ML based intra-CU LTM, and the following proposals were drawn from the analysis.

Proposal 1: Study AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-DU.

Proposal 2: To agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Reference

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

# Annex	TP for TR38.745

<Start of change>

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Editor Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) was introduced in Rel-18 to reduce overhead and interruption time through enabling mobility using lower-layer signaling.

In order to further improve reliability, reduce latency and optimize resource allocation of legacy intra-CU LTM mechanism, AI/ML based intra-CU LTM can be adopted to address potential mobility issues through predictive and data-driven approaches.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML based intra-CU LTM:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NB-CU.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB-CU.

Besides, AI/ML Model Inference located in gNB-DU is also be considered for AI/ML based intra-CU LTM.

#### 4.2.2.2		Input data of AI/ML based intra-CU LTM

As described above, a gNB may need the following information as input data for AI/ML based intra-CU LTM:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

-	Early synchronization information, including TCI state, RACH configuration.

From the gNB-DU:

-	UE L1 measurements

From the local gNB-CU:

-	Current/predicted resource status, including cell level and beam level

-	UE trajectory prediction

-	Current/predicted UE traffic

#### 4.2.2.3	Output data of AI/ML based intra-CU LTM

AI/ML based intra-CU LTM model can generate following information as output:

-	Predicated LTM candidate cells

-	Predicted LTM configuration for each candidate cells

-	Predicted TA value

-	Predicted activated TCI state

-	Predicated LTM target cell.

#### 4.2.2.4	Feedback of AI/ML based intra-CU LTM

To optimize the performance of AI/ML based intra-CU LTM model, following feedback can be considered to be collected:

-	Legacy UE performance feedback

-	Beam level UE performance feedback

-	SON reports (e.g., RLF, CEF, RA)

#### 4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-18 and Rel-19:

Potential F1 interface impact:

-	Enhanced existing procedure or new procedure to exchange information between gNB-CU and gNB-DU:

-	LTM prediction

-	UE L1 measurements

<End of change>