3GPP TSG-RAN WG3 Meeting #129 bis	R3-256796

Prague, Czech Republic, 13-17 October 2025

Agenda Item:		10.3.2

Source:		NEC

Title:			Considerations on RAN-CN interface options

Document for:		Discussion and decision

# Introduction

In the study scope of the new SI on 6G Radio [1], the following items have RAN3 involved:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide our thoughts on the RAN-CN interface options.

# Discussion

2.1	P2P vs SBI

Service-Based Interface (SBI) is introduced as a fundamental architectural component of the 5GC, which replaces traditional dedicated point-to-point (P2P) interfaces and serves as a logical service bus for Network Functions (NFs) using standardized APIs. This enables 5GC transfer to a cloud-native, microservices-based design which makes the network more flexible, scalable, and efficient. Figure 1 below depicts the 5G System Architecture with service-based interfaces in the Control Plane.

Figure 1: 5G System Architecture with SBI

Regarding the RAN-CN interface design for 6G, there are three options:

- Option 1: Traditional P2P interface (NG-like)

- Option 2: Fully SBI (API based)

- Option 3: Partial P2P interface and partial SBI (between new NFs)

In 6G, besides the services already supported in 5G, new services, including AI, sensing, immersive communication, NTN, are emergent, which will become new revenue streams for operators. As outlined in the 6G TR38.914, “The 6G RAN architecture shall support enhanced service awareness in RAN.” For RAN domain, in order to facilitate the quick deployment of these new services, SBI adopted in RAN will be a better choice, by which one RAN NF can directly consume other RAN NFs or CN NFs.

For example, for AI service, Nran interface which is an API based open interface can be used to support the exchange of signalling and data among RAN NFs or between RAN NFs and CN NFs. Nran interface can consist of multiple services, e.g. Nran_AI/ML/Data reporting or Nran_AI/ML/Model transfer, where a RAN NF acts as producer and other RAN/CN NFs as consumers. This will largely simplify the procedure and ease data accessibility and reusability.

However, considering the high standard effort for a groundbreaking design and the requirement of smooth interoperability with 5G, we suggest to adopt a two-step strategy: first to support Option 3, which can provide 6G new services in a more flexible and scalable way, and meanwhile keep traditional communication functions unchanged as legacy; and then to move toward Option 2 in a later phase, which realized a true cloud implementation of RAN.

Proposal 1: Adopt partial P2P interface (for traditional communication functions) and partial SBI (for new NFs) scheme for 6G RAN-CN interface.

2.2	STCP vs QUIC

SCTP has been extensively and long-term used as a transport layer in 4G and 5G RAN interfaces, e.g., 4G S1-C, 4G X2-C, 5G NG-C, 5G Xn-C, and 5G F1-C, providing highly stable and reliable transportation of control signaling. At the same time, we can observe some drawbacks of SCTP in 5G systems. For instance, it lacks a built-in security mechanism; using IPSec for SCTP to compensate for this introduces overhead cost and reduces transmission efficiency. Furthermore, SCTP has a relatively high connection establishment latency due to its multi-way handshake. Additionally, the complex upgrades and migrations for the SCTP supporting system increase the maintenance cost. Furthermore, even though SCTP can handle multiple streams of data at once, a delay in one stream can still block the others which then cause Head-of-line blocking, so the congestion control is also not perfectly resolved.

Observation 1: SCTP transport layer provides stable and reliable transportation of control signalling for e.g. RAN-CN interface in the 4G/5G system, but it also has some drawbacks in terms of security, connection establishment latency, migration, and congestion control.

In the 6G, the new requirements are proposed, like cloud-native deployment, service-based architecture proposed in the RANP meetings. Therefore, it is a chance to re-evaluate whether the SCTP as a transport layer protocol is suitable for 6G requirements.

Observation 2: The 6G raises new requirements and brings new challenges to the SCTP transport layer.

The new transport layer protocol QUIC (Quick UDP Internet Connections) has several attractive features for the 6G RAN-CN interface. It provides built-in security by default, and 0-RTT/1-RTT handshake directly reduces connection setup time. And QUIC has a better solution for congestion control because it supports multiple streams, and packet loss in one stream does not block others. Its use of a connection ID (instead of IP/port) enables better connection migration and software/hardware upgrading which is friendly to could-native deployment. Also, being implemented over UDP makes QUIC is implemented over UDP, which makes more flexible in data transportation and ideal for cloud RAN deployment.

Observation 3: Compared to SCTP, QUIC has several attractive features for 6G RAN-CN interface in terms of security, latency, congestion control, migration, and transportation flexibility.

However, it introduces a backward compatibility issue, since 6G nodes with QUIC are unable to communicate directly with existing 5G nodes with SCTP (e.g., 5G RAN to 6G CN, 6G RAN to 5G CN), potentially increasing the complexity of interworking transportation. Additionally, QUIC has introduced in CN but it is still a new technology for the RAN, which would have more workload in RAN3 compared to SCTP. Furthermore, SCTP has successful deployment in the current 5G/4G network system, while the practical performance of QUIC has not been verified in RAN.

Observation 4: Introducing QUIC in the 6G RAN-CN interface will have backward compatibility issues, more workload in RAN3, and unverified performance.

Proposal 2: RAN3 is suggested to study whether to keep using SCTP or to adopt the new QUIC technology.

# Proposals

In this contribution, we provide our views on the 6G RAN-CN interface options and made the following observations and proposals.

Proposal 1: Adopt partial P2P interface (for traditional communication functions) and partial SBI (for new NFs) scheme for 6G RAN-CN interface.

Observation 1: SCTP transport layer provides stable data transportation for RAN-CN interface in the 4G/5G system, but it also has some drawbacks in terms of security, throughput, latency, migration, and congestion control.

Observation 2: The 6G raises new requirements and brings new challenges to the SCTP transport layer.

Observation 3: Compared to SCTP, QUIC has several attractive features for 6G RAN-CN interface in terms of security, latency, congestion control, migration, and transportation flexibility.

Observation 4: Introducing QUIC in the 6G RAN-CN interface will have backward compatibility issues, more workload in RAN3, and unverified performance.

Proposal 2: RAN3 is suggested to study whether to keep using SCTP or to adopt the new QUIC technology.

# Reference

[1] RP-252912, New SI: Study on 6G Radio