3GPP TSG-RAN WG3 Meeting #129 bis	R3-256797

Prague, Czech Republic, 13-17 October 2025

Agenda Item:		10.5.1

Source:		NEC

Title:			AI/ML use cases for 6G

Document for:		Discussion and decision

# Introduction

In the study scope of the new SI on 6G Radio [1], the following items on AI/ML have RAN3 involved:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we provide our thoughts on the top AI/ML use cases for 6G, mainly from RAN3 point of view.

# Discussion

2.1	5G use cases

AI/ML is an advanced tool to help enhance network performance and maintain operation cost-effectiveness. For AI/ML for NG-RAN topic, in Rel-18 and Rel-19, RAN3 covered the following use cases for both non-split and split architecture:

- Load Balancing

- Energy Saving

- Mobility Optimization

- Network slicing

- Capacity and Coverage Optimization

In the newly kicked off Rel-20 SI, AI/ML-based mobility use case, including multi-hop UE trajectory across gNBs, intra-CU LTM, handover enhancements, e.g. inter-CU LTM, is included.

All the above-mentioned use cases are already justified to be beneficial for addressing the increasing complexity and dynamic behaviour of networks.

Considering 6GS should at least provide same performance as 5GS, we think 6GS also needs to support all the above use cases that are already in 5GS. Therefore, in the study of 6G AI/ML in RAN3, we suggest to first consider these 5GS already supported use cases, starting with all pre-Rel20 use cases. For the Rel-20 SI use case, we can decide whether also to include in the 2nd phase.

Proposal 1: In 6G AI/ML study, all pre-Rel20 use cases, as listed below, should be considered.

- Load Balancing

- Energy Saving

- Mobility Optimization

- Network slicing

- Capacity and Coverage Optimization

2.2	New use cases for 6G

6G will expand on top of the existing infrastructure of 5G, and it will be designed to surpass 5G by providing faster speeds, more efficient communication and wider network coverage than 5G. Additionally, 6G is expected to unlock a variety of innovative use cases that aim to leverage advancements in AI, sensing, NTN, immersive communication.

As new and promising future services become possible in 6G, attempts to explore more AI/ML use cases, which can provide new paradigms and opportunities across multiple fields, become necessary. Below are our preferred new use cases for 6G AI/ML.

2.2.1	AI/ML assisted QoE

QoE is defined as a mechanism targeting at collecting application layer measurements from UEs, and in 5G, QoE metrics includes average throughput, initial playout delay, buffer level. Considering QoE mechanism is a key feature for service quality evaluation, it should still be supported in 6G.

In order to optimize resource allocation per UE to improve user experience, QoE prediction by AI/ML model can be used in RAN for better resource reallocation. The study can include e.g. QoE measurement collection for AI/ML model training and inference, future QoE prediction for specific UE(s), QoE prediction exchange between RAN nodes for better handover decision and whole network optimization, actual QoE measurements feedback for model performance monitoring.

We can start with the QoE metrics that are already visible to RAN and focus on RVQoE prediction first. Further, as agreed in TR 38.914 “The 6G RAN architecture shall support enhanced service awareness in RAN”, it is also beneficial to investigate this use case taking into account additional QoE parameters that can be visible to RAN.

2.2.2	Cross-domain collaboration for AI/ML

In 5G, RAN and SA conducted AI/ML studies separately, which lead to end-to-end network performance optimization which involves different network domains cannot be guaranteed. In order to enhance overall network performance and user experience for 6G, the valuable data from 6GC can be used as input data or feedback data to AI/ML model in RAN to enable better actions.

In 6G study, we can study the information exchange between RAN and CN and between RAN and OAM for AI/ML based features. For example, for AI/ML based network slicing, predicted slice level resource status can be exchanged, besides, since OAM manages the life cycle of the end-to-end slice across the network domains and is in charge of slice configuration, slice resource allocation and slice performance management, if RAN feedback on SLA requirements fulfilment, e.g. PDCP throughput and PRB usage in RAN domain, can be provided to OAM, then SLA can be assured by timely and dynamic slice configuration adjustment.

In summary, cross-domain collaboration can greatly increase the performance of AI/ML based features and therefore we suggest to include this use case in the 6G AI/ML study.

2.2.3	Agentic AI RAN

AI agents are small, specialized pieces of software that can make decisions and operate cooperatively or independently to achieve system objectives. Agentic AI refers to AI systems composed of AI agents that can behave and interact autonomously in order to achieve their objectives.

In traditional RAN, the configuration is set in advance by OAM. Agentic AI RAN with which AI agents reside, on the other hand, are given a set of configurations by local AI agent and will then autonomously select which of those are best suited to achieve specific performance goals. This powerful agentic RAN with capability of autonomous configuration finding and dynamic environmental adaptation can achieve network maintenance complexity reduction and meanwhile network performance increase. In the 6G study, we suggest to study how to support interaction and knowledge transfer among AI agents.

Proposal 2: In 6G AI/ML study, the following new use cases can be considered:

- AI/ML assisted QoE

- Cross-domain collaboration for AI/ML

- Agentic AI RAN

# Proposals

In this contribution, we provide our views on the top AI/ML use cases that need to be included in 6G study and made the following proposals.

Proposal 1: In 6G AI/ML study, all pre-Rel20 use cases, as listed below, should be considered.

- Load Balancing

- Energy Saving

- Mobility Optimization

- Network slicing

- Capacity and Coverage Optimization

Proposal 2: In 6G AI/ML study, the following new use cases can be considered:

- AI/ML assisted QoE

- Cross-domain collaboration for AI/ML

- Agentic AI

# Reference

[1] RP-252912, New SI: Study on 6G Radio