3GPP TSG-RAN WG3 Meeting #129bis	R3-256833

Prague, Czech Republic, Oct 13th –17th, 2025

Agenda Item:	12.2.1

Source:	CMCC

Title:	(TP to TR 38.745) Discussion on AI/ML based Multi-hop UE trajectory across gNBs

WID/SID: 	FS_NR_AIML_NGRAN_Ph3

Document for:	Discussion

# 1 Introduction

In RAN#108, the new SID on AI/ML for NG-RAN in Rel-20 was approved [1] with the following objectives.

4	Objective

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

-   multi-hop UE trajectory across gNBs;

-   intra-CU LTM;

-   handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In this contribution, we would provide our initial considerations on AI/ML based multi-hop UE trajectory across gNBs and some proposals.

# 2 Discussion

## 2.1 General

In the discussion on UE Trajectory prediction in Rel-18 AI/ML for NG-RAN, the complete mechanism of UE Trajectory prediction/collection for one hop target NG-RAN node was defined, including the signaling procedures of UE Trajectory prediction and measured UE Trajectory collection, the information included in predicted UE Trajectory and measured UE Trajectory and the Configuration of UE Trajectory Collection, etc. For AI/ML based multi-hop UE trajectory across gNBs, we prefer to reuse the existing procedures and messages in Rel-18 as much as possible and only consider the necessary enhancements for Multi-hop UE trajectory across gNBs.

Observation 1: The complete mechanism of UE Trajectory prediction/collection for one hop target NG-RAN node was defined in Rel-18, including

- Signaling procedures of UE Trajectory prediction and measured UE Trajectory collection

- Information included in predicted UE Trajectory and measured UE Trajectory

- Configuration of UE Trajectory Collection

Proposal 1: For the normative work for Multi-hop UE trajectory across gNBs, it is proposed to reuse the existing procedures and messages of for one hop target node as much as possible and only consider the necessary enhancements in Rel-20.

From our point of view, how to convey the predicted information from the source node to the multiple target nodes and how to execute the measured information collection are the key issues for Multi-hop UE trajectory across gNBs. They may bring a series of enhancement requirements.

Observation 2: How to transmit the predicted information from the source node to the multiple target nodes and how to execute the measured information collection are the key issues for Multi-hop UE trajectory across gNBs. They may bring a series of enhancement requirements.

Proposal 2: RAN3 to consider the following issues as the start point for Multi-hop UE trajectory across gNBs:

- The prediction information transmission from the source node to the target nodes;

- The measured information collection and transmission from the target nodes to the source node.

## 2.2 Prediction information transmission

Regarding to the prediction transmission for Multi-hop UE trajectory across gNBs, the AI/ML model in source node has the capability to generate the multi-hop UE trajectory prediction which may include a list of cells related to multiple target nodes. That means the prediction information should be transmitted from the source node to the related target nodes in time. We think there may be two options:

- Option 1 (Hop-by-hop): The source node transmits the multi-hop prediction information to the first hop Target node (e.g., Target node-1 in Figure 1). And then Target node-1 convey the prediction information to its next hop Target node (e.g., Target node-2) when Target node-1 make sure UE will handover to Target node-2. And then Target node-2 send the related prediction information to its next hop Target node (e.g., Target node-3) when Target node-2 make sure UE will handover to Target node-3. That means the prediction information will be transmitted from one node to the next hop node in order.

Figure 1: UE trajectory prediction transmission-Hop-by-hop

- Option 2 (Point-to-multi-point): When the source node generates the multi-hop UE trajectory prediction, it transmits the prediction information to every potential target node directly (Option 2 is illustrated in Figure 2).

Figure 2: UE trajectory prediction transmission-Point-to-multi-point

Compare the above two options, we think Option 1 (Hop-by-hop) has more advantages. For example, for Hop-by-hop, if Target node-1 does not handover the UE to Target node-2, the prediction information will not be sent to Target node-2 to avoid the unnecessary signaling overhead for both Target node-1 and Target node-2. And since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18, the existing message (i.e., HANDOVER REQUEST message) could be reused to convey the prediction information. Moreover, if Target node-1 or Target node-2 also have the deployment of AI/ML model for UE Trajectory prediction, it could choose to either forward the prediction from the source node to its next hop Target node or generate a new prediction information and send to its next hop Target node to ensure the performance of UE Trajectory prediction.

Observation 3: Option 1 (Hop-by-hop) of multi-hop UE trajectory prediction transmission has the following advantages, e.g.,

- To avoid the unnecessary signaling overhead

- The existing message (i.e., HANDOVER REQUEST message) could be reused to convey the prediction information, since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18

- To enable the update or replacement of the prediction information from the source node and the lasted prediction information could be sent to the next hop target node to ensure the performance of UE Trajectory prediction

But for Option 2 (Point-to-multi-point), on one hand, it may cause the large signaling overhead and wasting of resources since the potential target nodes could get the prediction information from the source node to reserve resources for the coming UE but the UE may not be handed over to the nodes. On the other hand, the prediction forwarding may be limited by Xn interface between the source node and the target nodes. This may cause some target nodes may not get the prediction or make the forwarding procedure complex. Furthermore, the UE Trajectory Prediction cannot be sent in HANDOVER REQUEST message since the prediction will be sent to the nodes before UE is handed over. Therefore, a new procedure or message may be needed.

Observation 4: Option 2 (Point-to-multi-point) of UE trajectory prediction transmission may cause the large signaling overhead and wasting of resources. The prediction forwarding may be limited by Xn interface between the source node and target nodes which will make the procedure complex. And a new procedure or message for UE trajectory prediction transmission may be needed.

Based on the above analysis, we prefer to use Option 1 (Hop-by-hop) as the baseline of the UE trajectory prediction transmission from the source node to the target node for Multi-hop UE trajectory across gNBs.

Proposal 3: The solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order could be considered as the baseline for the prediction transmission for Multi-hop UE trajectory across gNBs.

If Proposal 3 is agreed, it may have been a long time from the generation of the predicted information of multi-hop UE trajectory at the source node to its acquisition by some target nodes in the UE trajectory. Generally, it is commonly accepted that the prediction accuracy could decrease as the prediction time increases. After a long time of forwarding, the predicted information may become outdated or invalid for the target nodes. Therefore, we propose that the time when the UE Trajectory prediction is generated, or the period of validity could be considered as the additional information to be provided together with the UE Trajectory prediction to avoid the confusion of HO decision or resource reservation caused by outdated/invalid prediction information. Based on the additional information, the target nodes could determine whether to use the predicted information or continue forwarding it to its next-hop node.

Observation 5: For Multi-hop UE trajectory scenario, some additional information (e.g., the generation time of the UE trajectory prediction or its period of validity) may be needed for the target nodes to determine whether to use the predicted information or continue forwarding it to its next-hop node.

Proposal 4: The prediction information of multi-hop UE trajectory across gNBs is necessary for AI/ML based multi-hop UE trajectory, which could be used to assist the target nodes for HO decision and resource reservation. The following information could be included in the prediction information:

- List of cells where the UE is predicted to connect (mandatory in Rel-18 UE Trajectory prediction)

- Predicted Time UE Stays in Cell (optional in Rel-18 UE Trajectory prediction)

- Additional information, e.g., the generation time of the UE trajectory prediction or its period of validity

## 2.3 Measured information collection and transmission

Similar with one hop target NG-RAN node scenario in Rel-18, we think the feedback (e.g., Measured UE trajectory and Measured UE performance) from the target nodes to the source node is also needed for AI/ML model monitoring and retraining.

Proposal 5: For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is also needed for AI/ML model monitoring and retraining. The following information could be considered as the starting point:

- Measured UE trajectory

- Measured UE performance

For the measured information transmission, there are also two main options:

- Option 1 (Hop-by-hop): The measured information is sent from one node to its previous hop node. As illustrated in Figure 1, Target node-3 sends measured information to Target node-2; Target node-2 sends the measured information to Target node-1. And then Target node-1 sends the information measured by itself and the measured information from Target node-2 and Target node-3 to source node.

- Option 2 (Point-to-multi-point): Target nodes report the measured information to the source node directly.

In Option 1 (Hop-by-hop), the intermediate hop nodes (e.g., Target node-1 or Target node-2 in Figure1) needs to collect the measured information in the cells where the UE connect within Target node and receive the measured information that is from its next hop node; and then send the information to its previous hop node. The intermediate hop node needs to keep UE context after handover execution for the duration of the measured information collection from its next hop node. It brings the implementation complexity for the intermediate hop nodes.

Observation 6: For Hop-by-hop, the intermediate hop nodes need to keep UE context after handover execution for the duration of the measured information collection from its next hop node. It brings the implementation complexity for the intermediate hop nodes.

In Option 2 (Point-to-multi-point), the feedback of measured information is limited by Xn interface. If there is no Xn interface between the source node and some target nodes, the measured information related to the target nodes cannot be sent to the source node directly, that may result in the missing of partial measured information or great impact on specs to introduce new feedback procedure.

Observation 7: For Point-to-multi-point, the feedback of measured information is limited by Xn interface between the source node and the target nodes. It may result in the missing of partial measured information or great impact on specs.

Based on the above analysis, we think the above two options have their own application scenarios. For example, in the case where there is Xn interface between the source node and target nodes, Point-to-multi-point (Option 2) solution could be used, that means the target nodes could send the measured information to the source node directly to avoid the related intermediate hop nodes keep UE context for a long time after handover execution. Otherwise, in the case where there is no Xn interface between the source node and target nodes, Hop-by-hop (Option 1) solution could be used. With the help of the intermediate hop nodes, the measured information could be reported to the source node to avoid the measured information missing.

Observation 8: The above two options could be used in different scenarios.

From the above analysis, we also observe that the information of Xn interface between the source node and target nodes is useful for target nodes to choose the appropriate feedback path. For example, when there is no Xn interface between a target node and the source node, with the help of the info of Xn interface the target node could find one intermediate hop node in the UE trajectory, which has the Xn interface with the target node and with the source node, respectively.

To be specific, the information of Xn interface could be provided by source node or target nodes. For example, when the source node generates the UE Trajectory Prediction, it could also provide the information of Xn interface between the source node and the related target nodes in the UE Trajectory Prediction. In another case where the information of Xn interface provided by target nodes, when UE is handed over to a target node, the node could add its information of Xn interface and provide information to its next hop node when it forwards the UE Trajectory Prediction to its next hop node. Therefore, the information of Xn interface between source node and target nodes could be considered as a part of the UE Trajectory Prediction and provided by source node or target nodes.

Observation 9: The information of Xn interface between source node and target nodes is helpful for target nodes to choose the appropriate feedback path to the source node for the measured information.

Proposal 6: RAN3 considers to introduce the information of Xn interface between the source node and target nodes in the UE Trajectory Prediction for Multi-hop UE trajectory scenario.

Proposal 7: The information of Xn interface between the source node and target nodes could be provided by source node or target nodes.

## 2.4 Signaling procedures

As we mentioned above, the signaling procedures of UE Trajectory prediction and measured UE Trajectory collection was defined in Rel-18. We propose to reuse the existing signaling procedures and messages for AI/ML based Multi-hop UE trajectory scenario, i.e.,

- The DATA COLLECTION REQUEST message is used to convey the related configuration of the measured information collection from the source node to the potential target nodes, including the measurement object, the trigger conditions of measured information collection, etc.

- The HANDOVER REQUEST message is used to convey the predicted Multi-hop UE trajectory and it could span across multiple NG-RAN nodes, for example, from the source node to the next hop target node or from one target node to its next hop target node;

- The DATA COLLECTION UPDATE message is used to report the measured information and it could span across multiple NG-RAN nodes, for example, from the target node to the source node or from one target node to its previous hop node.

Proposal 8: The existing signaling procedures and messages defined for UE Trajectory prediction/collection for one hop target NG-RAN node in Rel-18 could be reused for AI/ML based Multi-hop UE trajectory, i.e.,

- The DATA COLLECTION REQUEST message is used to convey the related configuration of the measured information collection from the source node to the potential target nodes, including the measurement object, the trigger conditions of measured information collection, etc.

- The HANDOVER REQUEST message is used to convey the predicted Multi-hop UE trajectory and it could span across multiple NG-RAN nodes, for example, from the source node to the next hop target node or from one target node to its next hop target node;

- The DATA COLLECTION UPDATE message is used to report the measured information and it could span across multiple NG-RAN nodes, for example, from the target node to the source node or from one target node to its previous hop node.

# 3 Conclusion

In this paper, we discuss on the solutions for AI/ML based multi-hop UE trajectory across gNBs, we have the following observations and proposals:

Observation 1: The complete mechanism of UE Trajectory prediction/collection for one hop target NG-RAN node was defined in Rel-18, including

- Signaling procedures of UE Trajectory prediction and measured UE Trajectory collection

- Information included in predicted UE Trajectory and measured UE Trajectory

- Configuration of UE Trajectory Collection

Proposal 1: For the normative work for Multi-hop UE trajectory across gNBs, it is proposed to reuse the existing procedures and messages of for one hop target node as much as possible and only consider the necessary enhancements in Rel-20.

Observation 2: How to transmit the predicted information from the source node to the multiple target nodes and how to execute the measured information collection are the key issues for Multi-hop UE trajectory across gNBs. They may bring a series of enhancement requirements.

Proposal 2: RAN3 to consider the following issues as the start point for Multi-hop UE trajectory across gNBs:

- The prediction information transmission from the source node to the target nodes;

- The measured information collection and transmission from the target nodes to the source node.

For Prediction information transmission

Observation 3: Option 1 (Hop-by-hop) of multi-hop UE trajectory prediction transmission has the following advantages, e.g.,

- To avoid the unnecessary signaling overhead

- The existing message (i.e., HANDOVER REQUEST message) could be reused to convey the prediction information, since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18

- To enable the update or replacement of the prediction information from the source node and the lasted prediction information could be sent to the next hop target node to ensure the performance of UE Trajectory prediction

Observation 4: Option 2 (Point-to-multi-point) of UE trajectory prediction transmission may cause the large signaling overhead and wasting of resources. The prediction forwarding may be limited by Xn interface between the source node and target nodes which will make the procedure complex. And a new procedure or message for UE trajectory prediction transmission may be needed.

Proposal 3: The solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order could be considered as the baseline for the prediction transmission for Multi-hop UE trajectory across gNBs.

Observation 5: For Multi-hop UE trajectory scenario, some additional information (e.g., the generation time of the UE trajectory prediction or its period of validity) may be needed for the target nodes to determine whether to use the predicted information or continue forwarding it to its next-hop node.

Proposal 4: The prediction information of multi-hop UE trajectory across gNBs is necessary for AI/ML based multi-hop UE trajectory, which could be used to assist the target nodes for HO decision and resource reservation. The following information could be included in the prediction information:

- List of cells where the UE is predicted to connect (mandatory in Rel-18 UE Trajectory prediction)

- Predicted Time UE Stays in Cell (optional in Rel-18 UE Trajectory prediction)

- Additional information, e.g., the generation time of the UE trajectory prediction or its period of validity

For Measured information collection and transmission

Proposal 5: For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is also needed for AI/ML model monitoring and retraining. The following information could be considered as the starting point:

- Measured UE trajectory

- Measured UE performance

Observation 6: For Hop-by-hop, the intermediate hop nodes need to keep UE context after handover execution for the duration of the measured information collection from its next hop node. It brings the implementation complexity for the intermediate hop nodes.

Observation 7: For Point-to-multi-point, the feedback of measured information is limited by Xn interface between the source node and the target nodes. It may result in the missing of partial measured information or great impact on specs.

Observation 8: The above two options could be used in different scenarios.

Observation 9: The information of Xn interface between source node and target nodes is helpful for target nodes to choose the appropriate feedback path to the source node for the measured information.

Proposal 6: RAN3 considers to introduce the information of Xn interface between the source node and target nodes in the UE Trajectory Prediction for Multi-hop UE trajectory scenario.

Proposal 7: The information of Xn interface between the source node and target nodes could be provided by source node or target nodes.

For Signaling procedures of AI/ML based Multi-hop UE trajectory

Proposal 8: The existing signaling procedures and messages defined for UE Trajectory prediction/collection for one hop target NG-RAN node in Rel-18 could be reused for AI/ML based Multi-hop UE trajectory, i.e.,

- The DATA COLLECTION REQUEST message is used to convey the related configuration of the measured information collection from the source node to the potential target nodes, including the measurement object, the trigger conditions of measured information collection, etc.

- The HANDOVER REQUEST message is used to convey the predicted Multi-hop UE trajectory and it could span across multiple NG-RAN nodes, for example, from the source node to the next hop target node or from one target node to its next hop target node;

- The DATA COLLECTION UPDATE message is used to report the measured information and it could span across multiple NG-RAN nodes, for example, from the target node to the source node or from one target node to its previous hop node.

# Reference

- RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3.

# Annex: Text Proposal for TR 38.745

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

For Multi-hop UE trajectory across gNBs, the AI/ML model developed in the source node has the capability to generate the multi-hop UE trajectory prediction which may include a list of cells related to multiple target nodes. And the prediction information should be transmitted from the source node to the related target nodes. For Multi-hop UE trajectory across gNBs, the following issues should be considered:

- The prediction information transmission from the source node to the target nodes;

- The measured information collection and transmission from the target nodes to the source node.

### 4.1.2	Solutions and standard impacts

The prediction information of multi-hop UE trajectory across gNBs is necessary for AI/ML based multi-hop UE trajectory, which could be used to assist the target nodes for HO decision and resource reservation.

The prediction information may include:

- List of cells where the UE is predicted to connect

- Predicted Time UE Stays in Cell

- Additional information, e.g., the generation time of the UE trajectory prediction or its period of validity

For the prediction transmission for Multi-hop UE trajectory across gNBs, the solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order is taken as the baseline.

For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is needed for AI/ML model monitoring and retraining.

The measured information at the target node may include:

- Measured UE trajectory

- Measured UE performance

The information of Xn interface between source node and target nodes may be needed for target nodes to choose the appropriate feedback path to the source node for the measured information.

The existing signaling procedures and messages defined for UE Trajectory prediction/collection for one hop target NG-RAN node in Rel-18 could be reused for AI/ML based Multi-hop UE trajectory.