3GPP TSG-RAN WG3 Meeting #129bis														R3-256834

Prague, CZ, 13th-17th October, 2025

Agenda item:		12.2.3

Source:				CMCC

Title:					(TP to TR 38.745) Discussion on AI/ML based handover enhancement

Document for:		Discussion

# Introduction

During RAN#109, a Release 20 new study item “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” was approved [1] with the following objectives:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In this paper, we provide our consideration on the AI/ML based handover enhancement, especially for inter-CU LTM.

# Discussion

## General

L1/L2-triggered mobility (LTM) is introduced to reduce the latency of the mobility procedure. When apply LTM mechanism, gNB provides the LTM candidate configurations to UE through RRC signalling and activate TCI states for one or multiple LTM candidate cells in advance to allow UE perform DL synchronized with those cells. Then, the gNB sends the cell switch command via MAC CE based on the L1 measurement report(s) from UE to trigger the UE switches to the target LTM candidate cell. Moreover, to further save the cell switch time, the early TA acquisition can be triggered before cell switch. The TA value can be provided by the network or realized through UE-based TA measurement.

For inter-CU LTM discussed in R19, inter-CU LTM in NR standalone scenario is prioritized and inter-CU LTM in NR-DC scenarios are supported as secondary priority. When it comes to the AI/ML assisted inter-CU LTM in R20, considering the limited time, it’s proposed that the inter-CU LTM in NR standalone scenario is also prioritized and the inter-CU LTM in NR-DC scenarios can be discussed with secondary priority if time allows.

Proposal 1: AI/ML assisted inter-CU LTM in NR standalone scenario is prioritized. AI/ML assisted inter-CU LTM in NR-DC scenarios can be discussed with secondary priority if time allows.

According to the R20 SID, new AI/ML-based mobility related use case will be studied, including multi-hop UE trajectory across gNBs, intra-CU LTM and other handover enhancement (especially for inter-CU LTM). From our perspective, AI/ML based intra-CU LTM may mainly focus on F1 interface enhancement for CU-DU split architecture. And the AI/ML based handover enhancement, especially for inter-CU LTM, can focus on Xn interface enhancement first, which means that non-split architecture should be discussed first. For the solution for split architecture, such as F1/E1 enhancement, it can be postponed after AI/ML based intra-CU LTM has a relative clear solution, and to see whether it can be reused.

Observation 1: The solutions for AI/ML based intra-CU LTM may mainly focus on F1 interface enhancement in CU-DU split architecture. Then, for AI/ML based handover enhancement (e.g. inter-CU LTM), Xn interface can be enhanced first in non-split architecture, and check whether the split architecture can reuse the solution of AI/ML based intra-CU LTM.

Proposal 2: For AI/ML based inter-CU LTM, prioritize non-split architecture solution first. The solution for split architecture proposes to be postponed to see whether the solution of AI/ML based intra-CU LTM can be reused.

Figure 9.2.3.5.2-2. Signalling procedure for inter-gNB LTM

In R19 mobility WI, the overall procedure for inter-CU LTM is shown in Figure 9.2.3.5.2-2 in TS 38.300.

From the signalling procedure, inter-CU LTM including 4 phases: LTM preparation (step 0-9), early sync (step 10-11), LTM cell switch (step 12-18) and LTM cell switch completion (step 19-22) phase. The following lists the potential enhancements by utilizing AI/ML.

- LTM preparation phase: In this phase, the LTM configurations are exchanged between gNBs. The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-CU LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. The candidate prepares and provides the LTM configuration(s) to the source gNB. During this procedure, the candidate cells prediction/determination can be optimized using AI/ML.

- Early sync phase: In this phase, the network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. Depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch. For RACH-less LTM, invalid TA value is possible to cause LTM failure. By introducing AI/ML function, valid TA value can be predicted, which can improve the robustness of LTM handover and ensure user experience.

- LTM cell switch phase: In this phase, LTM cell switch is triggered by cell switch command. From our understanding, the AI/ML function can be used to generate L1/L3 measurement prediction or LTM cell switch time prediction, which can assist the network making LTM cell switch decision and reserve resources in advance to improve the handover accuracy and further reduce handover latency.

- LTM cell switch completion phase: In this phase, if AI/ML is utilized for handover enhancement, the feedback information, such as measured information and UE performance after handover, should be collected to evaluate prediction accuracy, ensure the user experience and update the AI/ML model.

Based on the above analysis, potential enhancements by introducing AI/ML may include: candidate cell prediction/ determination, TA prediction, L1/L3 measurement prediction or LTM cell switch time prediction, and feedback information collection.

Proposal 3: RAN3 discusses the following potential enhancements for AI/ML based inter-CU LTM:

- Candidate cell prediction/determination

- TA prediction

- L1/L3 measurement prediction or LTM cell switch time prediction

- Feedback information collection

## Solutions for AI/ML based inter-CU LTM

The AI/ML based mobility optimization for non-split architecture has been discussed in R18 and R19 for traditional handover, and the solutions are captured in TR 37.817 and TR 38.743. Based on the solutions discussed in the previous releases, it’s proposed to take the previous solutions and procedures as baseline, and discusses the input/output/feedback for inter-CU LTM.

Proposal 4: The existing solutions and procedures defined for R18 and R19 AI/ML based mobility optimizations could be taken as baseline.

### Locations for AI/ML Model Training and Model Inference

In R18 & R19 AI/ML enabled use cases, the locations for AI/ML Model Training and Inference for non-split architecture are captured as following:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For CU-DU split architecture, the following locations are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

In R20, the existing deployments of AI/ML function can be applied to AI/ML based inter-CU LTM.

Proposal 5: The following deployments of AI/ML function can be considered for AI/ML based inter-CU LTM:

For non-split architecture:

    - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB

    - AI/ML Model Training and AI/ML Model Inference are both located in the gNB

    - For split architecture:

    - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU

    - AI/ML Model Training and Model Inference are both located in the gNB-CU

### Required data and standard impacts

To enable the AI/ML based inter-CU LTM, in addition to the existing AI/ML related information exchanged in the Xn interface, some LTM specific information also needs to be taken into account. For LTM, L1/L3 measurements, TA value for specific candidate cell(s) and TCI state information proposes to be discussed as input data. With the existing information discussed in previous release, the potential input information for AI/ML based inter-CU LTM can be concluded as:

Existing information:

- Current and predicted resource status

- UE trajectory prediction

- Current and predicted UE traffic

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report

Specific for inter-CU LTM:

- L1/L3 measurements

- TA value for specific candidate cell(s)

- TCI state information per candidate cell(s)

- UE capability of UE based TA measurement

Observation 2: The existing AI/ML related input information can be reused for AI/ML based inter-CU LTM, and some LTM specific information need to be considered, including L1/L3 measurements, TA values, TCI state information, etc.

Based on the input information, the following output information may include for LTM configuration optimization or handover enhancement:

- Predicted TA value for inter-CU LTM

- Predicted target candidate cell(s) for UE mobility

- Predicted UE(s) handover to the target candidate cell

- Predicted L1/L3 measurements

- UE trajectory prediction

- LTM cell switch time prediction or predicted cell switch command transmission time

Then, for feedback information, the following information could be discussed:

- System KPIs (e.g. throughput)

- UE performance feedback

- Actual UE trajectory

- Actual L1/L3 measurement results

Proposal 6: Capture the above required data for AI/ML based inter-CU LTM in TR 38.745.

Potential Xn interface impact:

- Enhance Xn interface to transfer the LTM related prediction information, such as predicted TA value

- Enhance Xn interface to transfer the feedback information, including UE performance, actual UE trajectory, etc

Proposal 7: Enhance the Xn interface to exchange the inter-CU LTM prediction and feedback information.

Based on the above discussion, we provide a TP for TR38.745 in Annex.

Proposal 8: Agree the TP in Annex for TR 38.745.

# Conclusion

In this contribution, the following observations and proposals on AI/ML based handover are provided:

Observation 1: The solutions for AI/ML based intra-CU LTM may mainly focus on F1 interface enhancement in CU-DU split architecture. Then, for AI/ML based handover enhancement (e.g. inter-CU LTM), Xn interface can be enhanced first in non-split architecture, and check whether the split architecture can reuse the solution of AI/ML based intra-CU LTM.

Observation 2: The existing AI/ML related input information can be reused for AI/ML based inter-CU LTM, and some LTM specific information need to be considered, including L1/L3 measurements, TA values, TCI state information, etc.

Proposal 1: AI/ML assisted inter-CU LTM in NR standalone scenario is prioritized. AI/ML assisted inter-CU LTM in NR-DC scenarios can be discussed with secondary priority if time allows.

Proposal 2: For AI/ML based inter-CU LTM, prioritize non-split architecture solution first. The solution for split architecture proposes to be postponed to see whether the solution of AI/ML based intra-CU LTM can be reused.

Proposal 3: RAN3 discusses the following potential enhancements for AI/ML based inter-CU LTM:

- Candidate cell prediction/determination

- TA prediction

- L1/L3 measurement prediction or LTM cell switch time prediction

- Feedback information collection

Proposal 4: The existing solutions and procedures defined for R18 and R19 AI/ML based mobility optimizations could be taken as baseline.

Proposal 5: The following deployments of AI/ML function can be considered for AI/ML based inter-CU LTM:

For non-split architecture:

    - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB

    - AI/ML Model Training and AI/ML Model Inference are both located in the gNB

    - For split architecture:

    - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU

    - AI/ML Model Training and Model Inference are both located in the gNB-CU

Proposal 6: Capture the above required data for AI/ML based inter-CU LTM in TR 38.745.

Proposal 7: Enhance the Xn interface to exchange the inter-CU LTM prediction and feedback information.

Proposal 8: Agree the TP in Annex for TR 38.745.

# References

[1] RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3”, ZTE, NEC

# Annex: TP for 38.745

## 4.3	Other handover enhancements

### 4.3.X AI/ML assisted inter-CU LTM

#### 4.3.X.1 Use case description

L1/L2-triggered mobility (LTM) is introduced to reduce the latency of the mobility procedure. When apply LTM mechanism, gNB provides the LTM candidate configurations to UE through RRC signalling and activate TCI states for one or multiple LTM candidate cells in advance to allow UE perform DL synchronized with those cells. Then, the gNB sends the cell switch command via MAC CE based on the L1 measurement report(s) from UE to trigger the UE switches to the target LTM candidate cell. Moreover, to further save the cell switch time, the early TA acquisition can be triggered before cell switch. The TA value can be provided by the network or realized through UE-based TA measurement.

Potential enhancements by introducing AI/ML may include: candidate cell prediction/determination, TA prediction, L1/L3 measurement prediction or LTM cell switch time prediction, and feedback information collection, which can assist the network to improve the robustness/accuracy of LTM handover and ensure user experience.

#### 4.3.X.2 Solutions and standard impacts

##### 4.3.X.2.1 Locations for AI/ML Model Training and Model Inference

For non-split architecture, the following deployment locations can be considered:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB

For split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU

- AI/ML Model Training and Model Inference are both located in the gNB-CU

##### 4.3.X.2.4 Input of AI/ML assisted inter-CU LTM

To enable the AI/ML assisted inter-CU LTM, the following information may need as input data:

From the local node:

- Current and predicted resource status

- L1/L3 measurements

- TA value for specific candidate cell(s)

- UE trajectory prediction

- Current and predicted UE traffic

- TCI state per candidate cell(s)

From the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report

- UE capability of UE based TA measurement

From neighbouring NG-RAN nodes:

- Current and predicted resource status

- UE history information

##### 4.3.X.2.5 Output of AI/ML assisted inter-CU LTM

The output data may include:

- Predicted TA value for inter-CU LTM

- Predicted target candidate cell(s) for UE mobility

- Predicted UE(s) handover to the target candidate cell

- Predicted L1/L3 measurements

- UE trajectory prediction

- LTM cell switch time prediction or predicted cell switch command transmission time

##### 4.3.X.2.6 Feedback of AI/ML assisted inter-CU LTM

The feedback data may include:

- System KPIs (e.g. throughput)

- UE performance feedback

- Actual UE trajectory

- Actual L1/L3 measurement results

##### 4.3.X.2.7 Standard impacts

Potential Xn interface impact:

- Enhance Xn interface to transfer the LTM related prediction information, such as predicted TA value

- Enhance Xn interface to transfer the feedback information, including UE performance, actual UE trajectory, etc.