3GPP TSG-RAN WG3 Meeting #129-bis	R3-256843

Prague, Czech, 13th – 17th Oct 2025

Title: 	Discussion on Introducing S1 Suspend and Resume

Source: 	CMCC

Agenda item: 	9.2.14

Document Type:	Discussion and Decision

# Introduction

In RAN3 129bis meeting, whether to introduce NG Suspend/ Resume during hard FLSO was discussed, and the following agreement was captured in chair note [1]:

Remove text from BL CRs related to suspend/resume short NG/S1 interruption, causing e.g. packet loss, that may be predictable or unpredictable, e.g. hard FLSO to be further discussed in Q4

And it is agreed to align with NR NTN on this issue for IoT NTN.

In this contribution, we will share our views on whether it is beneficial to introduce S1 Suspend/Resume for regenerative payload in IoT NTN.

# .2	Discussion

This discussion paper will focus on sharing the benefits of introducing S1 suspend/resume.

When the satellite moves into the edge area, hard link switch over happens dramatically, to connect the eNB from one NTN gateway to another.

In current procedure, it takes long time to set up new connection for hard FLSO. The eNB needs first to disconnect from old NTN GW, then adjust antenna to new NTN GW, setup transport layer, IPSec tunnel, SCTP, and at last S1 connections. And due to the unstable feeder link quality and different satellite implementation, the time of hard feeder link switch over can’t be predicated, and uncontrollable.

During this period of time, CN has no idea that the feeder link shut down until SCTP detect it, and CN will continue to send packets to eNB, trying paging but fail again and again.

So the current procedure leads to very bad service experience to the UE. And it will even bring too many useless signals over the feeder links.

And what makes it worse is that this hard FLSO will definitely happen when satellites enter into the edge geography area periodically.  All the UE served by this satellite will be impacted during hard FLSO. Which means we know a terrible issue happens in specific time and specific area, but we do nothing.

Observation 1: the current hard FLSO procedures for regenerative payload IoT NTN cause bad service experience and inefficient signals interworking over feeder links.

The satellite is clear about when the hard FLSO happens, it is a worthy solution if we can do some NG enhancement to avoid the outage of hard FLSO.

One simple solution is that before hard FLSO happens, the satellite notifies CN(MME) with a S1 suspend message, only one message for each eNB and MME pair, not per UE. And CN will stop to send packets to eNB until the hard FLSO completes.

And when the satellite comes back to the old AMF area, it can also use NG resume to quickly recover the RAN Configurations to MME, update the S1 interface and set up S1 connections efficiently.

Proposal 1: It is beneficial to introduce S1 Suspend/Resume procedures as an enhancement for regenerative payload for IoT NTN, to reduce S1 connection recover time.

Proposal 2: Discuss and agree the CRs [2] for stage 2 and [3] for stage 3 on support of S1 Suspend/Resume for IoT NTN.

This enhancement has no extra requirement for satellite’s storage, or any other resources, it is a very low-cost enhancement.

# 3	Summary

In this contribution, we provide additional input considerations on introduce NG Suspend/Resume during hard FLSO in for Regenerative Payload for NR NTN:

Observation 1: the current hard FLSO procedures for regenerative payload IoT NTN cause bad service experience and inefficient signals interworking over feeder links.

Proposal 1: It is beneficial to introduce S1 Suspend/Resume procedures as an enhancement for regenerative payload for IoT NTN, to reduce S1 connection recover time.

Proposal 2: Discuss and agree the CRs [2] for stage 2 and [3] for stage 3 on support of S1 Suspend/Resume for IoT NTN.

# 4	Reference

[1] RAN3_129_agenda_20250829_1620_EOM, chair note

[2] R3-256614, Suspend/resume S1 during hard FLSO, CR to 36.300

[3] R3-256732, Suspend/resume S1 during hard FLSO, CR to 36.413