3GPP TSG-RAN WG3 Meeting #129bis	             R3-256846

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.2.1

Source:	CMCC

Title:	Considerations on RAN data collection function for 6G RAN

Document for:	Discussion

# Introduction

A new study item was agreed at RAN#108 that aims to develop a one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study in TR38.914 [1].

One of the objectives of the study is:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

One of the items in the above objective is the study of the overall RAN side architecture to support of various services and functionalities (e.g. AI/ML, sensing, etc). Since this is the first meeting on 6G in RAN3, it is essential to identify and clarify the general principles of the overall RAN architecture. These principles should serve as guidance for future 6G research and be adaptable to diverse 6G service requirements. In this contribution, we will take data collection function as a start point, as it is considered as an indispensable RAN function and reflects our perspective as operators on the design of RAN functions and architectures in 6G.

# Discussion

### 2.1 Principles of 6G RAN Architecture

When considering the 6G RAN functions and architecture in the first day, as RAN and CN are holding different responsibilities in end-to-end network, the prerequisite for designing 6G RAN functions and architecture is to identify a clear and well-defined division of responsibilities between the RAN and CN. To avoid unnecessary inter-dependencies and redundancy of features across RAN and CN domains, the 6G RAN architecture shall allow for the RAN and the CN to evolve independently, and the introduction of a new RAN function must be carefully evaluated to determine whether a closed loop can be achieved within the RAN to realize the function as much as possible. While necessary supporting from the OAM or the CN, e.g., for security control or management, is not precluded, the realization of RAN functions shall remain within the RAN side.

Observation 1: The prerequisite for designing 6G RAN functions and architecture is to identify a clear and well-defined division of responsibilities between the radio access network and the core network.

Proposal 1: The 6G RAN architecture shall allow for the RAN and the CN to evolve independently.

Proposal 2: The introduction of a new RAN function must be carefully evaluated to determine whether a closed loop can be achieved within the RAN to realize the function as much as possible.

Regarding 6G, beyond the traditional communication service, a range of new services will be introduced. As mentioned in the 6G RAN SID, the radio access network architecture should be considered support of various services and functionalities (e.g. AI/ML, sensing, etc). Therefore, the emerging service paradigms in 6G are expected to be seamlessly integrated into the 6G RAN, enabling a flexible, scalable, and service awareness future radio access network. From the operator’s view, a unified RAN architecture is essential to accommodate the various services envisioned in 6G. Such comprehensive consideration in the initial phase of 6G will ensure long-term adaptability of the RAN architecture and prevent the need for repeated architectural adjustments.

Observation 2: The emerging service paradigms in 6G are expected to be seamlessly integrated into the 6G RAN, enabling a flexible, scalable, and service awareness future radio access network.

Proposal 3: A unified RAN architecture is essential to accommodate the various services envisioned in 6G.

In addition, to avoid impacting the legacy communication functions of base stations, new 6G service functions within the 6G RAN architecture shall be independent with traditional communication function. Once introducing the new service functions, RAN3 needs to evaluate whether they can be regarded as part of the default function of base station or should be integrated into a new function. After the function is final determined, the next step is to decide whether the introduction of authentication procedure or network management procedure is required for such new functions.

Proposal 4: To avoid impacting the legacy communication functions of base stations, new 6G service functions within the 6G RAN architecture shall be independent with traditional communication function.

Moreover, the outcome of RAN functions should be subject to performance consistency verification to ensure reliability, accuracy, and vendor-neutral trustworthiness. From an operator’s perspective, results provided solely by RAN functions of a specific vendor’s equipment cannot be deemed inherently trustworthy without such independent verification. To this end, the involvement of third parties is required to conduct testing and validation, and to provide performance evaluation on the quality of the results. Through such verification testing, it can also be assessed whether the results provided by vendors are optimal.

Proposal 5: The outcome of RAN functions shall be subject to performance consistency verification to ensure reliability, accuracy, and vendor-neutral trustworthiness.

### 2.2 Data Collection Function for 6G-RAN

At present, most 6G services entail requirements for data collection. As two representative services in 6G, both AI/ML and sensing inherently requires efficient and continuous data collection. The AI/ML service aims to optimize the network performance by utilizing AI/ML function which relies on large-scale, diverse, and continuously updated datasets to support the Life Cycle Management (LCM). Correspondingly, the transmission of large volume of AI data (e.g. for model training) and model transfer/delivery is essential to ensure the smooth operation of AI applications. Similarly, the sensing services also requires massive sensing measurement data received from multiple sensing entities to generate accurate sensing results. It should be noted that in 5G-A, the data collection function has already been recognized as a critical function within the RAN.

- Lessons from 5G

In the 5G-A, RAN spent lots of time on data collection function to collect AI data, and it is believed that some valuable lessons could be provided to the data collection framework design for 6G. During the discussion on data collection for UE-side model training in AI/ML for Air Interface, RAN plenary agreed following requirements [2]:

1.	The data collected is secured and data integrity and confidentiality for that data is ensured.

2.	User data privacy, anonymity and user consent is respected.

3.	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

4.	MNO has full visibility for standardized data.

5.	The design is futureproof and extendable.

The following solutions were proposed and studied in R18/R19, which have been captured in TR 38.843 [3].

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

For solution 1, RAN concluded that solution 1a has no 3GPP impacts, and solution 1b is precluded for standardization due to the potential privacy leakage risks of user and network.

For solution 2 and 3, transfer of data over CP and UP have been both considered and studied.

- CP solution: RAN has already concluded that transfer of data over CP can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.

- UP solution: RANP sent a LS [4] to SA2 and SA5 to study and provide feedback on fulfillment of visibility and controllability requirements on transfer of data over UP for solution 2 and 3 separately.

  - SA5 replied in LS [5] that it is assumed that the UP tunnel associated with data transfer is between UE and gNB for the solution 3. However, RAN2 replied in LS [6] that from RAN2 point of view UP tunnel between UE and gNB is not considered for 5G-A R19/20, as currently the UP tunnel doesn’t terminate in the gNB. Therefore, in our understanding, solution 3 with UP-based is not feasible for now.

  - SA2 replied in LS [7] that no conclusion has been reached yet on transfer of data over UP for solution 2, however a new 5G-A Rel-20 SA2 SID (SP-250413) has been approved which includes the objective of UE data collection, and the potential solutions are under discussion in SA2.

For CN-based solution 2, gNB is agnostic for the training data for the both the CP (by NAS) and UP (by UP tunnel between UE and UPF) solutions. So, when RAN needs the data for training, it needs to request CN to transfer the data back to RAN, which is an unnecessary round trip since the data was originally produced at RAN. Another potential issue is that when RAN obtains the data back from CN, the RAN network configuration may have changed, and the data becomes invalid for the new configuration, but RAN cannot realize the mismatch of the data and the network configuration due to RAN does not know the timing information of the data. Such kind of mismatch could make the training become inefficient and even useless. Whereas, for OAM-based solution 3, MDT framework is reused for data collection and gNB is aware of the content of the data. Also, RAN could store the data when it wants to use the data. Therefore, it makes sense to store the data in RAN when both the data consumer and data producer are in RAN.

Since there is no conclusion on solution 2 over UP in SA2, down-selection of potential solutions in RANP/RAN2 has been temporarily postponed. This situation has significantly impacted the progress of AI/ML for Air Interface feature, primarily due to excessive collaboration between RAN and SA working groups.

Therefore, we believe that AI data collection solutions for RAN-centric use cases should be addressed within RAN. Lessons from 5G indicate that RAN-centric functions may not need to involve CN, as involvement of CN only adds complexity to the overall functional design and slows standardization progress through continuous cross-group alignment.

Observation 3: Lessons from 5G indicate that involvement of CN for RAN-centric functions adds complexity to the overall function design and slows standardization progress through continuous cross-group alignment. It makes sense to store the data in RAN when both the data consumer and data producer are in RAN.

- Unified Data Collection Framework

To build on the lessons learned from data collection in 5G, the design of 6G data collection functions should aim for a unified data collection framework to support multiple 6G service requirements, which enables the corresponding entities access data collection function to obtain the data meets its own service requirement.

Currently, there are two understandings regarding to the unified data collection framework:

- Understanding 1: All data is initially collected by the core network and then distributed in a centralized manner by the Core Network.

- Understanding 2: Data is collected within the appropriate domain according to service requirements, with exposure to other domains when needed, under a unified framework that supports diverse use case.

As mentioned at the beginning part, the prerequisite for designing 6G RAN functions is to identify a clear and well-defined division of responsibilities between the radio access network and the core network, and same principle used for RAN data collection function. For understanding 1, we consider that a unified architecture does not mean that all data shall first be collected by the CN and then distributed. In certain cases, the data, such as L1 measurements and beam IDs for AI based Beam Management use case is collected, processed and used in RAN, which is irrelevant and useless to other domains. Therefore, it makes sense to just keep such data within the RAN. In comparison, understanding 2 is exactly the approach we are pursuing, as it reflects the principle of domain-specific functional responsibility.

Proposal 6: A unified data collection framework shall be designed to support multiple 6G service requirements, ensuring data is collected within the appropriate domain and exposed across domains only when necessary.

- AI/ML Service

To design a unified data collection framework to support diverse service, it is required to determine the domain for data collection function according to service requirements. From our view, the measurement data applied for RAN-centric AI use cases are layer 1 measurements or layer 3 measurements only collected, managed and applied solely within RAN side. Thus, it is unnecessary to send all collected data to CN. Moreover, the data for model training in 5G-A is collected per use case and per model type (e.g. UE-sided model and NW-sided model), which may result in duplicated data collection and reporting. For example, both AI/ML-based beam management case and AI/ML for mobility case needs L1 beam level measurements for high-performance model training. For RAN-centric use cases in 6G, RAN can maintain its own data repositories, where data collected and utilized exclusively within RAN, such as L1 or L3 measurements, can be managed and stored. In light of above analysis, 6GR should study a centralized data collection function in RAN side widely applied for RAN-centric AI use cases.

Observation 4: For RAN-centric use cases in 6G, RAN can maintain its own data repositories, where data collected and utilized exclusively within RAN, such as L1 or L3 measurements, can be managed and stored.

Observation 5: 6GR should study a RAN data collection function in RAN side widely applied for RAN-centric AI use cases.

- Sensing Service

For 6G sensing service, the sensing data are derived and generated from the raw sensing signal measured by each sensing entity. In use cases that only require sensing, the sensing result may only reflect the movement information of objects without involving identification information of the objects. Under these certain circumstances, such sensing data is considered more appropriate to collect and process locally within the RAN rather than in the CN, since the CN cannot interpret such low layer sensing measurement data.

Observation 6: In use cases that only require sensing, the sensing data is considered more appropriate to collect and process locally within the RAN rather than in the CN, since the CN cannot interpret such low layer sensing measurement data.

Furthermore, sensing service could also be used for object and intruder detection (e.g., UAV intrusion detection) in restricted areas or predefined secure areas around critical infrastructure, e.g., airports, light rail, government facilities, factories and research institutes, etc. Considering the sensing measurements might be sensitive, a centralized data storage and processing that enables data to remain within the private wireless networks can better meet the requirements for data privacy and minimize network latency. Since sensing and AI/ML have similar requirements in data collection, the centralized data collection function for AI/ML services in RAN side can also use for sensing service to collect data.

Observation 7: The centralized data collection function for AI/ML services in RAN side can also use for sensing service to collect data.

- Network Digital Twin Service

Another key service in 6G is Network Digital Twin (NDT), which enables proactive network state prediction and policy pre-verification, thereby supporting a higher level of network autonomy. To achieve accurate modeling and dynamic simulation of the network, NDT requires multi-dimensional data gathered from base stations, UEs and sensors, including radio channel measurements, topology information, resource utilization, traffic characteristics, environment context and user behavior patterns. Most of such data originates from the RAN, particular parameters related to the radio channel and resource utilization. Such data not only enhances a higher degree of network autonomy but also provides predictive analysis, anomaly detection, and resource optimization. In 5G-A, NDT has been introduced in SA5 to support autonomous network management and operation. However, the NDT deployed in MnS (Management Service) is insufficient to meet the near real-time prediction and pre-validation requirements of the RAN. Therefore, it is necessary to deploy dedicated function within RAN side for NDT data collection. Furthermore, since NDT simulations may need to reflect multi-vendor network typologies, centralized node on the RAN side is required to aggregate NDT data across different vendors. Likewise, given that NDT and AI have analogous requirements for data collection, the centralized data collection function for AI/ML services in RAN side can also use for NDT service to collect data.

Observation 8: The centralized data collection function for AI/ML services in RAN side can also use for NDT service to collect data.

Based on the above analysis of data collection functions for various 6G services, it can be observed that the services in 6G share the common characteristics that the data collection functions are all required in RAN side to deployed as a centralized function. From the operator’s perspective, introducing such centralized data collection function can provide benefits, as data from various services can be uniformly managed and controlled by the operator. Therefore, it is proposed that 6GR should study a unified centralized data collection function within the RAN to support emerging 6G services, e.g., AI/ML, sensing, NDT. From our view, the figure for radio access network data collection function for various service in 6G is illustrated as follows:

Figure1. Radio access network data collection function for various service in 6G

Proposal 7: 6GR should study a unified centralized data collection function within the RAN to support emerging 6G services, e.g., AI/ML, sensing, NDT.

With the deployment of a centralized data collection function for new 6G services within the RAN, it can also be leveraged for RAN for AI case to reduce deployment costs and enable dynamic allocation of computing resources. This centralized function allows computing resources to be shared across multiple base stations, enabling efficient offloading of AI applications and services from the UE and meeting the stringent requirements of real-time AI/ML services.

Proposal 8: The centralized function within the RAN can be leveraged for RAN for AI case to reduce deployment costs and enable dynamic allocation of computing resources.

# Conclusion

In this contribution, we take data collection function as a start point and reflects our perspective as operators on the design of RAN functions and architectures in 6G. The following observation and proposal are listed below:

Observation 1: The prerequisite for designing 6G RAN functions and architecture is to identify a clear and well-defined division of responsibilities between the radio access network and the core network.

Observation 2: The emerging service paradigms in 6G are expected to be seamlessly integrated into the 6G RAN, enabling a flexible, scalable, and service awareness future radio access network.

Observation 3: Lessons from 5G indicate that involvement of CN for RAN-centric functions adds complexity to the overall function design and slows standardization progress through continuous cross-group alignment. It makes sense to store the data in RAN when both the data consumer and data producer are in RAN.

Observation 4: For RAN-centric use cases in 6G, RAN can maintain its own data repositories, where data collected and utilized exclusively within RAN, such as L1 or L3 measurements, can be managed and stored.

Observation 5: 6GR should study a RAN data collection function in RAN side widely applied for RAN-centric AI use cases.

Observation 6: In use cases that only require sensing, the sensing data is considered more appropriate to collect and process locally within the RAN rather than in the CN, since the CN cannot interpret such low layer sensing measurement data.

Observation 7: The centralized data collection function for AI/ML services in RAN side can also use for sensing service to collect data.

Observation 8: The centralized data collection function for AI/ML services in RAN side can also use for NDT service to collect data.

Proposal 1: The 6G RAN architecture shall allow for the RAN and the CN to evolve independently.

Proposal 2: The introduction of a new RAN function must be carefully evaluated to determine whether a closed loop can be achieved within the RAN to realize the function as much as possible.

Proposal 3: A unified RAN architecture is essential to accommodate the various services envisioned in 6G.

Proposal 4: To avoid impacting the legacy communication functions of base stations, new 6G service functions within the 6G RAN architecture shall be independent with traditional communication function.

Proposal 5: The outcome of RAN functions shall be subject to performance consistency verification to ensure reliability, accuracy, and vendor-neutral trustworthiness.

Proposal 6: A unified data collection framework shall be designed to support multiple 6G service requirements, ensuring data is collected within the appropriate domain and exposed across domains only when necessary.

Proposal 7: 6GR should study a unified centralized data collection function within the RAN to support emerging 6G services, e.g., AI/ML, sensing, NDT.

Proposal 8: The centralized function within the RAN can be leveraged for RAN for AI case to reduce deployment costs and enable dynamic allocation of computing resources.

# References

- 3GPP TR 38.913 V18.0.0 (2024-03) Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 18)

- RP-243307 Way forward on AIML UE sided data collection

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-243316 LS on AI/ML UE sided data collection Source: TSG RAN To: TSG SA, WG SA2, WG SA5 Cc:	WG RAN2, WG SA3

- S5-250828 Reply LS on AI/ML UE sided data collection Source:	SA5	To: RAN, RAN2 Cc: SA, SA2, SA3

- R2-2503168 Reply LS on AI/ML UE sided data collection	 Source:	RAN2	 To:SA5	Cc: SA, SA2, SA3, RAN

- S2-2505713 Reply LS on AI/ML UE sided data collection Source:	SA2 To:	TSG RAN Cc:	TSG SA, SA3, SA5, RAN1, RAN2