3GPP TSG-RAN WG3 Meeting #129bis	             R3-256854

Prague, Czech, 13th – 17th Oct 2025

Agenda Item:	13.2

Source:	CMCC

Title:	Discussion on Network Architecture for Sensing

Document for:	Discussion

# Introduction

This contribution establishes the scope, framework, and key technical questions for RAN3's study of network architecture for gNB-based mono-static sensing targeting UAV detection as described below [1]. As the first meeting of this study item, the contribution focuses on scope clarification, architecture principles, and study methodology.

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

# Study Item Scope Clarification

## Mono-static vs. Bistatic Sensing Scope

The primary study addresses mono-static architecture where a single gNB performs both transmission and reception functions. This approach provides self-contained sensing capability per gNB with simpler deployment and operation characteristics. As a secondary consideration, the study item notes that "applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts." Bistatic sensing involves transmit and receive functions at different gNB, but importantly, no inter-gNB coordination procedures will be studied per the explicit study constraints.

The study shall focus on mono-static sensing architecture as the primary approach, with bistatic sensing considered for future applicability without additional architecture impacts.

# Key Architecture Questions for Study

## gNB-to-Sensing Function Interface Options

### SF Residing in Core

Figure 1 presents an architecture that leverages existing 5G core network infrastructure through the AMF using the established N2/NGAP interface. The SF communicates with the AMF using standard service-based architecture interfaces, and the AMF then forwards sensing-related messages to the gNB over the existing NGAP protocol. This approach treats sensing as an extension of existing network services rather than a fundamentally new capability. The architecture maintains unified control plane management at the AMF, providing consistent handling of both communication and sensing services.

Figure 1 SF Resides in Core, N2/N3

Figure 2 illustrates the architectural approach where the Sensing Function (similar to LMF) resides in the core network and communicates directly with the gNB through a dedicated interface (assuming the interface name is GS1). The GS1 interface carries both control plane signaling for sensing service configuration and user plane traffic for sensing data transport. This direct connection eliminates intermediate network functions, providing the shortest possible signaling path between the sensing control point and the sensing entity. The architecture is straightforward and enables rapid sensing service activation, making it suitable for scenarios where the SF is centrally deployed and latency optimization is a priority. However, this approach requires defining a completely new interface protocol and procedures.

Figure 2 SF Resides in Core, new direct interface

#### Control Plane Interface Options

Two control plane architectures exist for sensing service management. We assume the direct interface between gNB and Sensing Function is named GF2 for the following discussion purpose. Direct GS1-C interface establishes dedicated signaling between gNB and Sensing Function, while indirect approach extends NGAP on N2 interface through AMF.

- Direct GS1-C Advantages: Protocol optimized for sensing requirements without backward compatibility constraints. Eliminates AMF processing latency for service activation. Enables edge deployment without 5GC dependency. Independent evolution from communication services.

- Direct GS1-C Disadvantages: New protocol requires extensive standardization. Increased implementation cost and complexity. Additional interface for network management. Loss of unified control plane visibility at AMF.

- Indirect NGAP Advantages: Rapid deployment leveraging mature NGAP framework. Minimal implementation effort extending existing protocol. Unified control plane through AMF enables coordinated resource management. Reuses operational tools and procedures. Phased deployment with backward compatibility.

- Indirect NGAP Disadvantages: AMF processing adds control plane latency. Increased AMF load in dense sensing deployments. Limited protocol optimization due to backward compatibility requirements. Dependency on AMF availability.

Both control plane approaches are technically viable, with indirect approach favoring rapid deployment and direct approach optimizing performance.

RAN3 should define NGAP extensions for sensing service management as baseline control plane approach.

RAN3 should study direct GS1-C interface for future release addressing edge deployment and high-density scenarios.

#### User Plane Interface Options

User plane architecture determines sensing data transport path. Direct GS1-U bypasses UPF, while indirect approach uses enhanced GTP-U on N3 through UPF.

- Direct GS1-U Advantages: Ultra-low latency (single-digit ms) eliminating UPF processing. Minimal protocol overhead optimized for sensing. No UPF capacity constraints for high-rate data. Flexible edge deployment with local connectivity. Independent sensing and communication infrastructure scaling.

- Direct GS1-U Disadvantages: New protocol development for data encapsulation. Loss of unified user plane architecture. UPF functions (charging, lawful intercept) require alternative implementation. Separate network planning and operations.

- Indirect Enhanced GTP-U Advantages: Infrastructure reuse requiring only software upgrades. Unified user plane for communication and sensing traffic. Proven GTP-U scalability for millions of tunnels. Consistent QoS framework using sensing-specific 5QI values. Standard UPF functions (monitoring, charging, policy) apply to sensing. Leverages existing operational expertise.

- Indirect Enhanced GTP-U Disadvantages: UPF processing adds several milliseconds latency. Capacity considerations for high-rate sensing (>500 Mbps). GTP-U header overhead. Dependency on UPF availability. Protocol constraints from backward compatibility.

The existing GTP-U protocol architecture presents a fundamental gap for ISAC deployments. Current GTP-U tunnels are exclusively UE-centric, where every TEID is allocated in the context of a specific UE's PDU session and all tunneled traffic is associated with UE subscription and mobility management. This design assumption is deeply embedded in the protocol, affecting TEID allocation procedures, session lifecycle management, QoS flow mapping, and charging mechanisms. However, ISAC introduces network-originated sensing data that has no natural association with any UE. Sensing measurements such as environmental radar returns, spectrum monitoring results, or positioning reference signals are generated by the gNB itself as a network service rather than on behalf of any individual UE. The current architecture lacks mechanisms to establish, manage, and charge for GTP-U tunnels that carry network-service traffic independent of UE contexts. This creates ambiguity in TEID allocation (risk of collision with UE TEIDs), inability to properly differentiate sensing traffic from UE traffic for routing and QoS enforcement, and missing procedures for network-service tunnel lifecycle management decoupled from PDU session establishment. The GTP-U shall be enhanced as shown in Figure 3.

Figure 3 Enhanced GTP-U

Enhanced GTP-U provides substantial infrastructure reuse benefits, while direct interface optimizes latency for critical applications.

Establishing ISAC-specific GTP-U tunnels dedicated to sensing data transport between gNB and UPF is needed.

RAN3 should specify enhanced GTP-U with network-service TEID allocation and ISAC extension headers as baseline user plane.

RAN3 should study direct GS1-U interface for ultra-low latency (<5ms) and very high-rate (>500 Mbps) sensing services.

In summary, we have outlined the different requirements suited for the direct interface and the indirect interface.

Direct Interface Preferred For:

- End-to-end latency <5ms (user plane) or dynamic service management <10ms (control plane)

- Sensing data rate >500 Mbps aggregate per gNB

- Edge-deployed sensing functions co-located with gNB

- Standalone sensing networks without full 5GC

Indirect Interface Preferred For:

- Latency requirements >20ms

- Sensing data rate <100 Mbps per gNB

- Centralized sensing functions

- Initial deployments prioritizing rapid time-to-market

Considered both SA2 and RAN3 has not yet had a clear conclusion on which interface solution for control plane and user plane to support, the gNB is expected to have the capability to support flexible combination of control and user plane interface options (both direct, both indirect, or hybrid).

The gNB is expected to have the capability to support flexible combination of control and user plane interface options (both direct, both indirect, or hybrid).

### SF Co-located in RAN

Figure 4 depicts a distributed architecture where the Sensing Function is deployed as a separate entity within the NG-RAN infrastructure, in close proximity to the gNB. In this configuration, the SF and gNB communicate through a new interface that does not traverse the core network, enabling ultra-low latency sensing operations. Multiple sensing entities, can connect to the local SF for coordinated sensing operations within a geographic area. This architecture is particularly well-suited for applications requiring minimal latency, such as real-time object tracking or collision avoidance for UAV sensing, and for deployments integrated with a trusted third-party platform inside the network domain. The local processing capability reduces backhaul bandwidth requirements since raw sensing data does not need to traverse to a centralized location. This approach provides maximum flexibility for scaling sensing infrastructure independently from core network resources.

Figure 4 SF Co-located in RAN

RAN3 shall study RAN-located Sensing Function deployment as an option, defining the local interface between gNB and co-located SF.

## Whether to study DU-CU Split Architecture

RAN3 should decide whether to support DU-CU split architecture for sensing in the first meeting. If CU-DU split architecture is being considered, the sensing algorithms will place in DU which is close to RF hardware, and the CU will be the central node to receive the sensing result and allocate sensing configuration to DU. The F1-C interface would be extended with sensing-specific configuration and control signaling capabilities. While in F1-U interface, measurement result reporting mechanisms must accommodate sensing-specific data types and delivery requirements. The measurement reporting should reuse existing F1 data measurement reporting framework as much as possible.

If F1 architecture is being considered, F1-C interface would be extended with sensing-specific configuration and control messages as the primary interface enhancement.

If F1 architecture is being considered, F1-U interface extensions would be studied for deployments requiring sensing data transport from DU to CU, leveraging the existing F1 data measurement reporting framework.

# Conclusion

This contribution establishes the foundation for RAN3's gNB mono-static sensing architecture study. The following observations and proposals are discussed:

- Both control plane approaches are technically viable, with indirect approach favoring rapid deployment and direct approach optimizing performance.

- Enhanced GTP-U provides substantial infrastructure reuse benefits, while direct interface optimizes latency for critical applications.

- The study shall focus on mono-static sensing architecture as the primary approach, with bistatic sensing considered for future applicability without additional architecture impacts.

- RAN3 should define NGAP extensions for sensing service management as baseline control plane approach.

- RAN3 should study direct GS1-C interface for future release addressing edge deployment and high-density scenarios.

- Establishing ISAC-specific GTP-U tunnels dedicated to sensing data transport between gNB and UPF is needed.

- RAN3 should specify enhanced GTP-U with network-service TEID allocation and ISAC extension headers as baseline user plane.

- RAN3 should study direct GS1-U interface for ultra-low latency (<5ms) and very high-rate (>500 Mbps) sensing services.

- The gNB is expected to have the capability to support flexible combination of control and user plane interface options (both direct, both indirect, or hybrid).

- RAN3 shall study RAN-located Sensing Function deployment as an option, defining the local interface between gNB and co-located SF.

- If F1 architecture is being considered, F1-C interface would be extended with sensing-specific configuration and control messages as the primary interface enhancement.

- If F1 architecture is being considered, F1-U interface extensions would be studied for deployments requiring sensing data transport from DU to CU, leveraging the existing F1 data measurement reporting framework.

# References

RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR