3GPP TSG-RAN WG3 Meeting #129 bis	             R3-256855

Prague, Czech, 13th – 17th Oct 2025

Agenda Item:	13.3

Source:	CMCC

Title:	Discussion on RAN-CN Procedures Supporting ISAC

Document for:	Discussion

# Introduction

This contribution discusses on the necessary procedures and signaling enhancements for the RAN-CN interface to support Integrated Sensing and Communication (ISAC) services [1]. With SA2 defining the overall ISAC architecture framework [2], RAN3 must ensure the NG interface can efficiently support the sensing-related procedures and data flows required by the core network functions.

# Discussion

The integration of sensing capabilities into 5G networks requires new RAN-CN interface procedures to support sensing service lifecycle management, capability advertisement, data transport, and configuration provisioning. The existing NG interface was designed primarily for communication services and needs enhancement to handle the sensing data flows and ensure real-time sensing service requirements.

RAN3 shall evaluate whether existing NG interface architecture adequately supports ISAC requirements, study coexistence of sensing procedures with existing communication procedures, and assess need for dedicated sensing interface or enhancement of existing RAN-CN interface.

## RAN-CN Function Split for ISAC

A critical aspect of ISAC implementation is defining the appropriate function split between RAN and CN for sensing operations. This involves determining which function should be performed at the RAN versus CN, and what sensing data should be exchanged across the NG interface.

RAN-side ISAC Functions: The RAN should handle low-level sensing operations requiring real-time processing and direct access to RF signals, including:

- Raw sensing data collection: Gathering channel state information, radar reflections, and signal measurements from antenna arrays.

- Basic signal processing: Preliminary filtering, noise reduction, and basic target detection at the physical layer.

- Real-time sensing: Time-critical operations like collision detection that require immediate response.

- Local sensing coordination: Managing sensing operations across distributed antenna elements within a single site.

CN-side ISAC Functions: The CN should handle higher-level sensing operations that benefit from centralized processing and network-wide coordination, including:

- Sensing service orchestration: Managing end-to-end sensing services across multiple RAN nodes.

- Advanced data fusion: Combining sensing data from multiple gNBs for enhanced accuracy and coverage.

- Sensing result interpretation: Converting raw measurements into application-specific information.

- Sensing service exposure: Providing sensing results to external applications through standardized APIs.

RAN3 shall investigate the optimal RAN-CN function split for ISAC operations, defining which sensing functions are performed at RAN versus CN.

## Sensing Capability Advertisement

The core network needs visibility into the sensing capabilities available at each gNB as sensing entity to enable proper sensing service planning and resource allocation. This requires new procedures for:

- Sensing capability reporting: gNB must advertise its sensing capabilities including supported sensing methods, accuracy levels, coverage areas, and available sensing resources to the AMF during setup and updates.

- Dynamic capability updates: Procedures to inform CN of changes in sensing capabilities due to configuration updates, hardware changes, or interference conditions.

RAN3 shall investigate sensing capability advertisement procedures enabling CN functions to discover and utilize available RAN sensing resources. The capability of gNB includes sensing QoS, sensing report category and sensing area.

## Sensing Service Management

ISAC services require new lifecycle management procedures between RAN and CN:

- Service request: Signaling to request sensing services with specific parameters including target detection requirements, sensing accuracy, and reporting intervals.

- Service modification: Dynamic reconfiguration of active sensing services based on changing requirements or network conditions.

- Service release: Signaling to release sensing services including resource cleanup.

RAN3 shall define sensing service management procedures including service request, service modification, and service release between RAN and CN.

## Sensing Data Transport

The RAN-CN interface must support efficient transport of sensing data and results:

Sensing measurement reporting: New message structures for delivering sensing measurements from gNB to CN functions. Regard to the sensing measurement, the objective in RAN1 clearly states that metrics, measurements, and relevant measurement quantization for UAV use case should be identified and studied. During RAN1 discussion, sensing measurements can be divided into the following four levels according to different products of sensing processing:

- Level-1: Sensing raw data

Sensing raw data (e.g., IQ data) consists of time-domain samples or subcarrier-domain samples of original received signals from each antenna port. It contains all the information obtained by an ISAC system without signal processing.

- Level-2: Delay/angular/Doppler spectrums

Spectrums are the first-stage processing products from sensing raw data. It could retain most of the information using lower data volume than raw data. Delay spectrum, angular spectrum and Doppler spectrum are fundamental for most of the sensing algorithms.

- Level-3: Cloud of points

Cloud of points are acquired by implementing detection algorithms on delay, angular and Doppler spectrums. Most of the information may be discarded at this stage, which leads to a relatively low data volume.

- Level-4: Sensing result

Sensing result is the final output of sensing signal processing. A sensing result is described by target’s attributes, like its location, velocity and type. It has the lowest data volume.

As RAN1 already performed evaluations on above types of sensing measurements, and each of these sensing data types has its own supporters. From RAN3 perspective, it is recommended to wait for the conclusion made from RAN1.

For sensing measurement report, RAN3 needs to wait for the conclusion made from RAN1.

## Configuration and Policy Provisioning

The CN must be able to configure sensing parameters at the RAN:

- Sensing parameter configuration: Delivery of sensing-specific parameters including detection thresholds, measurement intervals, and target specifications

- Policy enforcement: Mechanisms to enforce sensing service policies including access control and priority management

- Quality of Sensing (QoS): New QoS framework specifically designed for sensing services with appropriate priority and resource allocation

RAN3 shall coordinate with SA2 ISAC architecture work to ensure RAN3 interface procedures align with overall system architecture requirements.

# Conclusion

The successful deployment of ISAC services requires comprehensive RAN-CN interface enhancements to support sensing-specific procedures and data flows. This contribution proposes that RAN3 develops the necessary signaling procedures to enable efficient sensing service delivery while maintaining backward compatibility with existing communication services.

We therefore have the following proposals:

- RAN3 shall evaluate whether existing NG interface architecture adequately supports ISAC requirements, study coexistence of sensing procedures with existing communication procedures, and assess need for dedicated sensing interface or enhancement of existing RAN-CN interface.

- RAN3 shall investigate the optimal RAN-CN function split for ISAC operations, defining which sensing functions are performed at RAN versus CN.

- RAN3 shall investigate sensing capability advertisement procedures enabling CN functions to discover and utilize available RAN sensing resources. The capability of gNB includes sensing QoS, sensing report category and sensing area.

- RAN3 shall define sensing service management procedures including service request, service modification, and service release between RAN and CN.

- For sensing measurement report, RAN3 needs to wait for the conclusion made from RAN1.

- RAN3 shall coordinate with SA2 ISAC architecture work to ensure RAN3 interface procedures align with overall system architecture requirements.

# References

RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication