3GPP TSG-RAN WG3 Meeting #129bis	R3-256879

Prague, CZ, 13 – 17 October 2025

Agenda item:	9.2.4

Source:	Nokia, NTT Docomo, BT

Title:	Correction of Paging NR Femtos

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

As part of the remaining open points of the work item (see tdoc [2]), RAN3 started to discuss solutions to solve the specific issue of paging femtos in some scenarios e.g. in shopping mall as addressed in tdoc [3].

Several solutions started to be discussed. Tdoc [3] proposed a solution for RRC Idle mode. This solution is further discussed here and an associated CR is proposed this time.

# 2	Paging repetition and NR Femtos

NR Femtos have very limited coverage. This creates special challenges with paging function because the AMF will typically make a first paging in the last NR Femto serving the UE and if the user has left the coverage of this last serving femto the AMF will typically escalade the paging at TA level for the paging repetition (second paging).

However, in many cases the end user has just moved away to a neighbor NR femto which is in close neighborhood i.e. the end user may be located under a neighbor of the last serving femto or in a neighbor of neighbor of the last serving femto. This is typically the case in a shopping mall where the end user has just moved to a next shop at the same floor of the shopping mall, or at a different floor.

Because the AMF doesn’t know the topology of the neighborhood and adjacency of the NR femtos, instead of making the second paging to this neighborhood, it will waste signaling and radio resources when paging in the full TA or RA involving tens to hundreds of cells.

Observation 1: upon paging failure to the last served NR Femto the paging escalation to TA or RA can lead to paging hundreds of NR Femtos and overlay macro gNBs in the area whereas the end user is likely near the last serving NR femto cell.

Proposal 1: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos.

At last RAN3#129 meeting a solution based on Area IDs was discussed to be further checked.

Introduce a RAN Node area ID

Each NR Femto can be configured in advance to belong to a RAN Node Area ID. The RAN Node Area ID is sent by the NR Femto to the connected NR Femto GW or to the	AMF at NG Setup Request.

This RAN Node Area ID is similarly included in the NGAP UE Context Release Complete message by the last serving NR Femto. When AMF pages the last serving NR Femto and it fails, there are two cases:

- If the NR femto is directly connected to AMF: AMF sends the second paging message (repetition) towards all NR femtos which belong to the same RAN Node Area ID (which it received in NG setup request).

- If the NR Femto is connected to the GW: the AMF sends the second paging message (repetition) including the RAN Node Area ID. The Femto GW relays the paging message to all NR Femtos which belong to this RAN Node Area ID (which it received in NG setup request).

The solution has minimum standards impact as illustrated in the CR in [4].

Proposal 2: RAN3 to discuss the NR Paging issue and the solution provided in [4].

# 3	Conclusion

This paper has explored the remaining paging femto issue and made following proposals:

Proposal 1: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos.

Proposal 2: RAN3 to discuss the NR Paging issue and the solution provided in [4].

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-255162, [TP for BL CR NR Femto TS 38.413] Completion of open points of NR Femto

- R3-255163, [TP for BL CR NR Femto TS 38.413] Paging issue with NR Femtos

- R3-256881, Correction of paging NR femtos