3GPP TSG-RAN3 #129-bis                                                             R3-256897

# Prague, Czech Republic, October 13-17, 2025

Agenda item:	10.4

Source:	Rakuten, Samsung

Title:	Discussion on RAN internal functional split and interfaces

Document for:	Discussion & Decision

# Introduction

In RAN#108 meeting [1], the 6G WG SID was approved with the following objectives on RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In RAN#109 meeting [3], the moderator’s summary [2] on 6G Requirements for architecture and migration has recorded the following online agreements. Proposal #6 is of particular interest for this paper.

In this contribution, we will share our initial views on RAN internal functional split, i.e. CU-DU split and CP-UP separation.

# RAN CU-DU split

The CU-DU split in 5G is the separation of the baseband unit (gNB) into a centralized unit (CU) and a distributed unit (DU) to enable a flexible and efficient Radio Access Network (RAN). This architecture provides significant advantages for network performance, cost, and flexibility that will be crucial for meeting the even more demanding requirements of 6G.

2.1 Benefits

The following are some of the benefits of the split architecture which were observed in 5G and will remain crucial for 6G also:

Enabling open eco-system: The CU-DU split is a foundational element of Open RAN, which mandates open, interoperable interfaces between network components from multiple vendors (like the F1 interface between the CU and DU). In an open environment, multi-vendor hardware and software can coexist on a common platform. This breaks proprietary vendor lock-in and allows the Mobile Network Operators (MNOs) to configure their networks using a cherry-picking approach, selecting the most competitive components from different vendors as shown in Figure 1, which reduces the total cost of ownership.

Figure 1: Open eco-system enabled by the RAN CU-DU split

Diverse Deployment Scenarios: The CU-DU split provides flexibility to support a variety of deployment scenarios, allowing the MNOs to tailor their network architecture to specific use cases, traffic patterns, and cost considerations. The choice of deployment scenario also depends on factors such as service requirements (e.g., latency), available transport network resources, and the desired level of centralization. The functional entities—RU, DU, CU-UP, and CU-CP—can be physically placed at various tiers in the network. These tiers include Cell site, Edge site, Regional Site and Central Site as. The choice of different options, from a fully distributed to a fully centralized RAN architecture, can address different requirements, offering the associated benefits (such as edge computing, high throughput at high mobility or centralized processing and simplified distributed coverage), within constraints (such as cost or transport requirements).

Figure 2: Deployment scenarios with disaggregated architecture

Network Virtualization and Cloud Native deployment: By decoupling software from proprietary hardware, the RAN split enables the network functions to be run on standard COTS servers. It also allows for independent horizontal scaling of infrastructure to address the constant evolution of radio access, instead of requiring frequent and costly vertical upgrades and hardware replacement of proprietary hardware. The split enables the deployment of CU and DU functions as virtualized network functions (VNFs) or containerized network functions (CNFs) on cloud infrastructure, leading to greater flexibility, scalability, and agility.

Efficient resource pooling: Connecting multiple DUs to a single, centralized CU allows for dynamic allocation of computing resources based on real-time network demands, preventing resource over-utilization or under-utilization in individual cells. Dynamic scaling of CU and DU components (e.g., scaling in when demand decreases and scaling out when demand increases) enhances resource utilization throughout the day. For instance, idle capacities in processing for one cell can be reused by other virtualized DUs in the same baseband pool, reducing the need to purchase additional equipment.

Flexibility and Scalability: This architecture allows network operators to add new sites or expand capacity by simply adding more DUs and CUs, rather than overhauling the entire network infrastructure. It grants operators unprecedented control over where and how functions are deployed based on the service needs.

Enhanced Operational Management: The split architecture supports reducing the operational cost. Consolidating CUs in a central location allows for simplified network management, maintenance, and software updates across many cell sites. This reduces the need for frequent and costly on-site visits. Moreover, it brings down the cost of power consumption. Centralizing the power-intensive processing equipment reduces overall energy consumption compared to powering baseband units at every cell site.

High Availability and Resiliency: The CU-DU split architecture enhances high availability and resiliency by decoupling failures, enabling centralized redundancy, and facilitating rapid recovery. In this model, if a Distributed Unit (DU) fails, the impact is isolated to that specific cell's coverage area, allowing the Centralized Unit (CU) to manage mobility and handovers to a neighboring, functioning cell, minimizing service disruption. Centralizing CUs in a data center allows for robust redundancy through server virtualization, where a failing CU instance can be quickly relocated or restarted on a healthy server without requiring physical hardware replacement at a cell site.

Observation 1: There are numerous operational advantages of the RAN CU-DU split-based architecture which includes enabling open ecosystem with multi-vendor support, diverse deployment options to meet operator’s unique needs, network virtualization and cloud deployment options with the associated benefits.

2.2 Enhanced Network Performance

Improved Mobility: Centralizing the CU allows for a larger area to be managed as a single site thereby increasing the intra-gNB mobility. This reduces the percentage of resource-intensive inter-gNB handovers, minimizing the risk of dropped calls and improving the user experience, especially in areas with high traffic or small cell deployment. With CU acting as an anchor for mobility, intra-CU mobility also reduces signaling to core, reduces latency & packet forwarding overhead.

Energy Efficiency: In times of low network traffic, the virtualized functions of the CU and DU can be consolidated onto fewer servers, allowing other hardware to be put into a low power "sleep" state, dramatically reducing energy consumption. Also, by aggregating the baseband processing of multiple DUs, the CU can handle varying loads more effectively. This provides a high resource multiplexing gain, ensuring that processing power is used more efficiently across the network rather than being siloed in individual, often underutilized, base stations. This is particularly advantageous in scenarios with disparate traffic loads across different geographical areas.

Advanced Features Support: The CU-DU split is a fundamental architectural change that enables a wide array of advanced features in 5G.

- Advanced load balancing: The CU manages multiple DUs, allowing it to pool and dynamically allocate computing resources to DUs based on real-time traffic demand. This prevents congestion in busy cells while ensuring resources are not wasted in lightly used areas, leading to more efficient network-wide utilization.

- Dedicated resource allocation for Slicing: The CU-DU split is essential for implementing end-to-end network slicing, allowing operators to create virtual network slices with dedicated resources and QoS tailored for different services (e.g., URLLC for autonomous vehicles, eMBB for immersive experiences).

- Mitigation of PDCP load imbalance: In 5G, Dual Connectivity allows a device to connect to two DUs simultaneously. The centralized CU can aggregate and manage the resulting split data streams (Packet Data Convergence Protocol, or PDCP), preventing one DU from becoming overloaded and ensuring a balanced load across all network resources

Observation 2: Significant enhanced network performance can be observed due to the RAN CU-DU split-based architecture. These include improved mobility, better energy efficiency and support for advanced features.

# RAN CU-CP and CU-UP separation

In 5G RAN architecture, there is functional decomposition of CU into control and user plane functions as shown in Figure 3. The CU-CP manages signaling, mobility, bearer setup, session control, etc., and the CU-UP handles user data transfer, including data packet forwarding and QoS enforcement. It allows the mobile service providers to bring the service closer to the user and reduce the user plane latency. 5G services based around Machine Type Communications (mMTC), Ultra Reliable Low Latency Communications (URLLC) and Fixed Wireless Access (FWA) generate unique traffic patterns as compared to the typical mobile data service. To support these, Control and User Plane Separation (CUPS) is needed.

Figure 3: Functional Decomposition of the CU

Like with the RAN CU-DU split, there are several operational benefits observed in 5G commercial deployments with this option of CP-UP split also.

Independent Scaling: Control and User Plane Separation (CUPS) applied to the CU functions results in the benefit of independent scaling of control plane and user plane functions. For example, more instances of CU-UP can be deployed to handle increase in user data without scaling with CU-CP. Similarly, massive IoT use cases might require more control plane handling and not necessarily more user plane.

Deployment Flexibility: There is significant deployment flexibility where the CU-CP can be centralized in a regional data center for global policy enforcement, mobility, security, etc., and the CU-UPs can be deployed at the edge, closer to the RAN or user, reducing data latency and backhaul load. This is also critical for support of Ultra-Reliable Low Latency (URLLC) requirements. The latency-critical applications benefit when CU-UP is placed at the edge. Furthermore, by locating CU-UP at the edge, the operator has the support to integrate with Multi-Access Edge Computing (MEC) apps more tightly, reducing round-trip time for services like AR/VR, industrial control, etc.

Resource Critical Feature Support: The split of control and user plane also enables Network Slicing and Qos differentiation. Each network slice can have its own CU-UP instance, tailored to specific user plane requirements (bandwidth, latency, reliability). The CU-CP can handle control for many slices, enabling efficient slice management.

Observation 3: The gains of CU-CP and CU-UP split includes independent scaling, deployment flexibility to adapt to various use cases, support for features which have specific resource requirements.

# Rakuten’s Deployment

Rakuten Mobile has widescale deployment of 4G and 5G sites both in terms of macro and small cells. Rakuten has embraced the true spirit of the Open RAN architecture and has collaborated with multiple vendors to adapt to various deployment use cases. Rakuten Mobile's deployment of a fully virtualized, cloud-native Open RAN network has had a significant impact on the Japanese telecom industry, influencing the adoption of CU/DU split architectures by the incumbent operators. Since its full-scale launch in April 2020, Rakuten Mobile has built a nationwide 5G network based on Open RAN standards, deploying over 17,000 outdoor base stations as of May 2024. By demonstrating the viability of Open RAN with open interfaces, Rakuten Mobile has encouraged other operators to adopt similar technologies, leading to a more diversified and resilient network infrastructure in Japan.

Rakuten has partnered with vendors for enterprise small cells, femto cells, 5G Sub6 small cells, 5G mmWave macros and indoor small cells. There are upwards of 150K+ small cells with third-party DUs in Rakuten Mobile deployment in Japan which firmly demonstrates the efficacy and viability of multi-vendor deployment in a mobile network.

Leveraging the capabilities offered by Rakuten, a European service provider has also embarked on constructing a Open RAN mobile network. For this deployment also, Rakuten has partnered with multiple vendors for 5G small cells. The ability to customize and enhance the network through software has resulted in streamlining network operations. From the commercial deployment point of view, the speed and ease of fixing issues in the field outperformed the legacy RAN where everything was bound to hardware and replacements took months.

# Critical Discussion

5.1 Technical challenges of the CU-DU split architecture

The CU-DU split architecture in 6G, building on its success in 5G, is a critical enabler for fulfilling the vision of a highly intelligent, flexible, and sustainable network. While challenges exist regarding latency, complexity, and longer time to roll out, there are several ways to address them.

Latency and signaling overhead: The split introduces additional latency due to the F1 interface between the CU and DU. Additionally, coordinating functions between the CU and DU over this interface increases signaling overhead, potentially consuming more network resources. In spite of this overhead, there is no reported KPI impact from the commercial deployments in terms of user impact or not meeting the QoS targets. Moreover, rendering DU more autonomous is a way to address the signaling and latency issues. The features can be designed such that CU-DU co-ordination should be limited to inevitable scenarios only.

Complexity of deployment and management: Managing and orchestrating virtualized resources across a distributed network infrastructure can be complex. There are many sophisticated network management systems with advanced orchestration capabilities available which are used to do this job efficiently. Moreover, the deployment options can be tailored to address specific requirements to support unique conditions like placing more time-critical functions at the DU and less time-critical functions at the CU.

Longer time to roll-out: Ensuring different vendors' equipment correctly communicates over the standardized F1 interface can require extensive interoperability testing and validation. And with hardware and software potentially coming from different vendors, identifying the root cause of network issues can be complex. However, we believe that there are points of trade-off to leverage the extensive benefits of the disaggregation.

Observation 4: There are some technical challenges to adoption of the RAN open interfaces like latency and signaling overhead, complexity of deployment and management and longer time to roll-out. However, there are multiple ways to address some of these issues.

5.2 Adoption of disaggregation in commercial deployments

One of the most important points of discussion is the current state of adoption of the CU-DU split with open F1 interface in the commercial deployments world-wide. Different studies peg this number to currently be in ballpark of 10% with a forecast of about 30% market share in the next couple of years [4]. Given this, it is critical to examine what technical and business inhibitors have been a hurdle to CU/DU split commercial adoption.

Disaggregated RAN isn’t plug-and-play: Multi-vendor integration across RU, DU, and CU introduces complexity which require interoperability validation and custom tuning and performance optimization across vendors.

Perception of risk: Operators are risk-averse with infrastructure investments. Traditional monolithic RAN systems are field-proven and well-supported. Disaggregated RAN components - especially from new vendors - lack the same track record. However, this is rapidly changing with the growing ecosystem of supporting vendors and with more operators adopting the Open RAN deployment.

Limited immediate ROI: The business case for disaggregated RAN is strongest at scale or in greenfield builds (e.g., Rakuten Mobile). Brownfield operators often do not see a clear ROI from moving to CU/DU split with open F1 interface unless they are already overhauling legacy networks or targeting edge-native use cases.

Strategic lock-in by incumbent vendors: Some of the vendors often develop proprietary implementations on the F1 interface to limit interoperability, making it hard to swap components. And it has been observed that product portfolio of some vendors do not offer options with the CU/DU split with open interfaces which discourages the MNOs from trying out the disaggregation option. These are strategic business decisions which benefit the legacy vendors.

These issues discussed above are often a barrier to entry for new vendors. And all this combined has resulted in slow adoption of the disaggregated architecture.

Observation 5: There are many reasons for slow adoption of the CU/DU split option with open interfaces. These include additional testing and customization requirements, limited immediate ROI, perception of risk and strategic lock in by incumbent vendors. These issues are often a barrier to entry for new vendors and deter the operators from trying out the disaggregated option.

# Why should the RAN split (CU-DU split) be carried forward to 6G

In 6G, where demands are even more stringent than in 5G, evolving the RAN architecture (including dynamic functional splits, more edge cloud, possibly non-terrestrial, AI/ML in the loop, etc.) is thought necessary by many in this industry. Following are several reasons justifying why it is important to continue to support these RAN split going forward with the 6G standardization activities.

Foundation for virtualization: 6G is envisioned as a fully cloud-native, intelligent, and highly automated network. The CU-DU split provides the necessary architectural foundation for a virtualized RAN that can be deployed on cloud platforms, a requirement for future 6G networks.

Enhanced AI/ML integration: The centralization of the CU enables a central point to embed Artificial Intelligence (AI) and Machine Learning (ML) functionalities. The AIML entity can collect data from multiple network nodes to perform real-time optimization, interference management, and dynamic resource allocation, a core feature of future 6G networks. The CU is an ideal location to host the AIML entity which collects data from across the network. This entity can be responsible for training the AIML models which are resource and computation intensive. The comparatively lightweight and less complex task inferencing can then be performed at the DUs.

Addressing higher traffic volumes: 6G will push traffic volumes far beyond what 5G handles today, requiring even more advanced resource management and coordination. The resource pooling and intelligent load balancing capabilities inherent in the CU-DU split will be essential for managing this explosive growth.

Maximizing network openness and agility: Continuing the Open RAN philosophy enabled by the CU-DU split will prevent vendor lock-in and encourage a more agile and innovative ecosystem for 6G development. This ensures the network can evolve rapidly to support new technologies and business models.

Support for diverse use cases: 6G will support a wider range of services with varying requirements for latency, bandwidth, and reliability, including non-terrestrial networks (NTN) like satellites. The flexible nature of the CU-DU split, with different possible functional splits, allows tailored deployments to meet these diverse needs.

- Highly dynamic geo-distributed computing: The CU-DU split is the basis for a future 6G network where computing resources are dynamically distributed across the network. This will support applications with ultra-low latency requirements, such as holographic communication and real-time augmented reality, by placing the compute power where it's most needed.

- Integrated Sensing and Communication (ISAC): 6G is expected to combine communication and sensing into a single technology. The split architecture would allow for the centralized processing of massive amounts of sensor data collected from the distributed DUs, enabling real-time environmental monitoring, accurate positioning, and enhanced awareness of the network environment.

- Seamless integration with NTN: The CU-DU split is critical for integrating satellite and High-Altitude Platform Station (HAPS) into the overall network. By keeping the lightweight DU on the satellite and the more intensive CU on the ground, the network can provide ubiquitous coverage in rural and remote areas while managing the unique constraints of NTN, such as resource limitations and mobility.

- Multi-TRP (Transmission and Reception Point): In 6G, the split architecture will enable more sophisticated multi-TRP operations. By centralizing the control plane in the CU, operators can orchestrate complex transmission and reception schemes involving multiple DUs to improve reliability and throughput.

- Advanced beamforming: By processing higher-layer signaling centrally, the CU can provide more sophisticated coordination for beamforming and signal processing, especially in massive MIMO deployments.

- Network Slicing and Multi Service Handling: It will be easier to configure logical splits and resource allocation per service type (URLLC, eMBB, IoT) using CU/DU architecture.

Observation 6: It is imperative that the support of CU-DU split with open F1 interface carries forward to the 6G design and standardization.

We’ve presented comprehensive discussion about the technical merits of the CU-DU split and backed it by the data from Rakuten’s deployment in Japan and other operators in Europe. Furthermore, we have also presented the technical and business reasons why 5G did not show significant adoption of these architectural features. We believe that there are several solutions to address the challenges, and a certain mindset shift is required by the operators and incumbent vendors in ensuring wider adoption of the split architecture. We have summarized the reasons why it is imperative to support the RAN split (CU-DU split) in 6G.  Based on all this, we propose that RAN3 take cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

Proposal 1: We propose that RAN3 takes cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

# CU-CP and CU-UP split in 6G

The advantages of the control and user plane separation which was observed in 5G will continue to be relevant in 6G also. This, combined with more aggressive demands of the 6G use cases, make it necessary to continue to support CUPS.

Distributed cloud architecture: 6G will likely have even more distributed compute requirements. Therefore, keeping CU-UP close to the users, while CU-CP remains centralized, optimizes for control-efficiency and data-latency.

Massive AI-native networking: CU-UP at the edge will furnish AI/ML agents with the much-needed real-time data locally. Whereas the control analytics can still be processed centrally at CU-CP.

Heterogeneous services: 6G is expected to support diverse services: holographic comms, tactile internet, massive twins, vehicular URLLC. These have very different CP vs UP needs. For example, drones need fast control updates; video streaming needs high-throughput user data.

Energy efficiency and sustainability: Turning off idle CU-UPs during off-peak hours can save energy. With the CUPS, the operators can data plane functions while keeping control active.

Observation 7: The advantages of the control and user plane separation which was observed in 5G will continue to be relevant in 6G also.

One of the most important agreements from RAN#109 influencing this study is the agreement “The 6G RAN architecture shall allow for control plane and user plane separation” [2] [3].

We argue that the above agreement cannot be a standalone agreement, because it is not possible to support the separation of control plane and user plane without the CU-DU disaggregation in place. The separation of CU-CP and CU-UP is an inherent feature of the overall disaggregated 5G RAN architecture, which assumes a CU-DU split is already in place. For the CU-CP/CU-UP split to exist, the CU itself must first be a distinct logical entity from the DU. If the CU and DU were monolithic (i.e., co-located and bundled as a single unit), there would be no separate CU to split into CU-CP and CU-UP entities.

Observation 8: As per agreement in RAN#109, it has been agreed that “The 6G RAN architecture shall allow for control plane and user plane separation”.

Observation 9: For the CU-CP/CU-UP split to exist, the CU itself must first be a distinct logical entity from the DU. If the CU and DU were monolithic (i.e., co-located and bundled as a single unit), there would be no separate CU to split into CU-CP and CU-UP entities.

Therefore, the separation of the CU-CP and CU-UP is a feature of a disaggregated RAN, not a condition that can be applied to a monolithic one. We propose that RAN3 takes into account the agreement from RAN#109 to allow the control and user plane separation in the 6G RAN architecture and concludes that it is possible to support this agreement only when the RAN CU-DU split already exists.

Proposal 2: We propose that RAN3 takes into account the agreement from RAN#109 to allow the control and user plane separation in the 6G RAN architecture and concludes that it is possible to support this agreement only when the RAN CU-DU split already exists.

# Conclusion

We have the following observations:

Observation 1: There are numerous operational advantages of the RAN CU-DU split-based architecture which includes enabling open ecosystem with multi-vendor support, diverse deployment options to meet operator’s unique needs, network virtualization and cloud deployment options with the associated benefits.

Observation 2: Significant enhanced network performance can be observed due to the RAN CU-DU split-based architecture. These include improved mobility, better energy efficiency and support for advanced features.

Observation 3: The gains of CU-CP and CU-UP split includes independent scaling, deployment flexibility to adapt to various use cases, support for features which have specific resource requirements.

Observation 4: There are some technical challenges to adoption of the RAN open interfaces like latency and signaling overhead, complexity of deployment and management and longer time to roll-out. However, there are multiple ways to address some of these issues.

Observation 5: There are many reasons for slow adoption of the CU/DU split option with open interfaces. These include additional testing and customization requirements, limited immediate ROI, perception of risk and strategic lock in by incumbent vendors. These issues are often a barrier to entry for new vendors and deter the operators from trying out the disaggregated option.

Observation 6: It is imperative that the support of CU-DU split with open F1 interface carries forward to the 6G design and standardization.

Observation 7: The advantages of the control and user plane separation which was observed in 5G will continue to be relevant in 6G also.

Observation 8: As per agreement in RAN#109, it has been agreed that “The 6G RAN architecture shall allow for control plane and user plane separation”.

Observation 9: For the CU-CP/CU-UP split to exist, the CU itself must first be a distinct logical entity from the DU. If the CU and DU were monolithic (i.e., co-located and bundled as a single unit), there would be no separate CU to split into CU-CP and CU-UP entities.

We have the following proposals:

Proposal 1: We propose that RAN3 takes cognizance of the merits of the CU-DU split with open F1 interface and recommend it for 6G architecture.

Proposal 2: We propose that RAN3 takes into account the agreement from RAN#109 to allow the control and user plane separation in the 6G RAN architecture and concludes that it is possible to support this agreement only when the RAN CU-DU split already exists.

# 9		Reference

[1] Chair Notes RAN#108

[2] RP-252880: Moderator's summary for RAN led 6G SI

[3] Chair Notes RAN#109

[4] Open RAN Market Size & Share Analysis - Growth Trends & Forecasts (2025 - 2030)