3GPP TSG-RAN WG3 #129bis	R3-256912

Prague, Czech Republic, 13-17 October 2025

Agenda Item:	12.2.1

Source:	Lenovo

Title: 		(TP to TR 38.745) Discussion on multi-hop UE trajectory

Document for:	Discussion and Approval

- Introduction

In this paper, we discuss the use case of multi-hop UE trajectory.

- Discussion

RAN3 spent a lot of time discuss the necessity to collect the measured UE trajectory from target gNB after handover, and ended up with supporting one-hop scenario for simplicity. On the other hand, we tend to believe that the multi-hop UE trajectory collection can mostly reuse the Rel18 one-hop UE trajectory collection approach with minimal standard impact. A simple example is given in the figure below with high level procedure signaling.

RAN3 is suggested to support multi-hop UE trajectory collection based on the Rel18 one-hop UE trajectory collection approach, i.e., the actual UE trajectory is collected hop by hop via Xn interface.

- Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to support multi-hop UE trajectory collection based on the Rel18 one-hop UE trajectory collection approach, i.e., the actual UE trajectory is collected hop by hop via Xn interface.

- TP to TR 38.745

***************************** CHANGE ENDS *****************************

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor Note: Capture the description of use case

In Rel18 AI/ML based mobility optimization discussion, NG-RAN node is capable of predicting UE trajectory in the future. In order to collect enough ground truth data for the NG-RAN node to train or monitor the AI/ML model, the source NG-RAN node may request the target NG-RAN node to measure and report the UE trajectory back to the source NG-RAN node after the handover operation.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

RAN3 understands that the single-hop UE trajectory collection approach supported in Rel18 can be extended to support the multiple-hop UE trajectory across gNBs with minimal standard impact, i.e., the actual UE trajectory is collected hop by hop via Xn interface. The detailed standard impact will be discussed in Rel19 normative phase.

An exemplary procedure with high level signalling flow is given below.

￼

Step 1: NG-RAN node 1 triggers a Data Collection Reporting Initiation procedure to configure NG-RAN node 2 being prepared to provide UE trajectory measurement after handover.

Step 2: NG-RAN node 1 triggers the Handover Preparation procedure to handover the UE to NG-RAN node 2. NG-RAN node 1 also indicates NG-RAN node 2 to measure and report the UE trajectory after handover as configured in step 1.

Step 3: NG-RAN node 1 sends the handover command to UE

Step 4~6: NG-RAN node 2 may decide to further handover the UE to another NG-RAN node 3. NG-RAN node 2 may perform the same Step 1 to Step 3 to request the UE trajectory measurement from NG-RAN node 3 after the handover.

Step 7: NG-RAN node 3 will measure the UE trajectory after UE is handed over to NG-RAN node 3 and report to NG-RAN node 2 as configured in step 4.

Step 8: Then NG-RAN node 2 will report the UE trajectory measurement, including both UE trajectory when UE is connected to NG-RAN node 2 and UE trajectory when UE is connected to NG-RAN node 3, to NG-RAN node 1 as configured in step 4.

***************************** CHANGE ENDS *****************************