3GPP TSG-RAN WG3 #129-bis																	R3-256924

Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1610 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Abnormal conditions for gNB-CU Configuration Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, InterDigital, Jio Platforms, FiberCop |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a predicted CCO issue is being cancelled, there needs to be a perfect match between the cells and beams initially predicted to be affected by the CCO issue, and the cells and beams for which the predicted CCO is being cancelled. The following cases of abnormal conditions may arise: - the gNB-DU serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel” but the listed cells and beams do not match the list of cells and beams contained in a previously received Predicted CCO Assistance Information IE. - the gNB-DU not serving the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE with the Predicted CCO issue IE set to “cancel”, but the Neighbour Future Coverage Modification Notification IE is not present. - The gNB-DU not controlling the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE. In the Predicted CCO Assistance Information IE, the Predicted CCO issue IE is present and set to “cancel”, but the Predicted Affected Cells and Beams IE is not present. - the gNB-DU not controlling the predicted affected cells and beams receives a GNB-CU CONFIGURATION UPDATE message which includes the Predicted CCO Assistance Information IE and the Neighbour Future Coverage Modification Notification IE. The Predicted CCO issue IE is set to “cancel”, but the list of cells and beams in the Predicted Affected Cells and Beams IE is not the same as the list of cells and beams contained in the Neighbour Future Coverage Modification Notification IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add abnormal conditions in the gNB-CU Configuration Update procedure for the cases above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the abnormal conditions described for cases above, the handling for predicted CCO issues is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

** skip unchanged **

#### 8.2.5.4	Abnormal Conditions

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is not present,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is present, and the list of cells and beams is not the same as the list of cells and beams contained in a previously received Predicted CCO Assistance Information IE,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE and assume that the predicted CCO issue is not cancelled.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is present, and the listed cells and beams are not served by the gNB-DU, and,

- the Neighbour Future Coverage Modification Notification IE is not present,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE and assume that the predicted CCO issue is not cancelled.

If the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE are contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the cells and beams in the Neighbour Future Coverage Modification Notification IE are not served by the gNB-DU, and

- the Predicted CCO issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is not present,

then the gNB-DU shall discard the received Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE and assume that the predicted CCO issue is not cancelled.

If the Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE are contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is present, and the listed cells and beams are not served by the gNB-DU, and

- the Neighbour Future Coverage Modification IE is present, and the list of cells and beams is not the same as the list of cells and beams in the Predicted Affected Cells and Beams IE,

then the gNB-DU shall discard the received Neighbour Future Coverage Modification Notification IE and the Predicted CCO Assistance Information IE and assume that the predicted CCO issue is not cancelled.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>

ANOTHER VERSION BELOW

t

#### 8.2.5.4	Abnormal Conditions

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is not present, or the Predicted Affected Cells and Beams IE is present but the list of cells and beams is not the same as the list of cells and beams contained in a previously received Predicted CCO Assistance Information IE,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE.

If the Neighbour Future Coverage Modification Notification IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Assistance Information IE is not present, or

- the Predicted CCO Assistance Information IE is present but the Predicted Affected Cells and Beams IE is not present,

then the gNB-DU shall discard the received Neighbour Future Coverage Modification Notification IE.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted Affected Cells and Beams IE is present, and the list of cells and beams includes cells and beams that are not served by the gNB-DU, and

- the Neighbour Future Coverage Modification Notification IE is not present, or the Neighbour Future Coverage Modification Notification IE is present but the list of cells and beams is not the same as the list of cells and beams that are not served by the gNB-DU in the Predicted Affected Cells and Beams IE,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>