3GPP TSG-RAN WG3 #129-bis																	R3-256925

Prague, Czech Republic, 13-17 October 2025

Title: 	Correction for gNB-DU Configuration Update

Source: 	Ericsson, InterDigital, Jio Platforms, FiberCop

Agenda item:	9.2.2

Document Type:	Discussion and Decision

# 1. Introduction

In this contribution we propose corrections for the gNB-DU Configuration Update F1AP procedure, in relation to cancelling a predicted CCO issue.

# 2. Discussion

According to the solution agreed in Rel-19 for AI/ML based CCO, when a predicted CCO issue is cancelled, a GNB-DUshould cancel the future coverage modification for all the corresponding predicted affected cells and beams.

The current procedure text for gNB-DU Configuration Update F1AP procedure captures this behaviour as follows:

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message and if the Future Coverage Modification Cause IE is set to “cancel”, the gNB-CU shall, if supported, consider it as a notification that the gNB-DU has cancelled the future coverage modifications indicated for the cells and beams listed in the Future Coverage Modification Notification IE.

According to current signaling design, the Future Coverage Modification Cause IE is associated with one cell for which the GNB-DU has generated the future coverage modification. Because of this, we think that the current specification presents some issues, discussed below.

A first issue derives from the fact that the current specification is inaccurate as it does not explicitly mention that the DU should indicate that the Future Coverage Modification Cause is set to “cancel” for all cells. This is because, to properly cancel a predicted CCO issue, the GNB-CU needs to be notified by GNB-DU that the future coverage modification the GNB-DU generated in response to the predicted CCO issue, is cancelled for all the cells involved. Below we propose an example of how the text can be corrected:

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message and if all the instances of the Future Coverage Modification Cause IE are set to “cancel”, the gNB-CU shall, if supported, consider it as a notification that the gNB-DU has cancelled the future coverage modifications indicated for the cells and beams listed in the Future Coverage Modification Notification IE.

Proposal 1: A gNB-CU can consider that the gNB-DU has cancelled the future coverage modifications for the cells and beams listed in the Future Coverage Modification Notification IE if all the instances of the Future Coverage Modification Cause IE included in a gNB-DU Configuration Update message are set to “cancel”.

A second issue arises from the fact that a GNB-DU may set some of the instances of the Future Coverage Modification Cause IE to “cancel”, and some other instances to a value different from "cancel". If this happens, the predicted CCO issue cannot be appropriately cancelled, because the gNB-CU receives mixed information from the gNB-DU, according to which the gNB-DU cancelled the future coverage modifications only for a portion of the cells and beams (and not all the cells and beams). If this scenario occur the cancelling cannot be completed correctly, and we think the specification should capture such abnormal condition.

Proposal 2: If the Future Coverage Modification Notification IE contains some instances of the Future Coverage Modification Cause IE set to “cancel”, and some other instances of the Future Coverage Modification Cause IE set to values different from "cancel", the gNB-CU shall ignore the IE and fail the gNB-DU Configuration Update procedure.

Proposal 3: Agree to the CR for TS 38.473 in R3-256926.

# 3. Conclusion

In this contribution, the following proposals are made:

Proposal 1: A gNB-CU can consider that the gNB-DU has cancelled the future coverage modifications for the cells and beams listed in the Future Coverage Modification Notification IE if all the instances of the Future Coverage Modification Cause IE included in a gNB-DU Configuration Update message are set to “cancel”.

Proposal 2: If the Future Coverage Modification Notification IE contains some instances of the Future Coverage Modification Cause IE set to “cancel”, and some other instances of the Future Coverage Modification Cause IE set to values different from "cancel", the gNB-CU shall ignore the IE and fail the gNB-DU Configuration Update procedure.

Proposal 3: Agree to the CR for TS 38.473 in R3-256926.