3GPP TSG-RAN WG3 #129-bis																	R3-256927

Prague, Czech Republic, 13-17 October 2025

Title: 	Stage 2 corrections for AI/ML based CCO

Source: 	Ericsson, InterDigital, Jio Platforms, FiberCop

Agenda item:	9.2.2

Document Type:	Discussion and Decision

# 1. Introduction

In this contribution to discuss stage 2 corrections to AI/ML based CCO specified in Rel-19.

# 2. Discussion

## 2.1 Behaviour when a CCO issue is predicted

In a split gNB1, when a CCO issue is predicted by the CU1w, the CU1 sends the Predicted CCO Assistance Information IE to the DU1 controlling the predicted affected cells and beams. The DU1, in response to the received Predicted CCO Assistance Information, generates a future coverage configuration for its cells and beams, and sends it to the CU in the Future Coverage Modification Notification IE.
After receiving from DU1 the Future Coverage Modification Notification, CU1 may sends to another connected DU2 the Neighbour Future Coverage Modification IE with the future coverage configuration for the affected cells and beams served by DU1. The DU2, in response to the received Neighbour Future Coverage Modification, generates a future coverage configuration for its cells and beams and sends the information back to the CU1.

At this point, the gNB1 sends the future coverage modification to a neighbouring gNB2 (CU2) in the Future Coverage Modification List IE, and CU2 sends to its underlying DUs (e.g., DU3) a Neighbour Future Coverage Modification IE. Similarly to DU2, also the DU3, in response to the received Neighbour Future Coverage Modification generates a future coverage configuration for its cells and beams and sends the information back to the CU2.

Hence, when a CCO issue is predicted, there are two scenarios in which a DU “responds” to its CU, namely:

Scenario 1: the DU serves the predicted affected cells and beams, and it receives the Predicted CCO Assistance Information IE from a CU

Scenario 2: the DU does not serve the predicted affected cells and beams, and it receives the Neighbour Future Coverage Modification IE from a CU

To capture the above, the current stage 2 (TS38.401) specifies the following:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

It can be appreciated how the first paragraph describes the information a DU receives in scenario 1, but no description is provided describing the DU response. The second paragraph describes the information received by a DU in scenario 2, and for that scenario the corresponding DU response is described.

The current text in TS38.401 is prone to be interpreted as if the DU “response” is specified for scenario 2, while the DU response is unspecified for scenario 1. On the other hand, according to the solution agreed for AI/ML based CCO in Rel-19, the DU “response” is exactly the same for both scenario 1 and 2, hence the specification is ambiguous in terms of DU behaviour for scenario 1.

Observation 1: In the current stage 2 text describing a gNB-DU behaviour when a CCO issue, it is not clear what is the behaviour of the DU controlling the predicted affected cells and beams, when receiving Predicted CCO Assistance Information from CU.

An easy way to solve this issue is to unify the two paragraphs, as shown below:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen. A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

With this simple correction, it becomes clear that the DU behaviour described in the sentence starting with “In response,” applies to both scenario 1 and 2, which is the behaviour agreed for AI/ML based CCO in Rel-19.

Proposal 1: In TS 38.401, section 7.11, unify the two paragraphs describing the received information at a DU when a CCO issue is predicted, and the corresponding DU response.

## 2.1 Behaviour when a predicted CCO issue is cancelled

When a predicted CCO issue is cancelled, the two following logical steps are used:

- a DU receives a notification that a previously predicted CCO issue is cancelled, and the list of associated predicted affected cells and beams

- if the DU has not yet applied the future coverage state change, it notifies the CU that the future coverage state change is cancelled

It is worth to note that step 2 is executed only if step 1 is executed.

Observation 2: A DU receives the notification that a predicted CCO issue is cancelled, and only after that, it informs the CU that the future coverage state associated with the cancelled predicted CCO issue (and not yet applied), is cancelled

We also observe that the steps above allow to cancel a predicted CCO issue both at the gNB predicting the issue, as well as at a neighbour gNB, with the same logic being used when the DU controls the predicted affected cells and beams and when the DU does not control such cells and beams.

The current text in stage 2 (38.401) describes the cancelling of a predicted CCO issue with two separate paragraphs:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled.

The gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

The two paragraphs above are separate, hence the text above can be read as if a DU may notify the CU that a previously generated future coverage state is not applied, without having previously been informed by the CU that the predicted CCO issue is cancelled. Such behaviour is not in agreement with the Rel-19 solution, and a clarification is needed.

Observation 3: The current stage 2 text describing the cancelling of a predicted CCO issue allows a DU to cancel a future coverage modification without having previously received an indication that the corresponding predicted CCO issue is cancelled, which is not in agreement with the Rel-19 solution.

We think that an easy way to solve this issue is to unify the two paragraphs, and add an explicit indication that the DU notifies the CU that a previously non applied future coverage state has been cancelled in response to a cancelled predicted CCO issue, as in the example proposed below:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled. In response, the gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

Proposal 2: In TS 38.401, section 7.11, unify the two paragraphs and indicate that DU notifies CU that a future coverage states not yet applied is cancelled in response to a cancelled predicted CCO issue.

Proposal 3: Agree to the CR for TS 38.401 in R3-256928.

# 3. Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: In the current stage 2 text describing a gNB-DU behaviour when a CCO issue, it is not clear what is the behaviour of the DU controlling the predicted affected cells and beams, when receiving Predicted CCO Assistance Information from CU.

Proposal 1: In TS 38.401, section 7.11, unify the two paragraphs describing the received information at a DU when a CCO issue is predicted, and the corresponding DU response.

Observation 2: A DU receives the notification that a predicted CCO issue is cancelled, and only after that, it informs the CU that the future coverage state associated with the cancelled predicted CCO issue (and not yet applied), is cancelled

Observation 3: The current stage 2 text describing the cancelling of a predicted CCO issue allows a DU to cancel a future coverage modification without having previously received an indication that the corresponding predicted CCO issue is cancelled, which is not in agreement with the Rel-19 solution.

Proposal 2: In TS 38.401, section 7.11, unify the two paragraphs and indicate that DU notifies CU that a future coverage states not yet applied is cancelled in response to a cancelled predicted CCO issue.

Proposal 3: Agree to the CR for TS 38.401 in R3-256928.