3GPP TSG-RAN WG3 #129-bis																R3-256931

Prague, Czech Republic, 13-17 October 2025

Title: 	On LS discussions with SA5 on Packet loss measurements in context of Release-19

Source: 	Ericsson, InterDigital, Jio Platforms, Verizon

Agenda item:	9.2.2

Document Type:	Discussion and agreement

# 1. Introduction

This contribution is on the discussions between RAN3 and SA5 during Release-19 on the topic of Packet Loss metric for UE Performance. RAN3 in Release-19 discussed split architectures as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture. However, discussions during Release-19 showed that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric based on the Release-18 specification and further discussion with SA5 was required for both the split and non-split architecture use-cases.

During R3#127-bis RAN3 sent an LS to SA5 (R3-252492) on the feasibility of measuring Packet Loss Rate in UL and DL at PDCP level. This was motivated based on discussions in RAN3 which concluded that PDCP-level measurements better reflect the UE performance at the service level. During RAN3#128, SA5’s response to the above LS was received (R3-253824), wherein SA5 provided references to the already available PDCP-level packet loss measurements in UL and DL in TS 28.552, which define NR cell level measurements.

RAN3 sent a further reply to the received LS in R3-253816 enquiring if the PDCP-level packet loss measurements suggested by SA5 are available with a granularity of per UE, where per DRB measurements are aggregated to generate per UE measurements, and if not to specify these measurements with per UE granularity. The motivation for the above was due to the fact that measurements in TS 28.552 are NR cell level measurements.

In response to the above LS, SA5 sent a further reply in R3-256518where SA5 confirmed that that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558. SA5 further attached corresponding CRs to 28.558 for Release-18 and Release-19 (S5-253900 and S5-253899) in the response.

# 2. Discussion

In the received Release-19 (S5-253899) and Release-18 (S5-253900) CRs, the following additions are implemented in TS 28.558 to be used as PDCP-level packet loss rate measurements:

- DL PDCP SDU Drop Rate (for non-split and split gNB)

- UL PDCU SDU Loss Rate (for non-split gNB)

SA5 further communicated that UL PDCP SDU Loss Rate for split gNB deployment has been already defined in clause 6.3.1.3 of TS 28.558.

Below we provide excerpts from the CR and from TS 28.558 on the definition of packet loss metrics.

###### 6.3.1.x.y1.1 DL PDCP SDU Drop Rate in gNB

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB. […]

[…]

c)	This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB without any part of it having been transmitted on the Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

[…]

###### 6.3.1.x.y2.1 DL PDCP SDU Drop Rate in gNB-CU-UP

- This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. […]

[…]

c)	This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

[…]

##### 6.3.1.y.1 UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB. […]

[…]

c)	This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

[…]

##### 6.3.1.3.1 UL PDCP SDU Loss Rate

a) This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. […]

[…]

c) This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d) Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

[…]

It can be seen from the above that each measurement for packet loss defined by SA5 is a (positive) integer that is computed by multiplying a decimal number (loss rate, lost packets divided by total number of packets) by 10^6. It is therefore obvious to deduce that the value range of the measurements referenced by SA5 is INTEGER in range (0..1000000).

Observation 1: The per-DRB per-UE packet loss measurements defined by SA5 (UL PDCP SDU Loss Rate and the DL PDCP SDU Drop Rate) in TS 28.558 are measurements represented by an integer with value range from 0 to 1000000

The Release 18 definition of the Average Packet Loss DL IE is tied to Packet Loss Rate IE (TS 38.423 clause 9.2.3.11), which is defined as an integer with a range (0..1000, …), where the represented value is a ratio of lost packets per number of packets sent, expressed in tenth of percent.

Comparing the definition of the PDCP-level packet loss metrics (from TS 28.558) and the Packet Loss Rate definition (in TS 38.423), there is an obvious disparity in the level of granularity of the two measurements. The packet loss metrics in TS 28.558 are defined as the drop rate (dropped packets / total number of packets) which is then scaled up by a factor of 106. The factor 106 is used to round the measurement to an integer value. Therefore, the smallest possible packet loss rate is 10-6 and the largest is 1, with each step having a granularity of 1x10-6. The Packet Loss Rate defined in TS 38.423 however, spans an integer range of 0 to 1000, where each step in the range has a granularity of a tenth of a percentage, i.e., 10-3, with this being the smallest representable value as well. If the existing definition of packet loss rate in TS 38.423 is used to represent the per-DRB per-UE PDCP-level packet losses, then

- The smallest possible representation of packet loss rate will be in the order of 10-3

- There is a loss of resolution/granularity of the measurement, as the measurement results are bucketed into a smaller range of values

- Assuming that RAN3 will reference the measurements defined by SA5 in TS 28.558, there is a mismatch between the reference measurement and the value range of the IE used to represent the UL/DL loss

Observation 2: There exists a significant limitation is using the Average Packet Loss DL metric as defined in TS 38.423 for the purpose of representing the DL PDCP SDU Drop Rate measured according to the definition in TS 28.558.

In addition to the discussion on the received LS from SA5, another aspect to consider is that the Release-18 definition of average packet loss in TS 38.423 concerns only downlink packet loss measurements. Measurements in the uplink were not considered as corresponding measurements in uplink were not defined by SA5. However, the above referenced SA5 CRs add support for per-DRB per-UE packet loss measurements in DL and in UL for Release-18 and Release-19.

Observation 3: The support of UL PDCP SDU Loss Rate in UL for split and non-split gNB in TS 28.558 makes it feasible to extend the Release-18 and Release-19 definitions of UE Performance metrics with Average Packet Loss Rate in UL.

Exacerbating the already mentioned issues of granularity and range, the existing definition of Packet Loss Rate IE used in UE performance according to TS 38.423 refers to the per-QoS-Flow level packet losses (clause 9.2.3.11), while the Average Packet Loss DL in context of UE performance is a per UE metric. The definition of Packet Loss Rate IE is reused by both Average Packet Loss DL IE (in UE performance) and GBR QoS Flow Information IE (used to denote the QoS Parameters for a GBR QoS Flow). The above, therefore, eliminates the possibility of revising the definition of Packet Loss Rate IE as it also affects the definition and purpose of GBR QoS Flow Information IE.

Observation 4: The definition of Packet Loss Rate IE which is used to define the Packet Loss Rate IE in UE Performance in TS 38.423 is also coupled with another IE. Therefore, changing the underlying definition causes further issues.

Based on observations 1, 2, and 4, and further supported by observation 3, a clear need to amend the existing Average Packet Loss DL metric in TS 38.423 to ensure that it can represent the collected measurement (DL PCDP SDU Drop Rate) can be seen. Furthermore, the newly introduced support for UL PDCP SDU Loss Rate in TS 28.558 enables to further extend the metrics contained in the UE Performance IE in TS 38.423 with UL Average packet loss measurements. As a matter of fact, discussions during RAN3#127-bis captured the following alternative solutions in context of Average Packet Loss DL IE in Release-18:

  - Opt1: Remove the IE

  - Opt2: Introduce new IE for packet loss metric in UE performance to replace the old IE

  - Opt3: Ignore the IE in R18

  - Opt4: Do nothing

RAN3 did not discuss shortlisting the above list further in the subsequent meetings as RAN3 was awaiting further clarification from SA5 in this regard. However, we believe that RAN3 has received sufficient information from SA5 in this regard and a decision can be made. The addition of UL loss rate measurements by SA5 now enables a fifth option as well:

  - Opt5: Introduce new IEs (both UL and DL) for packet loss metric in UE performance to replace the old IE for both Release-18 and Release-19.

Based on the above discussions, the following proposals are made:

Proposal 1: RAN3 to introduce a new IE in Rel-18 and Rel-19 for DL packet loss metric in UE Performance that has sufficient granularity and range to represent the DL PDCP SDU Drop Rate measurement defined in TS 28.558

Proposal 2: RAN3 to introduce a new IE in Rel-18 and Rel-19 for UL packet loss metric in UE Performance that has sufficient granularity and range to represent the UL PDCP SDU Loss Rate measurement defined in TS 28.558

A TP reflecting the main proposals in this paper with corresponding changes to Release-19 is presented in R3-256933 and changes corresponding to Release-18 is presented in R3-256932.

# 3. Conclusions

This paper makes the following observations and proposals:

Observation 1: The per-DRB per-UE packet loss measurements defined by SA5 (UL PDCP SDU Loss Rate and the DL PDCP SDU Drop Rate) in TS 28.558 are measurements represented by an integer with value range from 0 to 1000000

Observation 2: There exists a significant limitation is using the Average Packet Loss DL metric as defined in TS 38.423 for the purpose of representing the DL PDCP SDU Drop Rate measured according to the definition in TS 28.558.

Observation 3: The support of UL PDCP SDU Loss Rate in UL for split and non-split gNB in TS 28.558 makes it feasible to extend the Release-18 and Release-19 definitions of UE Performance metrics with Average Packet Loss Rate in UL.

Observation 4: The definition of Packet Loss Rate IE which is used to define the Packet Loss Rate IE in UE Performance in TS 38.423 is also coupled with another IE. Therefore, changing the underlying definition causes further issues.

Proposal 1: RAN3 to introduce a new IE in Rel-18 and Rel-19 for DL packet loss metric in UE Performance that has sufficient granularity and range to represent the DL PDCP SDU Drop Rate measurement defined in TS 28.558

Proposal 2: RAN3 to introduce a new IE in Rel-18 and Rel-19 for UL packet loss metric in UE Performance that has sufficient granularity and range to represent the UL PDCP SDU Loss Rate measurement defined in TS 28.558

A CR reflecting the main proposals in this paper is presented in R3-256932 (Rel-18) and R3-256933 (Rel-19).