3GPP TSG-RAN WG3 Meeting #129bis	R3-256950

# Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0493 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for WAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The “BH-AMF” has not been used, suggest to remove the definition. - In the Figure 6.1.7-1, there is a line to connect the “Neighbour NG-RAN node” with the “UE’s 5GC”, it causes the misunderstanding that the BH PDU sessions for transporting of Xn-C/Xn-U traffic is routed via UE’s 5GC. This is not correct. Instead, a line is needed to connect the “Neighbour NG-RAN node” with the “BH-5GC”. - It was agreed that the Additional ULI can be included in both UE-associated signalling and non-UE associated signalling. However, in clause 12.5, only the case that there is UE served by the WAB-gNB is mentioned, the non-UE associated case is not been covered. In SA2 spec as shown below, it referred to TS 38.401 for non-UE associated case. So, it is suggested to change the title to emphasize the Additional ULI and to capture the non-UE associated case as well. TS 23.501: - The MWAB-gNB indicates the Additional ULI to AMF(s) it connects to by NG Setup and RAN Configuration Update procedures as specified in TS 38.401 [42].. - In clause 12.8, the first sentence may cause misunderstanding that the WAB-MT is co-located with the BH-gNB. Suggest to make it clear. And some editorial changes to keep align with stage 3. |
| Summary of change: |  | - Remove the definition of “BH-AMF”. - Modify the Figure 6.1.7-1: - - remove the line between “Neighbour NG-RAN node” and “UE’s 5GC”, - - add a line between “Neighbour NG-RAN node” and “BH-5GC”, - - change the sentence “Backhaul PDU Session(s) for transporting of NG-C/NG-U interface traffic of WAB-gNB” to “Backhaul PDU Session(s) for transporting of NG-C/NG-U/Xn-C/Xn-U interface traffic of WAB-gNB”. - In clause 12.5, change the title to “Additional User Location Information”, and capture that the WAB-gNB can indicate the Additional ULI to the core network by NG Setup and RAN Configuration Update procedures. - In clause 12.8, some editorial change to make the spec more clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.1.7, 12.5, 12.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------

3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

BH-5GC: The 5GC serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-----------------Next Change-------------------

6.1.7	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-gNB. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.7-1 shows the WAB architecture for 5GS.

Figure 6.1.7-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.  The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.7-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.7-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.7-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.7-3: Protocol stacks for Xn Control plane and Xn User plane transport

-----------------Next Change-------------------

12.5	Additional User Location Information

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures, if required by the core network.

The WAB-gNB can indicate the Additional ULI to the core network by NG Setup and RAN Configuration Update procedures.

-----------------Next Change-------------------

12.8	Xn connection management

A WAB-gNB can establish an Xn connection with the BH-gNB serving the WAB-MT co-located with the WAB-gNB, and with the neighbouring gNBs. During the Xn setup or NG-RAN node configuration update, the WAB-gNB can include a WAB-MT identifier, to indicate that it is a WAB-gNB. In case the peer gNB is the WAB-MT’s BH-gNB, the WAB-MT ID makes the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB, and the cell ID of BH-gNB´s cell serving the WAB-MT.

Establishment of Xn connections between two WAB-gNBs can be avoided. To achieve this, the WAB-gNB may reject the Xn setup initiated by another WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.

A WAB-gNB should be configurable with respect to whether it should accept or reject Xn setup requests received from WAB-gNBs.

-----------------End of the First Change---------------