3GPP TSG-RAN WG3 Meeting #129bis	R3-256956

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1354 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing of LP-WUS disabling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, Nokia, Huawei, Qualcomm, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 agreed to support enabling/disabling LP-WUS for a UE with NAS signalling. RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS per UE in IDLE and INACTIVE and CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE. Based on RAN2 agreements, it is suggested to introduce an indication to disable LP-WUS in the NG Paging message and in the Core Network Assistance Information for RRC INACTIVE IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the LP-WUS Disable Indication IE in the PAGING message and in the Core Network Assistance Information for RRC_INACTIVE IE. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact on the LP-WUS function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to disable LP-WUS per UE. On the other hand, if LP-WUS is disabled over NAS but gNB still enable LP-WUS subgroup paging to UE, in case the UE is paged, other UEs which share the same subgroup with that UE will be falsely alarmed due to the associated LP-WUS aimed towards that UE, which is not desirable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.3.1.2, 8.3.4.2, 8.4.2.2, 8.4.4.2, 8.5.1.2, 9.2.4.1, 9.3.1.15, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 CR1573 TS 38.300 Draft CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

<<<omitted>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12]. If the LP-WUS Disable Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it to disable LP-WUS as described in TS 38.300 [8].

<<<omitted>>>>

NEXT CHANGE

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall

-	if supported, store the received IAB Authorization information in the UE context and use it as specified in TS 38.401 [2].

-	if supported, store the received Mobile IAB Authorization information in the UE context. If the Mobile IAB Authorized IE is set to "not authorized" for a mobile IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the mobile IAB-node will not serve any UE(s).

If the Security Key IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall store it and perform AS key re-keying according to TS 33.501 [13].

If the UE Security Capabilities IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall store them and take them into use together with the received keys according to TS 33.501 [13].

If the Index to RAT/Frequency Selection Priority IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the RAN Paging Priority IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

If the UE Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

-	use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

If the Core Network Assistance Information for RRC INACTIVE IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, replace the previously provided Core Network Assistance Information for RRC INACTIVE and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12]. If the LP-WUS Disable Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it to disable LP-WUS as described in TS 38.300 [8].

<<<omitted>>>>

NEXT CHANGE

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

<<<omitted>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12]. If the LP-WUS Disable Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it to disable LP-WUS as described in TS 38.300 [8].

If the CN Assisted RAN Parameters Tuning IE is included in the HANDOVER REQUEST message, the NG-RAN node may use it as described in TS 23.501 [9].

<<<omitted>>>>

NEXT CHANGE

8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF. Upon reception of the PATH SWITCH REQUEST message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Path Switch Request Transfer IE to the SMF associated with the concerned PDU session.

<<<omitted>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12]. If the LP-WUS Disable Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it to disable LP-WUS as described in TS 38.300 [8].

If the CN Assisted RAN Parameters Tuning IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node may use it as described in TS 23.501 [9].

<<<omitted>>>>

NEXT CHANGE

8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

The AMF initiates the Paging procedure by sending the PAGING message to the NG-RAN node.

At the reception of the PAGING message, the NG-RAN node shall perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the PAGING message, the NG-RAN node may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present, according to TS 38.300 [8].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 38.300 [8].

If the Paging Origin IE is included in the PAGING message, the NG-RAN node shall transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 36.304 [29]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [29]. The NG-RAN node should take into account the reception time of the PAGING message on the NG interface to determine when to page the UE.

If the NB-IoT Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 36.304 [29].

If the Enhanced Coverage Restriction IE is included in the PAGING message, the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the Paging Assistance Data for CE Capable UE IE is included in the Assistance Data for Paging IE in the PAGING message, it may be used for paging the indicated CE capable UE, according to TS 36.300 [17].

If the WUS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it to determine the WUS group for the UE, as specified in TS 36.304 [29].

If the E-UTRA Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 36.304 [29]. If the E-UTRA Paging Time Window IE is included in the E-UTRA Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [29]. The NG-RAN node should take into account the reception time of the PAGING message on the NGAP interface to determine when to page the UE.

If the CE-mode-B Restricted IE is included in the PAGING message and the Enhanced Coverage Restriction IE is not set to "restricted", the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the NPN Paging Assistance Information IE is included in the Assistance Data for Paging IE, the NG-RAN node may take it into account when determining the cells where paging will be performed.

If the NR Paging eDRX Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it according to TS 38.304 [12] and TS 23.501 [9]. If the NR Paging Time Window IE is included in the NR Paging eDRX Information IE, the NG-RAN node shall take this information into account to determine the UE’s paging occasion according to TS 38.304 [12].

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

If the LP-WUSPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for LP-WUS paging subgrouping of the UE, as specified in TS 38.300 [8].

If the LP-WUS Disable Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it to disable LP-WUS as described in TS 38.300 [8].

NEXT CHANGE

9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.3.1.138 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.3.1.139 | If this IE is present, the Paging DRX IE is ignored. | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| WUS Assistance Information | O |  | 9.3.1.143 |  | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| LP-WUSPS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |
| LP-WUS Disable Indication | O |  | ENUMERATED (true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

NEXT CHANGE

9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  |  |  | - |  |  |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  |  |  | - |  |  |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  |  |  | - |  |  |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  |  |  | - |  |  |  |
| TAI List for RRC Inactive |  | 1 |  |  |  |  | - |  |  |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  |  |  | - |  |  |  |
| >>TAI | M |  | 9.3.3.11 |  |  |  | - |  |  |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  |  |  | - |  |  |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  |  |  | YES |  | ignore |  |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | This IE is ignored if the Further Extended UE Identity Index Value IE is present. |  | YES |  | ignore |  |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  |  |  | YES |  | ignore |  |
| MICO All PLMN | O |  | 9.3.1.194 |  |  |  | YES |  | ignore |  |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  |  |  | YES |  | ignore |  |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) |  | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. |  | YES |  | ignore |  |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  |  |  | YES |  | ignore |  |
| Hashed UE Identity Index Value | O |  | 9.3.3.62 |  |  |  | YES |  | ignore |  |
| CN MT Communication Handling | O |  | ENUMERATED (supported, …) |  | This IE indicates the CN support of CN based MT communication handling. |  | YES |  | ignore |  |
| LP-WUSPS Assistance Information | O |  | 9.3.1.x |  |  |  | YES |  | ignore |  |
| Further Extended UE Identity Index Value | O |  | 9.3.3.X |  |  |  | YES |  | ignore |  |
| LP-WUS Disable Indication | O |  |  | ENUMERATED (true, …) |  |  |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

NEXT CHANGE

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AssociatedSessionID,

AuthenticatedIndication,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

BroadcastTransportFailureTransfer,

BroadcastTransportRequestTransfer,

BroadcastTransportResponseTransfer,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CEmodeBrestricted,

CEmodeBSupport-Indicator,

CNAssistedRANTuning,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformationForInactive,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

DL-CP-SecurityInformation,

DL-Signalling,

EarlyStatusTransfer-TransparentContainer,

EDT-Session,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EN-DCSONConfigurationTransfer,

EndIndication,

Enhanced-CoverageRestriction,

ERedCapIndication,

EUTRA-CGI,

EUTRA-PagingeDRXInformation,

Extended-AMFName,

Extended-ConnectedTime,

Extended-RANNodeName,

FiveGCAction,

FiveG-ProSeAuthorized,

FiveG-ProSePC5QoSParameters,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverFlag,

HandoverType,

IAB-Authorized,

IABNodeIndication,

IAB-Supported,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

IntersystemSONConfigurationTransfer,

LAI,

LocationReportingRequestType,

LPWUSPSAssistanceInformation,

LPWUSDisableIndication,

LTE-A2X-ServicesAuthorized,

LTEM-Indication,

LTEUESidelinkAggregateMaximumBitrate,

LTEV2XServicesAuthorized,

MaskedIMEISV,

<<<omitted>>>>

FROM NGAP-Containers

id-A2X-PC5-QoS-Parameters,

id-AerialUEsubscriptionInformation,

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-AssociatedSessionID,

id-AuthenticatedIndication,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-BroadcastTransportFailureTransfer,

id-BroadcastTransportRequestTransfer,

id-BroadcastTransportResponseTransfer,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CEmodeBrestricted,

id-CEmodeBSupport-Indicator,

id-CNAssistedRANTuning,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-DL-CP-SecurityInformation,

id-DL-Signalling,

id-EarlyStatusTransfer-TransparentContainer,

id-EDT-Session,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-ENDC-SONConfigurationTransferDL,

id-ENDC-SONConfigurationTransferUL,

id-EndIndication,

id-Enhanced-CoverageRestriction,

id-ERedCapIndication,

id-EUTRA-CGI,

id-EUTRA-PagingeDRXInformation,

id-Extended-AMFName,

id-Extended-ConnectedTime,

id-Extended-RANNodeName,

id-FiveGCAction,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5QoSParameters,

id-FiveG-ProSeUEPC5AggregateMaximumBitRate,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverFlag,

id-HandoverType,

id-IAB-Authorized,

id-IABNodeIndication,

id-IAB-Supported,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-IntersystemSONConfigurationTransferDL,

id-IntersystemSONConfigurationTransferUL,

id-LocationReportingRequestType,

id-LPWUSPSAssistanceInformation,

id-LPWUSDisableIndication,

id-LTE-A2X-ServicesAuthorized,

id-LTE-A2X-UE-PC5-AggregateMaximumBitRate,

id-LTEM-Indication,

id-LTEUESidelinkAggregateMaximumBitrate,

id-LTEV2XServicesAuthorized,

<<<omitted>>>>

-- **************************************************************

--

#### -- PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PAGING

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PagingIEs} },

...

}

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity				CRITICALITY ignore	TYPE UEPagingIdentity					PRESENCE mandatory	}|

{ ID id-PagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional		}|

{ ID id-TAIListForPaging				CRITICALITY ignore	TYPE TAIListForPaging					PRESENCE mandatory	}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional		}|

{ ID id-PagingOrigin					CRITICALITY ignore	TYPE PagingOrigin						PRESENCE optional		}|

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional		}|

{ ID id-NB-IoT-Paging-eDRXInfo			CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInfo				PRESENCE optional		}|

{ ID id-NB-IoT-PagingDRX				CRITICALITY ignore	TYPE NB-IoT-PagingDRX					PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction		PRESENCE optional		}|

{ ID id-WUS-Assistance-Information		CRITICALITY ignore	TYPE WUS-Assistance-Information			PRESENCE optional		}|

{ ID id-EUTRA-PagingeDRXInformation		CRITICALITY ignore	TYPE EUTRA-PagingeDRXInformation		PRESENCE optional		}|

{ ID id-CEmodeBrestricted				CRITICALITY ignore	TYPE CEmodeBrestricted					PRESENCE optional		}|

{ ID id-NR-PagingeDRXInformation		CRITICALITY ignore	TYPE NR-PagingeDRXInformation			PRESENCE optional		}|

{ ID id-PagingCause						CRITICALITY ignore	TYPE PagingCause						PRESENCE optional		}|

{ ID id-PEIPSassistanceInformation		CRITICALITY ignore	TYPE PEIPSassistanceInformation			PRESENCE optional		}|

{ ID id-LPWUSPSAssistanceInformation	CRITICALITY ignore	TYPE LPWUSPSAssistanceInformation		PRESENCE optional		}|

{ ID id-LPWUSDisableIndication      	CRITICALITY ignore	TYPE LPWUSDisableIndication  		   PRESENCE optional		},

...

}

<<<omitted>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CN-MT-CommunicationHandling,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-DownlinkTLContainer,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedMobilityInformation,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-EquivalentSNPNsList,

id-FurtherExtendedUEIdentityIndexValue,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-HashedUEIdentityIndexValue,

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-IntersystemMobilityFailureforVoiceFallback,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-LPWUSPSAssistanceInformation,

id-LPWUSDisableIndication,

<<<omitted>>>>

#### -- C

CoreNetworkAssistanceInformationForInactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-EUTRA-PagingeDRXInformation					CRITICALITY ignore	EXTENSION EUTRA-PagingeDRXInformation					PRESENCE optional	}|

{ ID id-ExtendedUEIdentityIndexValue				CRITICALITY ignore	EXTENSION ExtendedUEIdentityIndexValue					PRESENCE optional	}|

{ ID id-UERadioCapabilityForPaging					CRITICALITY ignore	EXTENSION UERadioCapabilityForPaging						PRESENCE optional	}|

{ ID id-MicoAllPLMN									CRITICALITY ignore	EXTENSION MicoAllPLMN											PRESENCE optional	}|

{ ID id-NR-PagingeDRXInformation					CRITICALITY ignore	EXTENSION NR-PagingeDRXInformation						PRESENCE optional	}|

{ ID id-PagingCauseIndicationForVoiceService		CRITICALITY ignore	EXTENSION PagingCauseIndicationForVoiceService		PRESENCE optional	}|

{ ID id-PEIPSassistanceInformation					CRITICALITY ignore	EXTENSION PEIPSassistanceInformation						PRESENCE optional	}|

{ ID id-HashedUEIdentityIndexValue					CRITICALITY ignore	EXTENSION HashedUEIdentityIndexValue						PRESENCE optional	}|

{ ID id-CN-MT-CommunicationHandling					CRITICALITY ignore	EXTENSION 	 CN-MT-CommunicationHandling				PRESENCE optional	}|

{ ID id-LPWUSPSAssistanceInformation				CRITICALITY ignore	EXTENSION LPWUSPSAssistanceInformation					PRESENCE optional	}|

{ ID id-FurtherExtendedUEIdentityIndexValue			CRITICALITY ignore	EXTENSION FurtherExtendedUEIdentityIndexValue			PRESENCE optional	}|

{ ID id-LPWUSDisableIndication				        CRITICALITY ignore	EXTENSION LPWUSDisableIndication					      PRESENCE optional	},

...

}

<<<omitted>>>>

#### -- L

LAC	::= OCTET STRING (SIZE (2))

LAI ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

lAC					LAC,

iE-Extensions		ProtocolExtensionContainer { {LAI-ExtIEs} } OPTIONAL,

...

}

LAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedCellInformation ::= CHOICE {

nGRANCell		LastVisitedNGRANCellInformation,

eUTRANCell		LastVisitedEUTRANCellInformation,

uTRANCell		LastVisitedUTRANCellInformation,

gERANCell		LastVisitedGERANCellInformation,

choice-Extensions		ProtocolIE-SingleContainer { {LastVisitedCellInformation-ExtIEs} }

}

LastVisitedCellInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LastVisitedCellItem ::= SEQUENCE {

lastVisitedCellInformation		LastVisitedCellInformation,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedCellItem-ExtIEs} }	OPTIONAL,

...

}

LastVisitedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional },

...

}

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedUTRANCellInformation ::= OCTET STRING

LineType ::= ENUMERATED {

dsl,

pon,

...

}

LocationReportingAdditionalInfo ::= ENUMERATED {

includePSCell,

...

}

LocationReportingReferenceID ::= INTEGER (1..64, ...)

LocationReportingRequestType ::= SEQUENCE {

eventType										EventType,

reportArea										ReportArea,

areaOfInterestList								AreaOfInterestList								OPTIONAL,

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID					OPTIONAL,

-- The above IE shall be present if the Event Type IE is set to the value “stop UE presence in the area of interest”

iE-Extensions		ProtocolExtensionContainer { {LocationReportingRequestType-ExtIEs} }		OPTIONAL,

...

}

LocationReportingRequestType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LocationReportingAdditionalInfo	CRITICALITY ignore	EXTENSION LocationReportingAdditionalInfo		PRESENCE optional }|

{ ID id-AdditionalCancelledlocationReportingReferenceIDList	CRITICALITY reject	EXTENSION AdditionalCancelledlocationReportingReferenceIDList  PRESENCE optional },

...

}

LoggedMDTNr ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

loggedMDTTrigger					LoggedMDTTrigger,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration	OPTIONAL,

wLANMeasurementConfiguration		WLANMeasurementConfiguration		OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration		OPTIONAL,

areaScopeOfNeighCellsList			AreaScopeOfNeighCellsList			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LoggedMDTNr-ExtIEs} } OPTIONAL,

...

}

LoggedMDTNr-ExtIEs	NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-EarlyMeasurement		CRITICALITY ignore	EXTENSION EarlyMeasurement		PRESENCE optional	},

...

}

LoggingInterval ::= ENUMERATED {

ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440,

infinity,

...

}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120, ...}

Links-to-log ::= ENUMERATED {

uplink,

downlink,

both-uplink-and-downlink,

...

}

LoggedMDTTrigger ::= CHOICE{

periodical			NULL,

eventTrigger			EventTrigger,

choice-Extensions		ProtocolIE-SingleContainer { {LoggedMDTTrigger-ExtIEs} }

}

LoggedMDTTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LPWUSPSAssistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {LPWUSPSAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

LPWUSDisableIndication ::= ENUMERATED {true, ...}

<<<omitted>>>>

NEXT CHANGE

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<<Omitted >>>>>>>>>>>>>>>>>>>>

id-AIOTSessionReleaseCommandTransfer					ProtocolIE-ID ::= 463

id-AIOTSessionReleaseCompleteTransfer					ProtocolIE-ID ::= 464

id-AIOTSessionReleaseRequestTransfer				ProtocolIE-ID ::= 465

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 466

id-AIOTFIdentifier									ProtocolIE-ID ::= 467

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 468

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 469

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 470

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 471

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 472

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 473

id-AIoT-NASPDU										ProtocolIE-ID ::= 474

id-AIoT-Support											ProtocolIE-ID ::= 475

id-AIOTFName											ProtocolIE-ID ::= 476

id-LPWUSPSAssistanceInformation							ProtocolIE-ID ::= 477

id-FurtherExtendedUEIdentityIndexValue					ProtocolIE-ID ::= 478

id-FiveGProSeLayer2MHUEtoNetworkRelay					ProtocolIE-ID ::= 479

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay		ProtocolIE-ID ::= 480

id-FiveGProSeLayer2MHRemote								ProtocolIE-ID ::= 481

id-FiveGProSeLayer3MHUEtoNetworkRelay					ProtocolIE-ID ::= 482

id-AdditionalULI										ProtocolIE-ID ::= 483

id-PduSetDelayBudgetDownlink							ProtocolIE-ID ::= 484

id-PduSetDelayBudgetUplink								ProtocolIE-ID ::= 485

id-PduSetErrorRateDownlink								ProtocolIE-ID ::= 486

id-PduSetErrorRateUplink								ProtocolIE-ID ::= 487

id-DLPDUSetInformationMarkingSupportIndication			ProtocolIE-ID ::= 488

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= 489

id-MMSID												ProtocolIE-ID ::= 490

id-Indication-of-bitrate-adaptation						ProtocolIE-ID ::= 491

id-SCGActivationTime									ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingControl			ProtocolIE-ID ::= 494

id-LPWUSDisableIndication					            ProtocolIE-ID ::= yyy

CHANGES END