3GPP TSG-RAN WG3 Meeting # 129bis	R3-257004

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1356 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Alternative QoS parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.501 section 5.7.1.2a mentions that Averaging window is part of Alternative QoS Parameters. When adding Avering window to AQP, NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which AQP can be used as reference if the QoS profile is not fulfilled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Averaging window in AQP that has the same default value as the one in the QoS Profile Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only impacts Alternative QoS parameters in the specs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.151, 9.4.5, 9.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1572 TS 38.473 CR 1619 TS 37.483 CR 0182 TS 38.300 draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  | - |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.135 | This IE is included if the PDU Set Packet Delay Budget IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Downlink | O |  | Packet Error Rate 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the DL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >PDU Set Error Rate Uplink | O |  | Packet Error Rate 9.3.1.81 | This IE is included if the PDU Set Error Rate IE in the UL PDU Set QoS Information IE is present and is ignored otherwise. | YES | ignore |
| >Averaging Window | O |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. This IE has the same value as the Averaging Window IE present in QoS Flow Level QoS Parameters IE in 9.3.1.12. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

SKIPPED TEXT UNCHANGED

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-AdditionalULI,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

id-DLPDUSetInformationMarkingSupportIndication,

id-MonitoringRequestonAvailableBitrate,

id-MMSID,

id-Indication-of-bitrate-adaptation,

id-SCGActivationTime,

id-Aerial-UE-FlightInformationReporting,

id-Aerial-UE-FlightInformationReportingControl,

id-AveragingWindow,

maxnoofAllowedAreas,

SKIPPED TEXT UNCHANGED

#### -- A

SKIPPED TEXT UNCHANGED

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQosParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			AlternativeQoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate												OPTIONAL,

guaranteedFlowBitRateUL				BitRate												OPTIONAL,

packetDelayBudget					PacketDelayBudget									OPTIONAL,

packetErrorRate						PacketErrorRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetDownlink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-AveragingWindow			CRITICALITY ignore	EXTENSION AveragingWindow				PRESENCE optional	},

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

SKIPPED TEXT UNCHANGED

id-PduSetDelayBudgetUplink								ProtocolIE-ID ::= 485

id-PduSetErrorRateDownlink								ProtocolIE-ID ::= 486

id-PduSetErrorRateUplink									ProtocolIE-ID ::= 487

id-DLPDUSetInformationMarkingSupportIndication			ProtocolIE-ID ::= 488

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= 489

id-MMSID													ProtocolIE-ID ::= 490

id-Indication-of-bitrate-adaptation						ProtocolIE-ID ::= 491

id-SCGActivationTime										ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingControl			ProtocolIE-ID ::= 494

id-AveragingWindow										ProtocolIE-ID ::= xxx

END

-- ASN1STOP

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////