3GPP TSG-RAN WG3 Meeting # 129bis	R3-257006

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1572 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Alternative QoS parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.501 section 5.7.1.2a mentions that Averaging window is part of Alternative QoS Parameters. When adding Avering window to AQP, NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which AQP can be used as reference if the QoS profile is not fulfilled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Averaging window in AQP that has the same default value as the one in the QoS Profile. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only impacts Alternative QoS parameters in the specs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.102, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 1356 TS 38.473 CR 1619 TS 37.483 CR 0182 TS 38.300 draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.2.3.102	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  |  | 1..<maxnoofQoSparaSets> |  |  | – |  |
| >Alternative QoS Parameters Set Index |  | M |  | 9.2.3.103 |  | – |  |
| >Guaranteed Flow Bit Rate Downlink |  | O |  | Bit Rate 9.2.3.4 |  | – |  |
| >Guaranteed Flow Bit Rate Uplink |  | O |  | Bit Rate 9.2.3.4 |  | – |  |
| >Packet Delay Budget |  | O |  | 9.2.3.12 |  | – |  |
| >Packet Error Rate |  | O |  | 9.2.3.13 |  | – |  |
| >Maximum Data Burst Volume |  | O |  | 9.2.3.15 | Maximum Data Burst Volume is specified in TS 23.501 [7]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >Averaging Window |  | O |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. This IE has the same value as the Averaging Window IE present in QoS Flow Level QoS Parameters IE in 9.2.3.5. | YES | ignore |
| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  | – |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.2.3.103 |  | – |  |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.2.3.4 |  | – |  |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.2.3.4 |  | – |  |  |
| >Packet Delay Budget | O |  | 9.2.3.12 |  | – |  |  |
| >Packet Error Rate | O |  | 9.2.3.13 |  | – |  |  |
| >Maximum Data Burst Volume | O |  | 9.2.3.15 | Maximum Data Burst Volume is specified in TS 23.501 [7]. This IE may be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |  |
| >PDU Set Delay Budget Downlink | O |  | 9.2.3.113 |  | YES | ignore |  |
| >PDU Set Delay Budget Uplink | O |  | 9.2.3.113 |  | YES | ignore |  |
| >PDU Set Error Rate Downlink | O |  | 9.2.3.13 |  | YES | ignore |  |
| >PDU Set Error Rate Uplink | O |  | 9.2.3.13 |  | YES | ignore |  |
| >Averaging Window | O |  | 9.2.3.14 | Averaging Window is specified in TS 23.501 [7]. This IE has the same value as the Averaging Window IE present in QoS Flow Level QoS Parameters IE in 9.2.3.5. | YES | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

SKIPPED TEXT UNCHANGED

id-MMSID,

id-Indication-of-bitrate-adaptation,

id-GeographyBasedMDT,

id-NetworkSliceAreaScopeofMDT,

id-SBFD-Frequency-Configuration,

id-NZP-CSI-RS-Resources-Config,

id-SRS-Resource-Configuration,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-SliceMeasurementInitiationResult,

id-SliceUEPerformance,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

id-AveragingWindow,

maxEARFCN,

maxnoofAllowedAreas,

SKIPPED TEXT UNCHANGED

#### -- A

SKIPPED TEXT UNCHANGED

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume				PRESENCE optional }|

{ ID id-PduSetDelayBudgetDownlink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional }|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional }|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate					PRESENCE optional }|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate					PRESENCE optional }|

{ ID id-AveragingWindow			CRITICALITY ignore	EXTENSION AveragingWindow				PRESENCE optional	},

...

}

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

SKIPPED TEXT UNCHANGED

id-FailureType																						ProtocolIE-ID ::= 527

id-FiveGProSeLayer3MHUEtoNetworkRelay																ProtocolIE-ID ::= 528

id-FiveGProSeLayer2MHUEtoNetworkRelay																ProtocolIE-ID ::= 529

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay													ProtocolIE-ID ::= 530

id-FiveGProSeLayer2MHRemote																			ProtocolIE-ID ::= 531

id-LTMInformationSCG-AddReq																			ProtocolIE-ID ::= 532

id-LTMInformationSCG-AddReqAck																		ProtocolIE-ID ::= 533

id-LTMInformationSCG-ModReq																			ProtocolIE-ID ::= 534

id-LTMInterSNExecutionNotification																	ProtocolIE-ID ::= 535

id-LTMPSCellInformation-AddReq																		ProtocolIE-ID ::= 536

id-LTMPSCellInformation-AddReqAck																	ProtocolIE-ID ::= 537

id-LTMPSCellInformation-UpdateReq																	ProtocolIE-ID ::= 538

id-LTMPSCellInformation-UpdateReqAck																ProtocolIE-ID ::= 539

id-LTMPSCellInformation-UpdateReqired																ProtocolIE-ID ::= 540

id-LTMPSCellInformation-UpdateConfirm																ProtocolIE-ID ::= 541

id-LTMPSCellInformation-ChangeRequired																ProtocolIE-ID ::= 542

id-LTMPSCellInformation-ChangeConfirm																ProtocolIE-ID ::= 543

id-LTM-DC-DataForwarding-Indicator

id-AveragingWindow

ProtocolIE-ID ::= xxx

END

-- ASN1STOP

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////