3GPP TSG RAN WG3 Meeting #129bis	             						R3-257041

Prague, Czech Republic, Oct. 14th –17th, 2025

Agenda Item:	10.5

Source:	AT&T

Title:	6G Data Management Framework Considerations

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-252912 [1]. The following objectives are of relevance to the AI/ML framework and corresponding data collection in 6G [1]:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we discuss the requirements and the main design principles for enabling an extensible data management framework in 6G.

# Requirements for an integrated data collection and management framework for 6G

6G is expected to natively enable a broad range of use cases and requirements, including new services that require support for various types of data, including AI/ML data collection and management. One of the cornerstones to achieve the 6G design goals is to establish an integrated framework for data collection to enable MNOs to effectively manage network capacity, foster innovation, and unlock new value opportunities. The following core principles should be followed for such an integrated framework:

- A unified data collection framework across domains to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 1: An integrated data management framework that includes AI/ML data collection and data management, with MNO control and visibility, is supported in 6G.

To ensure scalability and availability of data when and where it is needed, a unified data collection framework especially entails unification of data collection and management framework across the network domains, including RAN, CN and OAM. To that end, it is important, and well captured by the RAN SI, to have close collaboration between all relevant 3GPP working groups, including RAN2 and RAN3 with the relevant SA working groups, to avoid any fragmentation that would hinder extensibility and future integration.

Proposal 2: Study and specify a unified integrated 6G data management and collection framework in close collaboration between RAN2, RAN3, SA2 and SA5.

The unified data collection and management framework should further avoid fragmentation and per-use case specification and ensure scalability to new services and use cases as needed. The framework also allows efficient reuse of the collected data, thus improving network efficiency and reducing overhead. It further enables better observability into the network and unlocks better opportunities for network automation. Multiple termination points for data within the MNO network should then be supported as part of the framework, with MNO visibility, wherein data collected can be made available at the OAM, RAN, and CN, to be used when needed.

Proposal 3: The integrated 6G data management and collection framework supports multiple termination points, e.g. in the 6G RAN, 6G OAM and 6G CN.

Data collection and management, for AI/ML, encompasses data collection for training, performance monitoring as well as data transfer, both at the UE side and the network side. The specified 6G data management framework can further include AI/ML model LCM aspects such as AI/ML model/functionality activation/deactivation /switching/fallback. This will allow all AI/ML related aspects to be managed from within a new management plane, or data plane, which enables us to onboard use cases quickly and efficiently in 6G.

Proposal 4: The integrated 6G data management and collection framework encompasses data collection and management for AI/ML models across different domains (UE, RAN, etc.), including UE-sided data collection, network-sided data collection, performance monitoring, and data transfer.

The data collection framework should be designed to cater to the properties of the data being collected and used. This data is different from the user plane or the control plane traffic and should be specified as such.

Proposal 5: The integrated 6G data collection and management framework is designed to differentiate the data plane traffic from the user plane and control plane traffic.

The new data collection framework or newly designed data plane allows for much better data management as the network can collect data for multiple use cases simultaneously improving efficiency. Furthermore, the network can add relevant additional information and labels to the data. The network can also ensure that the privacy is preserved, and proprietary and security aspects of the data are properly addressed before the data is shared with the network or UE vendors. Additionally, it gives network control over when to collect data or transfer models so that there is minimal impact on system performance. Finally, the integrated data collection framework is extensible to be used to quickly develop and deploy AI/ML use cases based on the network requirements, reducing the standardization effort to onboard new use cases.

As we develop the new data plane framework, and we allow for multiple termination points for the data collection in the MNO network across multiple domains, a service-based design can be considered whereas data producers and data consumers are created to allow for the data to be available and consumed when and where it is needed, in a flexible and scalable manner.  This can naturally yield itself to a service-based design of the interface between the RAN and the CN, especially when considering the RAN for AI and sensing use cases within the data collection framework. The pTp NG interface can potentially also be enhanced to transport the data plane traffic as needed from the RAN to the CN.

Proposal 6: For the integrated 6G data collection and management framework, enhancements to the RAN-CN interface should be considered.

# Conclusion

In this contribution, we proposed a data management framework for 6G, and the corresponding requirements. We made the following proposals.

Proposal 1: An integrated data management framework that includes AI/ML data collection and data management, with MNO control and visibility, is supported in 6G.

Proposal 2: Study and specify a unified integrated 6G data management and collection framework in close collaboration between RAN2, RAN3, SA2 and SA5.

Proposal 3: The integrated 6G data management and collection framework supports multiple termination points, e.g. in the 6G RAN, 6G OAM and 6G CN.

Proposal 4: The integrated 6G data management and collection framework encompasses data collection and management for AI/ML models across different domains (UE, RAN, etc.), including UE-sided data collection, network-sided data collection, performance monitoring, and data transfer.

Proposal 5: The integrated 6G data collection and management framework is designed to differentiate the data plane traffic from the user plane and control plane traffic.

Proposal 6: For the integrated 6G data collection and management framework, enhancements to the RAN-CN interface should be considered.

# References

[1] RP-252912, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic